DEPARTMENT OF PUBLIC HEALTH AND ENVIRONMENT

Water Quality Control Commission

REGULATION NO. 35 - CLASSIFICATIONS AND NUMERIC STANDARDS FOR GUNNISON AND LOWER DOLORES RIVER BASINS

5 CCR 1002-35

[Editor’s Notes follow the text of the rules at the end of this CCR Document.]

_________________________________________________________________________

[Not shown: no changes to sections 35.1 through 35.5.]

35.6	TABLES

[Not shown: no changes to sections 35.6(1) through 35.6(3).]

(4)	Discharger-specific Variances

(a)	San Miguel Segment 12c (COGUSM12c):

Discharger-specific Variance, Town of Nucla (CO0048945), Adopted 6/8/2026.

Ammonia (acute/chronic): Initial AEL=11.6 mg/L (2/1-7/31 and 9/1-9/30), Final AEL= 9.3 mg/L (2/1-7/31 and 9/1-9/30).

Includes a Pollutant Minimization Program. (see 35.55(B)).

Expiration date: 12/31/2031.

[Not shown: no changes to sections 35.6(5) through 35.54.]

35.55	STATEMENT OF BASIS, SPECIFIC STATUTORY AUTHORITY AND PURPOSE; JUNE 8, 2026 RULEMAKING; FINAL ACTION AUGUST 10, 2026; EFFECTIVE DATE DECEMBER 31, 2026

The provisions of C.R.S. 25-8-202(1)(a), (b) and (2); 25-8-203; 25-8-204; and 25-8-402; provide the specific statutory authority for adoption of these regulatory amendments. The commission also adopted in compliance with 24-4-103(4) C.R.S. the following statement of basis and purpose.

BASIS AND PURPOSE

- Temporary Modifications

Pursuant to the requirements in the Basic Standards (at 31.7(3)), the commission conducted its biennial review of the status of all temporary modifications to determine whether the temporary modifications should be modified, eliminated, or extended.

- Temporary Modifications for Standards Other than Arsenic

There is currently one segment with a temporary modification for a standard other than arsenic in Regulation No. 35. The commission deleted the following temporary modification:

Upper Gunnison River Segment 12 (COGUUG12): Mt. Emmons Mining Company (MEMC) provided an update to the commission on progress being made in implementing its plan to resolve uncertainty for the acute and chronic cadmium temporary modifications, which have numeric operative values of 3.5 and 2.79 µg/L, respectively, and apply April through June on Coal Creek (expire 12/31/2027).

The commission deleted the temporary modifications early, based on MEMC’s evidence that there is no longer a demonstrated or predicted water quality-based effluent limit (WQBEL) compliance issue.

The Coal Creek watershed has been impacted by numerous anthropogenic and natural loading sources of metals, including the historic Keystone Mine, which was mined between the 1880s and 1970s. MEMC’s parent company first acquired interests in the mine in the 1970s to evaluate the possibility of developing molybdenum resources. The Keystone Mine water treatment plant was completed in 1981 to treat and discharge flow from the Keystone Mine. However, no mining has occurred at the Keystone Mine since prior to MEMC’s involvement at the site, nor has any molybdenum extraction occurred there.

While the Keystone Mine site was transferred to U.S. Energy in 2006, MEMC reacquired the Keystone Mine site in 2016, formally taking on reclamation responsibility. This transfer was the subject of a Memorandum of Understanding (MOU) entered into with state, federal, and local interests, with the purpose of protecting the public health, safety, welfare, and environment, and to serve as a basis for future cooperation and agreement among the parties to the MOU.

In 2017, the commission adopted MEMC’s proposed revisions and extension of the chronic cadmium temporary modification and adopted a new acute cadmium temporary modification. Section 35.45(N). In 2022, the commission extended the cadmium temporary modifications by five years, to expire December 31, 2027. Section 35.51(B)(1).

Over the past decade, MEMC has undertaken significant reclamation efforts in coordination and collaboration with stakeholders, with additional work ongoing. This extensive work has included reclamation of mine waste; improvements in drainage controls; road improvements; tailings impoundment improvements; completion of upmountain diversions to divert stormwater flows around faults, veins, and fractures to promote a reduction of surface water and shallow groundwater infiltration into the historical mine workings and naturally mineralized areas; and best management practices employed throughout construction.

The improvements at the site have successfully resulted in improvements to MEMC’s effluent quality, eliminating the predicted acute and chronic cadmium compliance issues. Therefore, the commission deleted the acute and chronic cadmium temporary modifications early.

The commission also deleted the following temporary modifications, which had been allowed to expire:

Upper Gunnison River: 21 (uranium, expired 12/31/2025)

- Temporary Modifications for Arsenic

There are currently 66 segments with a temporary modification for arsenic in Regulation No. 35. The commission took no action on the arsenic temporary modification.

The Water Quality Control Division (division) provided an update to the commission on progress being made in implementing its plan to resolve uncertainty for the chronic arsenic temporary modification. This temporary modification applies to segments with an arsenic standard of 0.02 μg/L (to protect the Water + Fish use) and dischargers with demonstrated or predicted water quality-based effluent limit (WQBEL) compliance problems. The temporary modification was first adopted in 2011 (38.79), adopted more broadly throughout the state in 2013 (35.36), extended in 2019 (35.47(B)), and extended in 2024 (35.54(A)(2)) to expire 12/31/2029.

The commission retained the temporary modifications without revision. The division demonstrated continued widespread instream nonattainment, demonstrated or predicted WQBEL compliance issues, the need for time to continue working to resolve the remaining uncertainty regarding the water quality standard necessary to protect uses and the extent to which existing quality is the result of natural or irreversible human-induced conditions, and maintenance of status quo. The division continues to make progress on resolving the uncertainty underlying the temporary modifications, including the extent to which water quality improvements are feasible. The division will continue to provide annual progress updates and participate in the next biennial temporary modifications rulemaking hearing (currently planned for 2028).

- Discharger-specific Variances (DSVs)

The commission reviewed the basis, available information, and progress towards achieving the alternative effluent limits (AELs) and implementing a Pollutant Minimization Program (PMP) for one discharger-specific variance (DSV) in Regulation No. 35. Because the DSV for the Town of Nucla expires December 31, 2026, it was necessary to revisit this DSV in this rulemaking hearing.

San Miguel Segment 12c (COGUSM12c): The commission adopted a subsequent DSV for the Town of Nucla (CO0048945) for acute and chronic ammonia that represents the highest degree of protection of the classified use that is economically feasible for Nucla. This subsequent DSV replaces the remaining term of Nucla’s original DSV (35.43(B); adopted 10/11/2016 and expires 12/31/2026).

The proposed initial AEL for ammonia for Nucla’s wastewater treatment facility (WWTF) is 11.6 mg/L as a 30-day rolling average for February, March, April, May, June, July, and September. The rationale and process for determining this seasonal DSV period is described in detail in WQCD Prehearing Statement Exhibit F-11. This initial AEL does not lower currently attained ambient water quality and is considered a reasonable effluent ammonia level that is feasible for the WWTF to achieve consistently given the uncertainty surrounding treatment performance of the lagoon system. The proposed final AEL for ammonia is 9.3 mg/L as a 30-day rolling average, representing a 20% reduction from the initial AEL. Again, this AEL is applicable to the proposed DSV months of February, March, April, May, June, July, and September. This 20% reduction from the initial AEL represents the level of incremental water quality improvement that is reasonably expected to be achievable prior to the expiration of the DSV on December 31, 2031. The commission ensures that the discharge will not contribute to any lowering of the currently attained ambient water quality by adopting an initial AEL that, at a minimum, represents the level currently achieved, as stated by its rule at 31.7(4)(b)(i)(C).

Nucla has substantially complied with its previous DSV by implementing PMP activities, including biosolids removal, lagoon relining and baffling, restoration of the disinfection system, and installation of diffused aeration. These improvements have increased treatment efficiency and improved overall lagoon performance (WQCD Prehearing Statement Exhibit F-7).

The commission adopted a subsequent PMP with this DSV (WQCD Prehearing Statement Exhibit F-2), which describes the activities Nucla will complete during the term of this variance to reduce effluent ammonia concentrations. The PMP includes efforts to secure funding, collect water quality samples, submit annual reports to the division, and further investigate treatment alternatives. Additionally, there are activities included in the PMP that are contingent on Nucla obtaining funding through grants and loans, including improvements to the collection system. Delays in securing funding could affect project timelines and delay implementation of improvements. These actions will also help establish a path forward to implementing additional ammonia removal technologies in the future, if necessary.

A comprehensive alternatives analysis (WQCD Prehearing Statement Exhibit F-8) and a comprehensive economic analysis (WQCD Prehearing Statement Exhibits F-4 and F-5) demonstrated that there are currently no economically feasible alternatives that would allow Nucla to meet its ammonia water quality-based effluent limits (WQBELs) and compliance with these WQBELs would likely cause substantial and widespread adverse social and economic impacts to the community. Due to Nucla’s limited number of ratepayers, low median household income, high costs of wastewater treatment and clean water control measures per household, and high poverty rates, it is not feasible for Nucla to make the capital investments that would be required to meet the ammonia WQBELs at this time.

The commission expects that Nucla will submit annual reports to the division describing the progress made on PMP implementation in November of each year until the end of the DSV. If, at the end of the DSV, it remains infeasible for Nucla to achieve ammonia WQBELs and Nucla substantially complied with all conditions of this variance, a subsequent DSV may be appropriate.