|  | 1a. Mainstem of the South Platte River from a point immediately below the confluence with Big Dry Creek to the confluence with St. Vrain Creek.. COSPMS01A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Ammonia(acute) = See section 38.6(4) for site-specific standards.. *Ammonia(chronic) = See section 38.6(4) for site-specific standards.. *Copper(acute) = Copper BLM-based FMB Cu FMB(ac)=26.4 ug/l. *Copper(chronic) = Copper BLM-based FMB Cu FMB(ch)=18.0 ug/l. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *D.O. (mg/L)(acute) = See section 38.6(4) for site-specific standards.. *D.O. (mg/L)(chronic) = See section 38.6(4) for site-specific standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; varies*.; varies*.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS*.; TVS*.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; 26.4*.; ---.. Copper.; ---.; 18.0*.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 1b. Mainstem of the South Platte River from a point immediately below the confluence with St. Vrain Creek to the Weld/Morgan County Line.. COSPMS01B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 2. Deleted.. COSPMS02.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... 3a. All tributaries to the South Platte River, including all wetlands, from a point immediately below the confluence with Big Dry Creek to the Weld/Morgan County line, except for listings in the subbasins of the South Platte River, and in Segments 3b, 5a, 5b, 5c, and 6.. COSPMS03A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3b. Hayesmount Tributaries, including all tributaries, including the Upper Hayesmount Tributary from the source to the confluence with Box Elder Creek and the Lower Hayesmount Tributaries from the source to the Denver Hudson Canal.. COSPMS03B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *D.O. (mg/L)(chronic) = When water is present, D.O. concentrations shall be maintained at levels that protect classified uses...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; narrative*.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4. Barr Lake and Milton Reservoir.. COSPMS04.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5a. Mainstem of Lone Tree Creek from the source to the confluence with the South Platte River.. COSPMS05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5b. Mainstem of Box Elder Creek from the confluence with Coyote Run to the Denver Hudson Canal.. COSPMS05B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *D.O. (mg/L)(chronic) = 15th percentile of D.O. measurements collected between 6:30 a.m. and 6:30 p.m...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 4.7*.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5c. Mainstems of Crow Creek and Box Elder Creek from their sources to their confluences with the South Platte River, except for listings in Segment 5b.. COSPMS05C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Water Supply.. Recreation N... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6. Lost Creek from the source to Interstate 76, including all its tributaries, stock ponds and wetlands.. COSPMS06.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Beryllium(T).; ---.; 100.. Cadmium.; ---.; ---.. Cadmium(T).; ---.; 10.. Chromium III.; ---.; ---.. Chromium III(T).; ---.; 100.. Chromium VI.; ---.; ---.. Chromium VI(T).; ---.; 100.. Copper.; ---.; ---.. Copper(T).; ---.; 200.. Iron.; ---.; ---.. Lead.; ---.; ---.. Lead(T).; ---.; 100.. Manganese.; ---.; ---.. Manganese(T).; ---.; 200.. Mercury(T).; ---.; ---.. Molybdenum(T).; ---.; 150.. Nickel.; ---.; ---.. Nickel(T).; ---.; 200.. Selenium.; ---.; ---.. Selenium(T).; ---.; 20.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc.; ---.; ---.. Zinc(T).; ---.; 2000.... |
| --- | --- |

|  | 7. All lakes and reservoirs tributary to the South Platte River from a point immediately below the confluence with Big Dry Creek to the Weld/Morgan County line, except for listings in the subbasins of the South Platte River, and in segments 4 and 8.. COSPMS07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 8. Riverside Reservoir.. COSPMS08.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. Mainstem of the Big Thompson River, including all tributaries and wetlands, within Rocky Mountain National Park.. COSPBT01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2. Mainstem of the Big Thompson River from the boundary of Rocky Mountain National Park to the Greeley-Loveland Canal Diversion (40.397884, -105.106482). All tributaries to the Big Thompson River, including all wetlands, from the boundary of Rocky Mountain National Park to the Home Supply Canal diversion (40.424430, -105.210449).. COSPBT02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Copper(acute) = 11 ug/L on the mainstem of the Big Thompson River from immediately above the Upper Thompson Sanitation District’s wastewater treatment plant outfall to the Home Supply Canal Diversion. TVS applies to the rest of the segment.. *Copper(chronic) = 7.5 ug/L on the mainstem of the Big Thompson River from immediately above the Upper Thompson Sanitation District’s wastewater treatment plant outfall to the Home Supply Canal Diversion. TVS applies to the rest of the segment.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; ---.. Copper.; ---.; 7.5*.. Copper.; 11*.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3. Mainstem of the Big Thompson River from the Greeley-Loveland Canal diversion (40.397884, -105.106482) to County Road 11H.. COSPBT03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4. Mainstem of the Big Thompson River from County Road 11H to I-25.. COSPBT04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:.. Fish Ingestion Standards... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5. Mainstem of the Big Thompson River from I-25 to the confluence with the South Platte River.. COSPBT05.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Water Supply.. Recreation E... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6. All tributaries to the Big Thompson River, including all wetlands, from the Home Supply Canal diversion (40.424430, -105.210449) to the confluence with the South Platte River, except for listings in segments 7, 8, 9, and 10.. COSPBT06.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Water Supply.. Recreation E... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 7. Buckhorn Creek from the source to the confluence with the Big Thompson River.. COSPBT07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 8. Mainstem of the Little Thompson River, including all tributaries and wetlands, from the source to the Culver Ditch diversion (40.259242, -105.200029).. COSPBT08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 9. Mainstem of the Little Thompson River from the Culver Ditch diversion (40.259242, -105.200029) to the confluence with the Big Thompson River.. COSPBT09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 10. All tributaries to the Little Thompson River, including all wetlands, from the Culver Ditch diversion (40.259242, -105.200029) to the confluence with the Big Thompson River.. COSPBT10.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11. Carter Lake.. COSPBT11.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *Temperature = DM and MWAT=CLL from 1/1-3/31 DM=22.4 and MWAT=22.7 from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12. Lake Loveland, Horseshoe Lake, Boyd Lake.. COSPBT12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to Boyd Lake and Lake Loveland.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13. Berthoud Reservoir, Johnstown Reservoir.. COSPBT13.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply.. DUWS... Qualifiers:.. Water + Fish Standards... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14. Welch Reservoir, Lonetree Reservoir, Boedecker Lake, Lon Hagler Reservoir.. COSPBT14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to Lonetree Reservoir.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15. All lakes and reservoirs tributary to the Big Thompson River within Rocky Mountain National Park.. COSPBT15.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen; ---; TVS. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16. All lakes and reservoirs tributary to the Big Thompson River from the boundary of Rocky Mountain National Park to the Home Supply Canal diversion (40.424430, -105.210449). This segment includes Lake Estes and Mary’s Lake.. COSPBT16.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to Mary’s Lake and Mirror Lake.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 17. All lakes and reservoirs tributary to the Big Thompson River from the Home Supply Canal diversion (40.424430, -105.210449) to the confluence with the South Platte River, except for listings in segments 12, 14, 18, and 19.. COSPBT17.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply.. DUWS*... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to Pinewood Lake.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 18. All lakes and reservoirs tributary to the Little Thompson River from the source to the Culver Ditch diversion (40.259242, -105.200029).. COSPBT18.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 19. All lakes and reservoirs tributary to the Little Thompson River from the Culver Ditch diversion (40.259242, -105.200029) to the confluence with the Big Thompson River, except for listings in segments 11 and 13.. COSPBT19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. Mainstem of the Cache La Poudre River, including all tributaries and wetlands, within Rocky Mountain National Park and the Rawah, Neota, Comanche Peak, and Cache La Poudre Wilderness Areas.. COSPCP01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2a. Mainstem of the Cache La Poudre River, including all tributaries and wetlands, from the boundaries of Rocky Mountain National Park and the Rawah, Neota, Comanche Peak, and Cache La Poudre Wilderness Areas to a point immediately below the confluence with the South Fork Cache La Poudre River.. COSPCP02A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 2b. Mainstem of the Cache La Poudre River, including all tributaries and wetlands, from a point immediately below the confluence with the South Fork Cache La Poudre River to the Munroe Gravity Canal Headgate (also known as the North Poudre Supply Canal diversion; 40.691700, -105.255292), except for listings in segments 1 and 3.. COSPCP02B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3. Elkhorn Creek, including all tributaries and wetlands, from the source to a point immediately above the confluence with Manhattan Creek.. COSPCP03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4. Deleted.. COSPCP04.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... 5. Deleted.. COSPCP05.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... |
| --- | --- |

|  | 6. North Fork of the Cache La Poudre River, including all tributaries and wetlands, from the source to the inlet of Halligan Reservoir.. COSPCP06.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7. North Fork of the Cache La Poudre River, including all tributaries and wetlands, from the inlet of Halligan Reservoir to the confluence with the Cache La Poudre River, except for listings in segments 8 and 20.. COSPCP07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. Middle Fork Rabbit Creek, including all tributaries and wetlands, from the source to the confluence with Rabbit Creek. Stonewall Creek, including all tributaries and wetlands, from the source to the confluence with the North Fork of the Cache La Poudre River. North Fork Lone Pine Creek and South Fork Lone Pine Creek, including all tributaries and wetlands, from the source to the confluence with Lone Pine Creek.. COSPCP08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9. Deleted.. COSPCP09.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... |
| --- | --- |

|  | 10a. Mainstem of the Cache La Poudre River from the Munroe Gravity Canal Headgate (also known as the North Poudre Supply Canal diversion; 40.691700, -105.255292) to a point immediately above the Larimer County Ditch diversion (40.656612, -105.185244).. COSPCP10A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 10b. Mainstem of the Cache La Poudre River from a point immediately above the Larimer County Ditch diversion (40.656612, -105.185244) to Shields Street in Ft. Collins, Colorado.. COSPCP10B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11a. Mainstem of the Cache La Poudre River from Shields Street in Ft. Collins to immediately below the Timnath Reservoir Inlet (40.577031, -105.047436).. COSPCP11a.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; 1.; 2.7.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 11b. Mainstem of the Cache La Poudre River from immediately below the Timnath Reservoir Inlet (40.577031, -105.047436) to Prospect Road.. COSPCP11b.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 2.7.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12a. Mainstem of the Cache La Poudre River from Prospect Road to U.S. Hwy 85 in Greeley.. COSPCP12A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; 1.; 2.7.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 12b. Mainstem of the Cache La Poudre River from U.S. Hwy 85 in Greeley to the confluence with the South Platte River.. COSPCP12B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 2.7.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13a. All tributaries to the Cache La Poudre River, including all wetlands, from the Munroe Gravity Canal Headgate (also known as the North Poudre Supply Canal diversion; 40.691700, -105.255292) to the confluence with the South Platte River, except for listings in segments 6, 7, 8, 13b, and 13c.. COSPCP13A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13b. Mainstem of Boxelder Creek from its source to a point immediately above Slab Canyon Wash. Mainstems of South Branch of Boxelder Creek, North Branch of Boxelder Creek, and Sand Creek from their sources to their confluences with the mainstem of Boxelder Creek.. COSPCP13B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13c. Mainstem of Boxelder Creek from a point immediately above Slab Canyon Wash to the confluence with the Cache La Poudre River.. COSPCP13C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 14. Horsetooth Reservoir.. COSPCP14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *Temperature = DM=CLL and MWAT=CLL from 1/1-3/31 DM=CLL and MWAT=22.8 from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*.; B.. acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 15. Watson Lake.. COSPCP15.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 16. Reservoir #4 (40.719045, -105.033743), Water Supply Reservoir #3 (40.665205, -105.089882), Claymore Lake, College Lake, Dixon Reservoir, Robert Benson Lake, Black Hollow Reservoir, Seeley Lake.. COSPCP16.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Nitrogen(chronic) = applies only above the facilities listed at 38.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 17. All lakes and reservoirs tributary to the Cache La Poudre River within Rocky Mountain National Park and the Rawah, Neota, Comanche Peak, and Cache La Poudre Wilderness Areas.. COSPCP17.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 18. All lakes and reservoirs tributary to the Cache La Poudre River from the boundaries of Rocky Mountain National Park and the Rawah, Neota, Comanche Peak, and Cache La Poudre Wilderness Areas to the Munroe Gravity Canal Headgate (also known as the North Poudre Supply Canal diversion; 40.691700, -105.255292).. COSPCP18.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *Temperature = See 38.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 19. All lakes and reservoirs tributary to the North Fork of the Cache La Poudre River from the source to the inlet of Halligan Reservoir.. COSPCP19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Nitrogen(chronic) = applies only above the facilities listed at 38.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 20. All lakes and reservoirs tributary to the North Fork of the Cache La Poudre River from the inlet of Halligan Reservoir to the confluence with the Cache La Poudre River. This segment includes Halligan Reservoir and Seaman Reservoir.. COSPCP20.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Nitrogen(chronic) = applies only above the facilities listed at 38.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *Temperature = DM and MWAT=CL,CLL from 1/1-3/31 Seaman Reservoir DM=CLL and MWAT=22.5 from 4/1-12/31 All others DM and MWAT=CL,CLL from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 21. All lakes and reservoirs tributary to the Cache La Poudre River from the Munroe Gravity Canal Headgate (also known as the North Poudre Supply Canal diversion; 40.691700, -105.255292) to the confluence with the South Platte River, except for listings in segments 14, 15, 16, 19, 20, 22, and 23.. COSPCP21.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to North Poudre Reservoir No. 3.. *Nitrogen(chronic) = applies only above the facilities listed at 38.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 22. Fossil Creek Reservoir.. COSPCP22.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:.. Fish Ingestion Standards... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |
| 23. Douglas Reservoir. |  |  |
| COSPCP23.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details... | Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002... | Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS... |

|  | 1. All tributaries to the Laramie River, including all wetlands, which are within the Rawah Wilderness Area.. COSPLA01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2a. Mainstem of the Laramie River from the source to the National Forest boundary, and all tributaries and wetlands from the source to the Colorado/Wyoming border, except for listings in Segment 1.. COSPLA02A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 2b. Mainstem of the Laramie River from the National Forest boundary to the Colorado/Wyoming border.. COSPLA02B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3. All lakes and reservoirs tributary to the Laramie River within the Rawah Wilderness Area.. COSPLA03.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4. All lakes and reservoirs tributary to the Laramie River from the source to the Colorado/Wyoming border, except for listings in Segment 3.. COSPLA04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1a. Mainstem of the South Platte River from the Weld/Morgan County line to the Morgan/Washington County line.. COSPLS01A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 1b. Mainstem of the South Platte River from the Morgan/Washington County line to the Colorado/Nebraska border.. COSPLS01B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 2. All tributaries to the South Platte River, including all wetlands, from the Weld/Morgan County line to the Colorado/Nebraska border.. COSPLS02.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. Discharger Specific Variance(s):.. Ammonia(ac/ch) = See Section 38.6(6) for details on the variance for the Town of Crook.. Expiration Date of 12/31/2031.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Beryllium(T).; ---.; 4.0.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3. Jackson Reservoir, Prewitt Reservoir, North Sterling Reservoir, Jumbo (Julesburg), Empire Reservoir, Vancil Reservoir.. COSPLS03.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Nitrogen(chronic) = applies only above the facilities listed at 38.5(4)).. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4). *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *Temperature = See 38.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4. All lakes and reservoirs tributary to the South Platte River from the Weld/Morgan County line to the Colorado/Nebraska border, except for listings in Segment 3.. COSPLS04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Nitrogen(chronic) = applies only above the facilities listed at 38.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Beryllium(T).; ---.; 4.0.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. Mainstem of the South Fork of the Republican River from a point 23 miles above the Colorado/Kansas border (39.582154, -102.350838) to the Colorado/Kansas border.. COSPRE01.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2. Deleted.. COSPRE02.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... |
| --- | --- |

|  | 3. Mainstem of the North Fork of the Republican River from the source to the Colorado/Nebraska border. Mainstem of Chief Creek from the source to the confluence with the North Fork of the Republican River.. COSPRE03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4. Mainstem of the Arikaree River from the confluence of the North and South Forks to the Colorado/Kansas border.. COSPRE04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5. Mainstem of Black Wolf Creek from the source to the confluence with the Arikaree River.. COSPRE05.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6. All tributaries to the Republican River system in Colorado, including all wetlands, except for listings in segments 1, 3, 4 and 5.. COSPRE06.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Beryllium(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 7. Mainstem of the North Fork of the Smoky Hill River and mainstem of the Smoky Hill River, including all tributaries and wetlands, from the source to the Colorado/Kansas border.. COSPRE07.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Beryllium(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 8. All lakes and reservoirs tributary to the Republican River and Smoky Hill River in Colorado.. COSPRE08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Nitrogen(chronic) = applies only above the facilities listed at 38.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Beryllium(T).; ---.; 4.0.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

STREAM CLASSIFICATIONS and WATER QUALITY STANDARDS – FOOTNOTES

(A)	Whenever a range of standards is listed and referenced to this footnote, the first number in the range is a strictly health-based value, based on the Commission’s established methodology for human health-based standards. The second number in the range is a maximum contaminant level, established under the federal Safe Drinking Water Act that has been determined to be an acceptable level of this chemical in public water supplies, taking treatability and laboratory detection limits into account. Control requirements, such as discharge permit effluent limitations, shall be established using the first number in the range as the ambient water quality target, provided that no effluent limitation shall require an “end-of-pipe” discharge level more restrictive than the second number in the range. Water bodies will be considered in attainment of this standard, and not included on the Section 303(d) List, so long as the existing ambient quality does not exceed the second number in the range.

(B) 	Assessment of adequate refuge shall rely on the Cold Large Lake table value temperature criterion and applicable dissolved oxygen standard rather than the site-specific temperature standard.