DEPARTMENT OF PUBLIC HEALTH AND ENVIRONMENT

Air Quality Control Commission

# REGULATION NUMBER 29

# Emission Reduction Requirements for Lawn and Garden Equipment

# 5 CCR 1001-33

[Editor’s Notes follow the text of the rules at the end of this CCR Document.]

_________________________________________________________________________

Outline of Regulation

PART A	Emission Reduction Requirements for Lawn and Garden Equipment

PART B	Statement of Basis, Specific Statutory Authority, and Purpose

Pursuant to Colorado Revised Statutes § 24-4-103 (12.5), materials incorporated by reference are available for public inspection during normal business hours, or copies may be obtained at a reasonable cost from the Air Quality Control Commission (the Commission), 4300 Cherry Creek Drive South, Denver, Colorado 80246-1530. The material incorporated by reference is also available through the United States Government Printing Office, online at www.govinfo.gov. Materials incorporated by reference are those editions in existence as of the date indicated and do not include any later amendments.

PART A	Emission Reduction Requirements for Lawn and Garden Equipment

I.	Applicability and general provisions

I.A.	This part applies to the federal government, state government agencies, local governments, and commercial lawn and garden operators that use lawn and garden equipment, as defined in Section II.

I.B.	The use restrictions in Section III. do not apply to lawn and garden equipment:

I.B.1.	Used for the purpose of abating or preventing damage during a declared emergency or equipment used by first responders to provide emergency services.

I.B.2.	Used for the purpose of fire hazard reduction and post-fire recovery activities in or near the wildland areas or the wildland urban interface.

I.B.3.	Used for the purpose of riparian, forest, or grassland management.

I.B.4.	Used for public safety purposes.

I.B.5.	Nothing in this Section I.B. limits the applicability of the recordkeeping and reporting provisions in Section IV.I.C. Severability. If any section, clause, phrase, or standard contained in these regulations is for any reason held to be inoperative, unconstitutional, void, or invalid, the validity of the remaining portions thereof will not be affected and the Commission declares that it severally passed and adopted these provisions separately and apart.

II.	Definitions

II.A.	“Commercial lawn and garden operator” means a corporation, partnership, limited partnership, proprietorship, sole proprietorship, firm, enterprise, or association that maintains an outdoor property for compensation or that performs a commercial activity relating to providing lawn and garden services.

II.B.	“Commercial activity” means activities supplying a service undertaken with the goal of profit-making. Commercial activity does not apply to attempts at profit-making by a person under the age of 18, provided they are not performing a commercial activity for an entity owned, controlled, or directed by someone over the age of 18.

II.C.	“Federal government” means each department, agency, and instrumentality of the executive, legislature, and judicial branches of the United States government, and each officer, agent, or employee thereof, required to comply with State regulation of air pollution as set forth in 42 U.S.C. § 7418 (1990).

II.D.	“Landscaping” means decorative or protective vegetation that enhances appearance surrounding buildings or roadways; areas that enhance appearance and create useable space for outdoor activities around a home; a planned outdoor space set aside for cultivation, display, and enjoyment of herbs, fruits, flowers, vegetables, trees, or ornamental shrubs.

II.E.	“Lawn and garden equipment” means equipment whose primary purpose is to assist with cleanup or maintenance of a lawn or garden area of a property. Examples of this type include, but are not limited to:

II.E.1.	Aerators.

II.E.2.	Brush cutters.

II.E.3.	Chainsaws.

II.E.4.	Dethatchers.

II.E.5.	Edgers.

II.E.6.	Generators, when used for lawn and garden services (e.g., charging or operating electric equipment).

II.E.7.	Hedge trimmers.

II.E.8.	Leaf blowers.

II.E.9.	Power washers.

II.E.10.	Push or walk-behind lawn mowers.

II.E.11.	Pruners (e.g., pole saws).

II.E.12.	Rotary tillers.

II.E.13.	String trimmers.

II.E.14.	Wood splitters.

II.F.	“Lawn and garden services” means landscaping services, grass/lawn mowing, weeding, grass/lawn trimming, removal and disposal of debris and trash, leaf cleanup and removal, planting or maintenance of any plants (e.g., trees, bushes, hedges, shrubs, flowers, other plants). Lawn and garden services do not include activities such as forest or grassland management.

II.G.	“Local government” means municipalities, county governments, city and county governments, public school districts, and special districts.

II.H.	“Municipality” means a city or town as defined in § 31-1-101(6), C.R.S., (2023).

II.I.	“Ozone nonattainment area” means an area within Colorado designated by the Commission and approved by the U.S. Environmental Protection Agency under the Code of Federal Regulations Title 40, § 81.306 (July 24, 2024), in which ambient air concentrations exceed the National Ambient Air Quality Standards for ozone.

II.J.	“Special district” means a quasi-municipal corporation or political subdivision as defined in § 32-1-103(20), C.R.S., (2023).

II.K.	“State government agency” means any agency, board, bureau, commission, department, division, institution, or office of the executive or judicial departments of state government, including institutions of higher education, located within the state of Colorado.

III.	Use restrictions

III.A.	Beginning June 1, 2025, no state government agency in Colorado can use gasoline-powered push and hand-held lawn and garden equipment with an internal combustion engine smaller than 19 kW (25 horsepower) between June 1 and August 31 of each year.

III.B.	Beginning June 1, 2025, neither the federal government nor any local government can use gasoline-powered push and held-held lawn and garden equipment with an internal combustion engine smaller than 7 kW (10 horsepower) between June 1 and August 31 of each year in the ozone nonattainment area.

III.C	The restrictions in Sections III.A. and III.B. also apply to lawn and garden services contracted for and provided to the federal government, a state government agency, or a local government.

III.D.	Commercial lawn and garden operators in Colorado must not use more than the specified percentage of covered equipment when conducting lawn and garden services in the ozone nonattainment area between June 1 and August 31 of each year, in accordance with Table 1.

III.D.1.	Covered equipment includes gasoline-powered push and hand-held string trimmers, hedge trimmers, blowers, edgers, pruners, and push or walk-behind lawn mowers with an internal combustion engine smaller than 7 kW (10 horsepower).

III.D.2.	The compliance percentages listed in Table 1 specify the allowable proportion of a commercial lawn and garden operator’s total covered equipment that may be used between June 1 through August 31 (e.g., in 2027, a commercial lawn and garden operator may not use covered equipment in a quantity that exceeds 90 percent of the total number of pieces of equipment from the company’s inventory).

| Table 1: Use restriction schedule for covered equipment use |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Year | 2027 | 2028 | 2029 | 2030 | 2031 | 2032 |
| Use restriction percentage | 90% | 75% | 60% | 40% | 20% | 10% |

IV.	Recordkeeping and reporting

IV.A.	State government agencies, local governments, the federal government, and commercial lawn and garden operators must maintain records for five (5) years demonstrating compliance with Sections III.A. through III.C. Records must be made available to the Division upon request.

IV.B.	On or before June 1, 2026, June 1, 2027, March 1, 2028, and March 1 of each year thereafter, all state government agencies, local governments, and the federal government conducting or contracting for lawn and garden services subject to Section III. must submit information for the preceding calendar year (e.g., by June 1, 2026, report, submit information for the period of June 1, 2025, through August 31, 2025) using a Division-approved format. The report must include:

IV.B.1.	A list of all gasoline-powered push and hand-held lawn and garden equipment with an internal combustion engine smaller than 19 kW (25 horsepower) used or potentially used by the state government agency from June 1 to August 31.

IV.B.2.	A list of all gasoline-powered push and hand-held lawn and garden equipment with an internal combustion engine smaller than 7 kW (10 horsepower) used or potentially used by the federal government or local government within the ozone nonattainment area from June 1 to August 31.

IV.B.3.	For the equipment listed in Section IV.B.1. and IV.B.2.,

IV.B.3.a.	The lawn and garden equipment type, and horsepower, manufacturer.

IV.B.3.b.	For gasoline-powered equipment used during the June 1 through August 31 time period, documentation demonstrating the circumstances requiring the use of such equipment such as supply chain issues, need for heavy-duty scale equipment, or a purpose listed in Section I.B.

IV.B.4.	The company name and designated contact person for the lawn and garden services contractor(s), if applicable, and a description of the services (e.g., list of activities, duration, frequency, expected equipment use) provided.

IV.C.	Beginning on or before March 1, 2028, and March 1 of each year thereafter, commercial lawn and garden operators must submit information for the preceding calendar year (e.g., by March 1, 2028, report, submit information for the period of June 1, 2027, through August 31, 2027) using a Division-approved format. The report must include:

IV.C.1.	A list of the push and hand-held string trimmers, hedge trimmers, blowers, edgers, pruners, and push or walk behind lawn mowers used by the commercial lawn and garden operator within the ozone nonattainment area from June 1 to August 31. Gasoline-powered equipment must be included only if the equipment has an internal combustion engine smaller than 7 kW (10 horsepower).

IV.C.2.	For the equipment listed in Section IV.C.1.,

IV.C.2.a.	The lawn and garden equipment type, horsepower, manufacturer, and documentation demonstrating the circumstances requiring the use of such equipment for a purpose listed in Section I.B.

IV.C.2.b.	For non-gasoline-powered equipment, the lawn and garden equipment type and manufacturer.

IV.D.	Each report must be accompanied by a certification by a responsible official that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.

PART B	STATEMENTS OF BASIS, SPECIFIC STATUTORY AUTHORITY AND PURPOSE

>>>>>>>

II.	Adopted: DATE, 2026

This Statement of Basis, Specific Statutory Authority, and Purpose complies with the requirements of the Colorado Administrative Procedure Act § 24-4-103, C.R.S., the Colorado Air Pollution Prevention and Control Act §§ 25-7-110 and 25-7-110.5, C.R.S., (the State Air Act) and the Air Quality Control Commission’s (Commission) Procedural Rules, 5 Code Colo. Reg. § 1001-1.

Basis

On October 7, 2022, EPA reclassified the Denver Metro/North Front Range (DM/NFR) to severe for the 2008 8-hour Ozone National Ambient Air Quality Standard of 75 parts per billion (ppb) (2008 ozone NAAQS), after 2019-2021 ozone data failed to show attainment. See 87 Fed. Reg. 60926. Separately, EPA has also designated the DM/NFR as marginal nonattainment for the 2015 ozone NAAQS of 70 ppb, effective August 3, 2018 (83 Fed. Reg. 25776 (June 4, 2018)). On October 7, 2022, EPA reclassified the DM/NFR and northern Weld County to moderate for the 2015 ozone NAAQS, after 2019-2021 ozone data failed to show attainment. See 87 Fed. Reg. 60897. On July 24, 2024, EPA reclassified the DM/NFR and northern Weld County to serious for the 2015 ozone NAAQS, based on a voluntary reclassification request submitted by Colorado. See 89 Fed. Reg. 59832.

In a continued effort to reduce ozone precursor emissions, in 2024, the Commission adopted a new Regulation Number 29, Emission Reduction Requirements for Lawn and Garden Equipment to limit the use of certain gasoline-powered lawn and garden equipment statewide by state agencies and specifically in the ozone nonattainment area by local government and federal agencies during June through August; thereby reducing the ozone precursors emissions emitted from this equipment.

In this rulemaking, the Commission adopted a use-restriction schedule for commercial lawn and garden operators’ use of certain covered equipment in the ozone nonattainment area during the summer months (June 1 through August 31). The adopted use restrictions are based on electrification market trends and stakeholder input received during and following the last rulemaking hearing. The restrictions are expected to achieve reductions in emissions from smaller push and hand-held gasoline-powered lawn and garden equipment.

Specific Statutory Authority

The State Air Act, specifically § 25-7-105(1), C.R.S., directs the Commission to promulgate such rules and regulations as are consistent with the legislative declaration set forth in § 25-7-102, C.R.S., and that are necessary for the proper implementation and administration of Article 7. Section 25-7-106, C.R.S., provides the Commission maximum flexibility in developing an effective air quality program and promulgating such combination of regulations as may be necessary or desirable to carry out that program. Specifically, § 25-7-106(1)(c), C.R.S., authorizes the Commission to promulgate emission control regulations applicable to the entire state, specified areas or zones, or when a specified class of pollution is present. Section 25-7-106(6), C.R.S., further authorizes the Commission to require owners and operators of any air pollution source to monitor, record, and report information. Sections 109(1)(a) and (2) authorize the Commission to promulgate regulations requiring effective practical air pollution controls for significant sources and categories of significant sources, including regulations pertaining to nitrogen oxides and hydrocarbons. Sections 25-7-105(1)(a)(I) and 25-7-301, C.R.S., authorize the Commission to promulgate a SIP which will assure attainment and maintenance of national ambient air quality standards and prevent significant deterioration of air quality in conformance with Article 7 of the State Air Act.

Purpose

In a continued effort to reduce ozone precursor emissions, the Commission adopted strategies to reduce emissions from gasoline-powered lawn and garden equipment used by commercial lawn and garden operators in the ozone nonattainment area.

Use Restrictions

The Commission adopted in-use restrictions on the use of certain smaller push and hand-held gasoline-powered lawn and garden equipment. Covered equipment includes gasoline-powered push and hand-held string trimmers, hedge trimmers, blowers, edgers, pruners, and push or walk-behind lawn mowers with an internal combustion engine smaller than 7 kW (10 horsepower). The use restriction applies to any commercial lawn and garden operator that operates covered equipment in the ozone nonattainment area for any period of time between June 1 to August 31. The Commission adopted these restrictions considering electrification market share research reported in November of 2025, which suggested electrification availability of gasoline-powered equipment as well as high demand for such equipment by commercial operators. Alternative equipment that may be used during the summer months include electric and manually operated lawn and garden equipment, which is already readily available. The Commission adopted a tiered use-restriction schedule based on the percentage of gas-powered equipment that may be used each year to ensure commercial operators have an ample compliance timeline. The Commission understands that commercial lawn and garden operators may also be able to defer lawn and garden activities to periods outside of the June through August timeframe, alleviating the need to acquire or use alternative equipment.

Aligned with the previously adopted provisions for government entities, the use restriction does not apply to activities conducted by subject entities for the purposes of preventing or abating damage such as forest, riparian, noxious weed, or grassland management; emergency response; public safety; or fire hazard reduction.

Annual Reporting

The Commission adopted similar annual reporting obligations for commercial lawn and garden operators as those previously adopted for government entities. Beginning March 1, 2028, commercial operators are required to report the push and hand-held string trimmers, hedge trimmers, blowers, edgers, pruners, and push or walk-behind lawn mowers, identified as gasoline-powered or non-gasoline-powered, that were used in the ozone nonattainment area during the ozone season (June 1 through August 31). This ensures compliance with use restriction requirements and allows the Division to collect information that will inform future lawn and garden emission reduction strategies.

Although the Commission did not extend the use-restriction to activities done for the purpose of preventing or abating damage; it did adopt reporting obligations surrounding these activities to better understand the equipment needed under other special circumstances.

Further, this will include any typographical, grammatical, and formatting errors throughout the regulation.

Incorporation by Reference

Section 24-4-103(12.5), C.R.S., of the State Administrative Procedure Act allows the Commission to incorporate by reference rules and regulations. The criteria of § 24-4-103(12.5), C.R.S., are met by including specific information and making the regulations available because repeating the full text of each of the rules or regulations incorporated would be unduly cumbersome and inexpedient. To fully comply with these criteria, the Commission includes reference dates to rules and reference methods incorporated in Regulation Number 29.

Additional Considerations

Colorado must continue to reduce ozone concentrations to attain both the 2008 ozone NAAQS and the 2015 ozone NAAQS. The federal Clean Air Act (CAA) does not expressly address all of the provisions adopted by the Commission. Rather, federal law establishes the ozone NAAQS and requires Colorado to develop a SIP adequate to attain the NAAQS. Therefore, the Commission adopted certain revisions to Regulation Number 29 to make progress towards attainment of the 2008 and 2015 ozone NAAQS. These revisions do not exceed or differ from the federal act due to state flexibility in determining what control strategies to implement to reduce emissions. However, where the proposal may differ from federal rules under the federal act, in accordance with § 25-7-110.5(5)(b), CRS, the Commission determines:

- Regulation Number 29 restricts the use of certain lawn and garden equipment within a specified time of year. There are federal emission standards for some lawn and garden equipment but there are no comparative federal use restrictions.

- The federal emissions standards discussed in (I) are primarily technology-based in that they largely prescribe the use of specific technologies to comply.

- The CAA establishes the 2008 and 2015 NAAQS and requires Colorado to develop SIP revisions that will ensure attainment of the NAAQS. The ozone NAAQS were not determined taking into account concerns unique to Colorado. Similarly, EPA develops equipment standards considering national information and data, not Colorado specific issues or concerns.

- In addition to the 2008 NAAQS, Colorado must also comply with the lower 2015 ozone NAAQS. Regulation Number 29 may improve the ability of the regulated community to comply with new requirements insofar as the estimated emission reductions may prevent or reduce the need to adopt additional emission reduction strategies into the SIP for the more stringent NAAQS. The emission reductions estimated from the implementation of Regulation Number 29 will also improve public health and environmental quality.

- EPA has established Colorado’s ozone attainment deadlines. There is no timing issue that might justify changing the time frame for implementation of federal requirements.

- The revisions to Regulation Number 29 address emissions from lawn and garden equipment in a cost-effective manner, allowing for continued growth of Colorado’s industry.

- The revisions to Regulation Number 29 establish reasonable equity for owners and operators subject to these rules by providing the same standards for similarly situated entities.

- If Colorado does not attain the ozone standards, EPA will further reclassify Colorado’s ozone nonattainment areas. This outcome may subject others to increased costs due to the implementation of any additional, necessary permitting actions and/or emission reduction strategies.

- Where necessary, the revisions to Regulation Number 29 include minimal monitoring, recordkeeping, and reporting requirements that correlate, where possible, to similar federal or state requirements.

- Demonstrated technology is available to comply with Regulation Number 29.

- As set forth in the Economic Impact Analysis, the revisions to Regulation Number 29 will reduce emissions in a cost-effective manner.

- Alternative rules could also provide reductions in VOC and NOx to help to attain the NAAQS. However, a no action alternative would not achieve the reductions from lawn and garden equipment associated with Regulation Number 29.

As part of adopting the revisions to Regulation Number 29, the Commission has taken into consideration each of the factors set forth in § 25-7-109(1)(b), C.R.S.

Colorado must continue to reduce ozone concentrations to attain both the 2008 and 2015 ozone NAAQS. To the extent that § 25-7-110.8, C.R.S requirements apply to this rulemaking, and after considering all the information in the record, the Commission hereby makes the determination that:

- These rules are based upon reasonably available, validated, reviewed, and sound scientific methodologies, and the Commission has considered all information submitted by interested parties.

- Evidence in the record supports the finding that the rules shall result in a demonstrable reduction of VOCs and NOx emissions.

- Evidence in the record supports the finding that the rules shall bring about reductions in risks to human health and the environment that justify the costs to implement and comply with the rules.

- The rules are the most cost-effective to achieve the necessary and desired results, provide the regulated community flexibility, and achieve the necessary reduction in air pollution.

- The rule will maximize the air quality benefits of regulation in the most cost-effective manner.