COLORADO DEPARTMENT OF PUBLIC HEALTH AND ENVIRONMENT

WATER QUALITY CONTROL COMMISSION

5 CCR 1002-35

REGULATION NO. 35

CLASSIFICATIONS AND NUMERIC STANDARDS
FOR
GUNNISON AND LOWER DOLORES RIVER BASINS

APPENDIX 35-1

Stream Classifications and Water Quality Standards Tables

Effective 12/31/2026

### Abbreviations and Acronyms

Aq	=	Aquatic
°C	=	degrees Celsius
CL	=	cold lake temperature tier
CLL	=	cold large lake temperature tier
CS-I	=	cold stream temperature tier one
CS-II	=	cold stream temperature tier two
D.O. 	=	dissolved oxygen
DM	=	daily maximum temperature
DUWS	=	direct use water supply
E. coli 	=	Escherichia coli
EQ	=	existing quality
mg/L 	=	milligrams per liter
mg/m2	=	milligrams per square meter
mL	=	milliliter
MWAT	=	maximum weekly average temperature
OW 	=	outstanding waters
sc	=	sculpin
SSE	=	site-specific equation
T	=	total recoverable
t	=	total
tr	=	trout
TVS	=	table value standard
µg/L	=	micrograms per liter
UP	=	use-protected
WS	=	water supply
WS-I	=	warm stream temperature tier one
WS-II	=	warm stream temperature tier two
WS-III	=	warm stream temperature tier three
WL	=	warm lake temperature tier

|  | 1. All tributaries and wetlands to the Gunnison River within the La Garita, Powderhorn, West Elk, Collegiate Peaks, Maroon Bells, Raggeds, Fossil Ridge, or Uncompahgre Wilderness Areas.. COGUUG01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.02.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2. All tributaries and wetlands from Beaver Creek to Meyers Gulch, from the West Elk Wilderness boundary to their confluences with Blue Mesa Reservoir, Morrow Point Reservoir, or the Gunnison River, excluding Steuben Creek and Willow Creek and their tributaries.. COGUUG02.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.02.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3. Mainstem of the Taylor River, including all tributaries and wetlands, from the source to a point immediately below the confluence with Illinois Creek, except for listings in Segment 1.. COGUUG03.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4. Mainstem of the Taylor River, including all tributaries and wetlands, from a point immediately below the confluence with Illinois Creek to the confluence with the Gunnison River, except for listings in Segment 1.. COGUUG04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5a. Mainstem of the East River, including all tributaries and wetlands, from its source to a point immediately above the confluence with the Slate River, except for specific listings in Segment 1.. COGUUG05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5b. Mainstem of the East River from a point immediately above the Slate River to the confluence with the Gunnison River.. COGUUG05B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6a. All tributaries and wetlands to the East River from a point immediately above its confluence with the Slate River to its confluence with the Gunnison River, except for listings in Segments 6b and 6c.. COGUUG06A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation U... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6b. Cement Creek, including all tributaries and wetlands, from the source to a point immediately above the confluence with Horse Basin Creek.. COGUUG06B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6c. Cement Creek, including all tributaries and wetlands, from a point immediately above the confluence with Horse Basin Creek to the confluence with the East River.. COGUUG06C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7. Mainstem of the Slate River from its source to a point immediately above the confluence with Coal Creek.. COGUUG07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. Mainstem of the Slate River from a point immediately above the confluence with Coal Creek to the confluence with the East River.. COGUUG08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = summer criteria apply from 6/1-10/15..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I*.; CS-I*.; C.. acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9. All tributaries and wetlands to the Slate River except for specific listings in Segments 1, 10a, 10b, 11, 12 and 13.. COGUUG09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 210.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10a. Mainstem of Oh-Be-Joyful Creek from the boundary of the Raggeds Wilderness Area to the confluence with the Slate River.. COGUUG10A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; 8.6.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 10b. Mainstem of Redwell Creek, including all tributaries and wetlands, from the source to the confluence with Oh-Be-Joyful Creek.. COGUUG10B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; 407.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11. Mainstem of Coal Creek from a point immediately above the confluence with Elk Creek to a point immediately above the Keystone Mine discharge (38.867117, -107.023627). Elk Creek and its tributaries and wetlands from its source to its confluence with Coal Creek.. COGUUG11.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 210.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12. Mainstem of Coal Creek, including all tributaries and wetlands, from a point immediately above the Keystone Mine discharge (38.867117, -107.023627) to the confluence with the Slate River, with the exception of Wildcat Creek.. COGUUG12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/191.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13. Mainstem of Woods Creek from the source to the confluence with Washington Gulch.. COGUUG13.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14. Mainstem of the Gunnison River from its inception at the confluence of the East and Taylor rivers to the inlet of Blue Mesa Reservoir.. COGUUG14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15a. All tributaries and wetlands to the Gunnison River from its inception at the confluence of the East and Taylor Rivers to the County Road 32 road crossing near the inlet of Blue Mesa Reservoir except for the specific listings in Segments 1, 15b, 16a, 16b, 17 through 24, and 26.. COGUUG15A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation U.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1950.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 15b. South Beaver Creek, including all tributaries and wetlands, from the source to the Saguache/Gunnison County line.. COGUUG15B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 16a. Mainstem of Ohio Creek, including all tributaries and wetlands, from the source to a point immediately below 7 Road, except for listings in Segment 1.. COGUUG16A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16b. Mainstem of Ohio Creek from a point immediately below 7 Road to the confluence with the Gunnison River.. COGUUG16B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = summer criteria apply from 4/16 - 11/15..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I*.; CS-I*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 17a. West Antelope Creek, including all tributaries and wetlands, from the source to the confluence with Antelope Creek.. COGUUG17A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 17b. Mainstem of Antelope Creek, including all tributaries and wetlands, from the source to the confluence with the Gunnison River, excluding the listings in Segment 17a.. COGUUG17B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 18a. Mainstem of Tomichi Creek and its wetlands from the source to the confluence with Porphyry Creek.. COGUUG18A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 18b. Mainstem of Tomichi Creek and its wetlands from the confluence with Porphyry Creek to the confluence with the Gunnison River.. COGUUG18B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = DM and MWAT=CS-II from 11/1-3/31 DM=CS-II and MWAT=18.9 from 4/1-10/31 See temperature assessment locations at 35.6(6)...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*.; C.. acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 19. All tributaries to Tomichi Creek, including wetlands, which are within the boundaries of the Gunnison National Forest, except for specific listings in Segments 20 through 24. Mainstems of Barret, Razor, and Quartz Creeks from their sources to their confluences with Tomichi Creek. Hot Springs Creek from its source to the inlet of Hot Springs Reservoir.. COGUUG19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 20. Mainstem of Indian Creek, including all tributaries and wetlands, from the source to the confluence with Marshall Creek.. COGUUG20.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = lowest practical level. *Uranium(chronic) = lowest practical level..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; LPL*.; LPL*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 21. Mainstem of Marshall Creek, including all tributaries and wetlands, from the source to the confluence with Tomichi Creek, except for listings in Segment 20.. COGUUG21.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 22. Mainstem of Gold Creek from Browns Gulch to the confluence with Quartz Creek.. COGUUG22.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 23. Mainstem of Cochetopa Creek, including all tributaries and wetlands, from the source to a point immediately below the confluence with West Pass Creek with the exception of Segment 1.. COGUUG23.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 24. Mainstem of Cochetopa Creek from a point immediately below the confluence with West Pass Creek to the confluence with Tomichi Creek.. COGUUG24.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 25. The segments of the Gunnison River which interconnect Blue Mesa Reservoir, Morrow Point Reservoir, and Crystal Reservoir.. COGUUG25.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 26. All tributaries, including wetlands, which are tributary to the Gunnison River from County Road 32 to the inlet of Blue Mesa Reservoir, Blue Mesa Reservoir, Morrow Point Reservoir, Crystal Reservoir, or the segments of the Gunnison River that interconnect those reservoirs, except for specific listings in Segments 1, 2, 29a, 29b, 30, 31, and 32.. COGUUG26.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation U.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 27. Deleted.. COGUUG27.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... 28. Deleted.. COGUUG28..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... |
| --- | --- |

|  | 29a. Mainstem of the Lake Fork of the Gunnison including all tributaries and wetlands, from the source to a point immediately above the confluence with Eaton Creek. Cebolla Creek, including all tributaries and wetlands, from the source to the Hinsdale/Gunnison County line. Powderhorn Creek, including all tributaries and wetlands, from the source to the confluence with Cebolla Creek. This segment excludes the specific listings in Segments 1, 29b, 30, 31, and 32.. COGUUG29A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 29b. Mainstem of the Lake Fork of the Gunnison, including all tributaries and wetlands, from a point immediately above the confluence with Eaton Creek, to Blue Mesa Reservoir. Cebolla Creek, including all tributaries and wetlands, from the Hinsdale/Gunnison County line, to Blue Mesa Reservoir, excluding the listings in Segment 29a.. COGUUG29B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 30. Mainstem of Henson Creek, including all tributaries and wetlands, from the source to the confluence with the Lake Fork of the Gunnison, except for the specific listings in Segments 31 and 32.. COGUUG30.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 31. Mainstem of Palmetto Gulch Creek, including all tributaries and wetlands.. COGUUG31.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 32. North Fork of Henson Creek including all tributaries and wetlands, from its source to the confluence with Henson Creek, except for specific listings in Segment 1.. COGUUG32.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 33. All lakes and reservoirs that are tributary to the Gunnison River and within the La Garita, Powderhorn, West Elk, Collegiate Peaks, Maroon Bells, Raggeds, Fossil Ridge, or Uncompahgre Wilderness Areas.. COGUUG33.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.02.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 34. All lakes and reservoirs tributary to the Taylor River and the East River, from their sources to their confluence at the inception of the Gunnison River, excluding the listings in Segments 33, 35 and 37. This segment includes Meridian Lake, Nicholson Lake, Peanut Lake, Glazer Reservoir (38.874441, -106.999868), Lake Grant, Lily Pond, Pothole Reservoirs 1 and 2, Texas Lake, Mirror Lake, and Spring Creek Reservoir.. COGUUG34.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Glazer Reservoir.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 35. All lakes and reservoirs tributary to Redwell Creek.. COGUUG35.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; ---.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; 8.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 36. All lakes and reservoirs tributary to the Gunnison River from its inception at the confluence of the Taylor and East Rivers, to the inlet of Blue Mesa Reservoir, excluding the listings in Segment 33. This segment includes Kenny Moore Reservoir, Hot Springs Reservoir, Needle Creek Reservoir, Vouga Reservoir, Moss Lake, Dome Lakes, and McDonough Reservoirs 1 and 2.. COGUUG36.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 37. All lakes and reservoirs tributary to Blue Mesa Reservoir, Morrow Point Reservoir, Crystal Reservoir or the segments of the Gunnison River that interconnect them, excluding the listings in Segments 33 and38. This segment includes Fish Creek Reservoirs 1 and 2, Hampton Lake, High Park Lake, Watson Lake, Butte Lake, Swanson Lake, Fitzpatrick Lake, Evergreen Lake (38.325447, -107.365786), Dry Lake, Devils Lake, Powderhorn Lakes, Soderquist Reservoir, Rainbow Lake, Cataract Lake, Castle Lakes, Crystal Lake, and Waterdog Lake.. COGUUG37.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Evergreen Lake.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 38. Lake San Cristobal, Taylor Park Reservoir, Blue Mesa Reservoir, Morrow Point Reservoir, Crystal Reservoir, and Silver Jack Reservoir.. COGUUG38.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Nitrogen(chronic) = applies only above the facilities listed at 35.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = DM and MWAT=CLL from 1/1-3/31 Lake San Cristobal, Taylor Park Reservoir, Blue Mesa Reservoir DM=24.2 and MWAT=16.6 from 4/1-12/31 All others DM and MWAT=CLL from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. All tributaries to North Fork of the Gunnison River, including all wetlands, within the West Elk or Raggeds Wilderness Areas.. COGUNF01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 2. Mainstem of North Fork of the Gunnison River from its inception at the confluence of Muddy Creek and Anthracite Creek to the Black Bridge (41.75 Drive) above Paonia.. COGUNF02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 3. Mainstem of North Fork of the Gunnison River from the Black Bridge (41.75 Drive) above Paonia to the confluence with the Gunnison River.. COGUNF03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.; 4/1 - 9/30.. Recreation P.; 10/1 - 3/31.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = DM and MWAT=CS-II from 11/16-3/15 DM=26.5 and MWAT=21.9 from 3/16-11/15 See temperature assessment location at 35.6(6)..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*.; C.. acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; 4/1 - 9/30.; ---.; 126.. E. coli (per 100 mL).; 10/1 - 3/31.; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4a. All tributaries and wetlands to Muddy Creek within national forest boundaries. Anthracite Creek, including all tributaries and wetlands, from the source to the confluence with Muddy Creek. All tributaries and wetlands to the North Fork of the Gunnison from its inception at the confluence of Muddy Creek and Anthracite Creek to the confluence with the Gunnison River within national forest boundaries. This segment excludes the listings in Segments 1 and 4c.. COGUNF04A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 4b. Muddy Creek, including all tributaries and wetlands, from the national forest boundary to the confluence with Anthracite Creek, except for the specific listings in Segment 1.. COGUNF04B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 4c. All tributaries and wetlands to Lake Irwin from their sources to the inlet of Lake Irwin.. COGUNF04C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 5a. Mainstems of Hubbard Creek, Terror Creek, and Minnesota Creek, from the national forest boundary to their confluences with the North Fork of the Gunnison River; mainstem of Jay Creek from its source to its confluence with the North Fork of the Gunnison River.. COGUNF05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 5b. Mainstem of Roatcap Creek, including all tributaries and wetlands, from the source to the confluence with the North Fork of the Gunnison. Leroux Creek from the national forest boundary to its confluence with the North Fork of the Gunnison River.. COGUNF05B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6a. All tributaries, including wetlands, to the North Fork of the Gunnison River from its inception at the confluence of Muddy Creek and Anthracite Creek to the confluence with the Gunnison River, and not within national forest boundaries, except for the specific listings in Segments 5a, 5b, 6b, and 6c.. COGUNF06A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6b. Mainstem of Bear Creek and Stevens Gulch, including all tributaries and wetlands. All tributaries and wetlands, to the North Fork of the Gunnison River that are north of the North Fork of the Gunnison River, from a point immediately above the confluence with Roatcap Creek to the confluence with the Gunnison River, and are not within national forest boundaries. All tributaries and wetlands to the North Fork of the Gunnison River that are south of the North Fork of the Gunnison River, from a point immediately above the confluence with Minnesota Creek to the confluence with the Gunnison River, and are not within national forest boundaries. This segment excludes the listings in Segments 5a and 5b.. COGUNF06B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6c. Thompson Creek from the Gunnison National Forest boundary to its confluence with the North Fork of the Gunnison River.. COGUNF06C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7. Paonia Reservoir and Overland Reservoir.. COGUNF07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CLL.; CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. All lakes and reservoirs that are tributary to the North Fork of the Gunnison River and within the West Elk or Raggeds Wilderness areas.. COGUNF08.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9. All lakes and reservoirs tributary to Muddy Creek, Paonia Reservoir, or Anthracite Creek. All lakes and reservoirs tributary to the North Fork of the Gunnison River from its inception at the confluence with Muddy Creek and Anthracite Creek to the confluence with the Gunnison River, and within national forest boundaries, excluding the specific listing in Segments 7 and 8. This segment includes Island Lake, Aspen Leaf Reservoir, Floating Lake, Tomahawk Reservoir, Dollar Lake, Lost Lake, Lost Lake Slough, Lake Irwin, Terror Creek Reservoir, Minnesota Reservoir, Beaver Reservoir, Lone Cabin Reservoir, Todd Reservoir, Holy Terror Reservoir (aka Eagle River Reservoir), Goodenough Reservoir, Dogfish Reservoir, Hilltop Reservoir, Willow Reservoir, Doughty Reservoir, Reynolds Reservoir, Hanson Reservoir, Bailey Reservoir, Owens Reservoir, Gray Reservoir, and Patterson Reservoirs.. COGUNF09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Nitrogen(chronic) = applies only above the facilities listed at 35.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10. All lakes and reservoirs tributary to Roatcap Creek and Jay Creek from their sources to their confluences with the North Fork of the Gunnison River. All lakes and reservoirs tributary to Hubbard Creek, Terror Creek, Minnesota Creek, or Leroux Creek, and are not within national forest boundaries.. COGUNF10.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 11. All lakes and reservoirs tributary to the North Fork of the Gunnison River from its inception at the confluence of Muddy Creek and Anthracite Creek to the confluence with the Gunnison River, and not within national forest boundaries, except for the specific listings in Segments 7, 9, and 10. This segment includes Roeber Reservoir.. COGUNF11.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. All tributaries to the Uncompahgre River, including all wetlands, which are within the Mt. Sneffels or Uncompahgre Wilderness Areas.. COGUUN01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2. Mainstem of the Uncompahgre River from the source (Poughkeepsie Gulch) to a point immediately above the confluence with Red Mountain Creek.. COGUUN02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3a. Mainstem of the Uncompahgre River from a point immediately above the confluence with Red Mountain Creek to a point immediately above the confluence with Cascade Creek.. COGUUN03A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 7438.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3b. Mainstem of the Uncompahgre River from a point immediately above the confluence with Cascade Creek to a point immediately above the confluence with Dexter Creek.. COGUUN03B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = summer criteria apply from 6/1-10/15..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I*.; CS-I*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 2971.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3c. Mainstem of the Uncompahgre River from a point immediately above the confluence with Dexter Creek to a point immediately below the confluence with Dallas Creek.. COGUUN03C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1793.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3d. Mainstem of the Uncompahgre River from a point immediately below the confluence with Dallas Creek to the inlet of Ridgway Reservoir.. COGUUN03D.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 2053.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3e. Mainstem of the Uncompahgre River from the outlet of Ridgway Reservoir to a point immediately above the outlet of the South Canal near Uncompahgre.. COGUUN03E.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = summer criteria apply from 4/1-11/15..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II*.; CS-II*.; C.. acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3f. Mainstem of the Uncompahgre River from a point immediately above the outlet of the South Canal to a point immediately above the Highway 90 bridge in Montrose.. COGUUN03F.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4a. Mainstem of the Uncompahgre River from the Highway 90 bridge at Montrose to Gunnison Road.. COGUUN04A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4b. Mainstem of the Uncompahgre River from Gunnison Road to the upstream boundary of Confluence Park.. COGUUN04B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4c. Mainstem of the Uncompahgre River from the upstream boundary of Confluence Park to the confluence with the Gunnison River.. COGUUN04C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1108.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5. All tributaries to the Uncompahgre River, including all wetlands, from the source to a point immediately below the confluence with Dexter Creek, except for specific listings in Segments 1, 6a, 6b, and 7 through 9.. COGUUN05.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6a. Mainstem of Red Mountain Creek from the source to immediately above the confluence with the East Fork of Red Mountain Creek.. COGUUN06A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6b. Mainstem of Red Mountain Creek from immediately above the confluence with the East Fork of Red Mountain Creek to the confluence with the Uncompahgre River. All tributaries and wetlands to Red Mountain Creek within Corkscrew and Champion basins.. COGUUN06B.. Designation.. UP..; Classifications.. Agriculture.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT... acute.; chronic.. D.O. (mg/L).; ---.; 3.0.. pH.; ambient.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; ---.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; ---.; ---.. Nitrate.; ---.; ---.. Nitrite.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic.; ---.; ---.. Cadmium.; ---.; ---.. Chromium III.; ---.; ---.. Chromium VI.; ---.; ---.. Copper.; ---.; ---.. Iron.; ---.; ---.. Lead.; ---.; ---.. Manganese.; ---.; ---.. Mercury(T).; ---.; ---.. Molybdenum(T).; ---.; ---.. Nickel.; ---.; ---.. Selenium.; ---.; ---.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc.; ---.; ---.... |
| --- | --- |

|  | 7. Mainstem of Gray Copper Gulch from the source to the confluence with Red Mountain Creek.. COGUUN07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 2338.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/655.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 8. Mainstem of Mineral Creek from the source to the confluence with the Uncompahgre River.. COGUUN08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; ---.; 5.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; ---.; 4.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 9. Mainstem of Imogene Creek from its source to its confluence with Sneffels Creek. Mainstem of Sneffels Creek, including all tributaries and wetlands, from a point 1.5 miles above its confluence with Imogene Creek at 37.974979, -107.753960 (WGS84) to its confluence with Imogene Creek. Mainstem of Canyon Creek from its inception at the confluence of Imogene Creek and Sneffels Creek to the confluence with the Uncompahgre River.. COGUUN09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation P... Qualifiers:.. Fish Ingestion... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 10a. All tributaries to the Uncompahgre River, including all wetlands, from a point immediately below the confluence with Dexter Creek to the South Canal near Uncompahgre, except for specific listings in Segments 1, 10b, and 11.. COGUUN10A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 10b. Mainstem of Kettle Gulch from the road crossing at 38.101201, -107.75949 to the County Road 23 crossing.. COGUUN10B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 11. Mainstem of Coal Creek from the source to the Park Ditch. Mainstem of Dallas Creek from the source of the East and West Forks to the confluence with the Uncompahgre River. Mainstem of Cow Creek from the Uncompahgre Wilderness Area boundary to a point immediately below the confluence with Nate Creek. All tributaries and wetlands to Cow Creek from the Uncompahgre Wilderness Area boundary to the confluence with the Uncompahgre River. Mainstems of Billy Creek, Onion Creek and Beaton Creek from the source to the confluence with the Uncompahgre River. Mainstem of Beaver Creek from the source to the confluence with the East Fork of Dallas Creek. Mainstem of Pleasant Valley Creek from the source to the confluence with Dallas Creek.. COGUUN11.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 12. All tributaries to the Uncompahgre River, including all wetlands, from the South Canal near Uncompahgre to the confluence with the Gunnison River, except for specific listings in Segments 13, 14, 15a and 15b.. COGUUN12.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1400.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13a. Mainstem of East Fork Dry Creek and Pryor Creek from their sources to the national forest boundary; West Fork Dry Creek from its source to its confluence with East Fork Dry Creek; mainstem of West Fork Spring Creek and Middle Spring Creek from their sources to their confluence, and mainstem of Mexican Gulch from the source to the Section line dividing Section 19 and 30, T49N, R9W.. COGUUN13A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13b. Mainstem of East Fork Dry Creek from the national forest boundary to its confluence with West Fork Dry Creek. Pryor Creek from the national forest boundary to its confluence with East Fork Dry Creek. Mainstem of Spring Creek from the source to a point immediately below the confluence with Devinny Canyon.. COGUUN13B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13c. Mainstem of Spring Creek from a point immediately below the confluence with Devinny Canyon to Popular Road at the mouth of Spring Canyon.. COGUUN13C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 14. East and West Forks of Horsefly Creek, including all tributaries and wetlands, from their sources to a point immediately above their confluence. Happy Canyon Creek, including all tributaries and wetlands, from the source to the most downstream national forest boundary.. COGUUN14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation P... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 15a. Mainstem of Happy Canyon from a point immediately below the West Canal to the confluence with the Uncompahgre River; mainstem of Horsefly Creek from a point immediately below the confluence with Wildcat Canyon to the confluence with the Uncompahgre River.. COGUUN15A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation P... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15b. Mainstem of Dry Creek from the confluence of the East and West Forks to immediately above the confluence with Coalbank Canyon Creek.. COGUUN15B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16. All lakes and reservoirs tributary to the Uncompahgre River and within the Mt. Sneffels or Uncompahgre Wilderness Areas.. COGUUN16.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/50.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 17. All lakes and reservoirs tributary to the Uncompahgre River from the source to a point immediately below the confluence with Dexter Creek, except for listings in Segment 16. This segment includes Lake Como, Ptarmigan Lake, Crystal Lake, and Lake Lenore.. COGUUN17.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 18. All lakes and reservoirs tributary to the Uncompahgre River from a point immediately below the confluence with Dexter Creek to a point immediately below the South Canal near Uncompahgre, excluding the listings in Segment 16 and 19. All lakes and reservoirs tributary to the East Fork of Dry Creek or the West Fork of Dry Creek from their sources to their confluence. This segment includes Black Lake, Blue Lakes, Ulah Brown Spring, Lake Otonawanda, West Lake, Dry Lake, Elephant Reservoir, Buckhorn Lakes, Silesca Pond and Olathe Reservoirs 1 and 2.. COGUUN18.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Lake Otonawanda.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 19. Ridgway Reservoir.. COGUUN19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CLL.; CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 20. Sweitzer Lake (a.k.a. Garnet Mesa Reservoir).. COGUUN20.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 21. All lakes and reservoirs tributary to the Uncompahgre River from a point immediately below the South Canal near Uncompahgre to the confluence with the Gunnison River, excluding the listings in Segments 18, 20, and 22.. COGUUN21.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P... Qualifiers:.. Fish Ingestion... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 22. Fairview Reservoir.. COGUUN22.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P.. Water Supply.. DUWS... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. Mainstem of the Gunnison River from the outlet of Crystal Reservoir to Highway 65 (38.772574, -108.002634).. COGULG01.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 2. Mainstem of the Gunnison River from Highway 65 (38.772574, -108.002634) to the confluence with the Colorado River.. COGULG02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; 480.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3a. All tributaries to the Gunnison River, including all wetlands, which are within national forest boundaries, from the outlet of Crystal Reservoir to the confluence with the Colorado River, except for specific listings in the North Fork Gunnison River sub-basin, Uncompahgre River sub-basins, and Segments 3b, 10, 11a, 11b, and 12.. COGULG03A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3b. Mainstem of Big Dominguez Creek, Little Dominguez Creek, Escalante Creek, Potter Creek, and Roubideau Creek, including all tributaries and wetlands, within the boundaries of the Uncompahgre National Forest.. COGULG03B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4a. All tributaries to the Gunnison River, including all wetlands which are not within national forest boundaries, from the outlet of Crystal Reservoir to the confluence with the Colorado River, except for specific listings in the North Fork of the Gunnison River sub-basin, the Uncompahgre River sub-basin, and in Segments 3a, 3b, 4b, 4c, 5a, 5b, 5c, 6a, 6b, 6c, 7, 8a, 8b, 10 and 12.. COGULG04A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4b. All tributaries and wetlands to Reeder, Hollenbeck, and Juniata Reservoirs, and the mainstem of Kannah Creek below the point of diversion for public water supply (38.961321, -108.229830).. COGULG04B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4c. Mainstem of Red Rock Creek from the boundary of Black Canyon of the Gunnison National Park to the confluence of the Gunnison River.. COGULG04C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5a. Mainstem of North Fork Escalante Creek from the national forest boundary to the confluence with Escalante Creek.. COGULG05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5b. Mainstem of Roubideau Creek from the national forest boundary to a point immediately above the confluence with Potter Creek. Mainstem of Monitor Creek from the national forest boundary to the confluence with Potter Creek. Mainstem of Potter Creek from immediately below Monitor Creek to the confluence with Roubideau Creek.. COGULG05B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 5c. All tributaries and wetlands to Roubideau Creek from the national forest boundary to a point immediately below the confluence with Potter Creek, excluding the portion of the mainstems of Potter Creek and Monitor Creek in Segment 5b. All tributaries and wetlands to Escalante Creek from the national forest boundary to the Delta/Montrose County line (38.668215, -108.328144), excluding listings in Segment 5a. All tributaries and wetlands to Little Dominguez Creek from the national forest boundary to the confluence with Big Dominguez Creek. All tributaries and wetlands to Big Dominguez Creek from the national forest boundary to the confluence with the Gunnison River.. COGULG05C.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6a. Mainstem of Escalante Creek from the national forest boundary to the Delta/Montrose County line (38.668215, -108.328144); mainstem of Little Dominguez from the national forest boundary to Big Dominguez Creek; mainstem of Big Dominguez from the national forest boundary to the Gunnison River.. COGULG06A.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 6b. Mainstem of Roubideau Creek from Potter Creek to the Gunnison River. Mainstem of East Creek from the source to the Gunnison River.. COGULG06B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6c. Mainstem of Escalante Creek from the Delta/Montrose County line (38.668215, -108.328144) to the Gunnison River.. COGULG06C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 7a. Mainstem of Ward Creek, from the national forest boundary to the confluence with Dirty George Creek.. COGULG07A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 7b. Mainstem of Surface Creek from the point of diversion of water supply (38.965216, -107.876031) to the confluence with Tongue Creek; mainstem of Tongue Creek from its inception at the confluence of Ward Creek and Dirty George Creek to the confluence with the Gunnison River; mainstem of Youngs Creek from the national forest boundary to the confluence with Kiser Creek; mainstem of Kiser Creek from the national forest boundary to the confluence with Ward Creek.. COGULG07B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 8a. Mainstem of Surface Creek, including all tributaries and wetlands, from the national forest boundary to the point of diversion for public water supply (38.965216, -107.876031).. COGULG08A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 8b. Mainstem of Kannah Creek, including all tributaries and wetlands, from the national forest boundary to the point of diversion for public water supply (38.961321, -108.229830).. COGULG08B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 9. Fruitgrowers Reservoir.. COGULG09.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.; 4/1 - 10/31.. Recreation P.; 11/1 - 3/31... Qualifiers:.. Fish Ingestion... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; 4/1 - 10/31.; ---.; 126.. E. coli (per 100 mL).; 11/1 - 3/31.; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10. Mainstem of the Smith Fork from the confluence of the North Smith Fork and South Smith Fork to the confluence with the Gunnison River.. COGULG10.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 11a. All tributaries to the Smith Fork, including all wetlands, which are within national forest boundaries except for specific listings in Segment 11b; Doug Creek from the source to the confluence with Muddy Creek.. COGULG11A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11b. All tributaries to the Smith Fork, including all wetlands, which are within the West Elk Wilderness Area.. COGULG11B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12. All tributaries to the Smith Fork, including all wetlands, which are not within national forest boundaries, except for the specific listing in Segment 11a.. COGULG12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13. Crawford Reservoir.. COGULG13.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14. All lakes and reservoirs tributary to the Gunnison River, from the outlet of Crystal Reservoir to the confluence with the Colorado River, and within national forest boundaries, excluding listings in the North Fork of the Gunnison River sub-basin, the Uncompahgre River sub-basin, and Segments 15, 17 and 18. This segment includes Trickle Reservoir, Hale Reservoir, Marcott Park Reservoir, Cherry Lane Reservoir, Cole Reservoirs, Cedar Mesa Reservoir, Kehmeier Reservoir, Weir and Johnson Reservoir, Bonita Reservoir, Blanche Park Reservoir, Vela Reservoir, Knox Reservoir, Military Park Reservoir, Eureka Park Reservoir, Carbonate Park Reservoirs, Prebble Reservoir, Youngs Creek Reservoirs, Kiser Reservoir, Donnely Reservoir, Kiser Slough Reservoir, Baron Lake, Upper Eggleston Lake, Upper Hotel Lake, Hotel Lake, Arch Slough, Alexander Lake, Deep Ward Lake, Kennicott Slough Reservoir, Womack Reservoirs, Deep Slough Reservoir, Scotland Peak Reservoir, Boulder Lake Reservoir, Basin Reservoir 1, Clear Lake, Granby Reservoirs, Dugger Reservoir, Carson Lake, Crane Lake, Flowing Park, Blue Lake, Chambers Reservoir, Scales Lakes, Grand Mesa Reservoirs, Anderson Reservoirs, Bolen Reservoir, Bolen-Anderson-Jacobs Reservoir 2, Hollenbeck Reservoir 2, Cliff Lake Reservoir, Lee Reservoirs, Lone Pine Reservoirs, Bullfrog Reservoir, Twin Lake, Harry White Reservoirs, Beaver Dam Reservoir, and Fruita Reservoirs 1 and 2.. COGULG14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5-9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15. Island Lake, Eggleston Lake, and Trickle Park Reservoir (aka Park Reservoir).. COGULG15.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CLL.; CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5-9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16. All lakes and reservoirs that are tributary to the Gunnison River, from the outlet of Crystal Reservoir to the confluence with the Colorado River, and not within national forest boundaries, excluding the listings in the North Fork of the Gunnison sub-basin, the Uncompahgre River sub-basin, and Segments 9, 13, and 19. This segment includes Poison Springs Reservoir, Dry Fork Reservoir, Delta Reservoir, Winkler Reservoir, Desert Reservoir, Alkali Reservoir, Cheney Reservoir, Juniata Reservoir, Hallenbeck Reservoir, Reeder Reservoir, Enochs Lake, Gobbo Reservoir, Schrader Reservoir, and King Reservoir.. COGULG16.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Hallenbeck Reservoir and Juniata Reservoir.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 17. All lakes and reservoirs tributary to the Smith Fork, and within national forest boundaries excluding the listings in Segment 18. All lakes and reservoirs tributary to Doug Creek.. COGULG17.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 18. All lakes and reservoirs tributary to the Smith Fork, and are within the West Elk Wilderness Area.. COGULG18.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 19. All lakes and reservoirs tributary to the Smith Fork, which are not within national forest boundaries, excluding the listings in Segment 17. This segment includes Gould Reservoir.. COGULG19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. All tributaries, including wetlands, to the San Miguel River that are within the boundaries of the Lizard Head or Mount Sneffels Wilderness Areas.. COGUSM01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 2. All tributaries and wetlands to the San Miguel River from its source to a point immediately below the confluence of Leopard Creek, except for listings in Segments 1, 6a, 6b, 7a, 7b, and 8.. COGUSM02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 3a. Mainstem of the San Miguel River from its inception at the confluence of Bridal Veil and Ingram Creeks to a point immediately above the confluence of Marshall Creek.. COGUSM03A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; ---.. Uranium.; varies*.; varies*.. Zinc.; ---.; 190.... 3b. Mainstem of the San Miguel River from a point immediately above the confluence of Marshall Creek to a point immediately above the confluence of the South Fork San Miguel River.. COGUSM03B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = DM=13.9 and MWAT=9 from 10/1-10/31 DM=13 and MWAT=9 from 11/1-3/31 DM=14 and MWAT=9 from 4/1-5/31 DM=21.7 and MWAT=17 from 6/1-9/30..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; ---.; TVS.. Copper.; ---.; ---.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; 190.... |
| --- | --- |

|  | 4a. Mainstem of the San Miguel River from a point immediately above the confluence of the South Fork of the San Miguel River to a point immediately below the CC ditch.. COGUSM04A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4b. Mainstem of the San Miguel River from a point immediately below the CC ditch to a point immediately below the confluence of Naturita Creek.. COGUSM04B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = DM=13 and MWAT=9 from 11/1-2/29 DM=30.9 and MWAT=23.3 from 3/1-10/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5a. Mainstem of the San Miguel River from a point immediately below the confluence of Naturita Creek to a point immediately below the confluence of Coal Canyon.. COGUSM05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 5b. Mainstem of the San Miguel River from a point immediately below the confluence of Coal Canyon to its confluence with the Dolores River.. COGUSM05B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6a. Mainstem of Ingram Creek, including all tributaries and wetlands, from the source to the confluence with the San Miguel River.. COGUSM06A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; ---.; 190.... 6b. Mainstem of Marshall Creek, including all tributaries and wetlands, from the source to the confluence with the San Miguel River.. COGUSM06B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; ---.; 190.... |
| --- | --- |

|  | 7a. Mainstem of Howard Fork, including all tributaries and wetlands, from a point immediately below the confluence of Swamp Gulch to the confluence with the South Fork of the San Miguel River, except for listings in Segment 7b.. COGUSM07A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7b. Mainstem of Waterfall Creek, including all tributaries and wetlands, from the source to the confluence with Howard Fork.. COGUSM07B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. Mainstem of the South Fork of the San Miguel River from its inception at the confluence of the Howard and Lake Forks to its confluence with the San Miguel River.. COGUSM08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/80.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9a. All tributaries to the San Miguel River, including all wetlands, from a point immediately below the confluence of Leopard Creek to the Dolores River that are within the boundaries of the Uncompahgre National Forest, except for listings in Segments 9b and 10a.. COGUSM09A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 9b. All tributaries and wetlands to Tabeguache Creek that are within the boundaries of the Uncompahgre National Forest.. COGUSM09B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 10a. Mainstem of Tabeguache Creek from its source to the Uncompahgre National Forest boundary.. COGUSM10A.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/75.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10b. Mainstem of Naturita Creek from the point it exits the Uncompahgre National Forest at the most downstream boundary to the confluence with the San Miguel River.. COGUSM10B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/75.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 10c. Mainstem of Tabeguache Creek from the point it exits the Uncompahgre National Forest to the confluence with the San Miguel River.. COGUSM10C.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/75.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11a. All tributaries to Miramonte Reservoir and West Naturita Creek from their sources to the Uncompahgre National Forest Boundary below Miramonte Reservoir. The mainstems of Beaver and Horsefly Creeks from the Uncompahgre National Forest boundary to their confluences with the San Miguel River.. COGUSM11A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 11b. Mainstem of Saltado Creek from the Uncompahgre National Forest boundary to the confluence with the San Miguel River.. COGUSM11B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 12a. All tributaries and wetlands to Naturita Creek. All tributaries and wetlands to the San Miguel River from a point immediately below the confluence with Leopard Creek to a point immediately above Horsefly Creek. This segment excludes the listings in Segments 9a, 11a, 11b, 12b, and 12c.. COGUSM12A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 12b. All tributaries and wetlands to the San Miguel River from a point immediately above Horsefly Creek to the confluence with the Dolores River, excluding the listings in Segments 9a, 9b, 10a, 10b, 10c, 11a, 12a, 12c, and 12d. Maverick Draw, including all tributaries and wetlands, from its source to the confluence with Naturita Creek.. COGUSM12B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 12c. Mainstem of Calamity Draw from Lincoln Street in Nucla (38.264075, -108.555087) to the confluence with the San Miguel River.. COGUSM12C.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:.. Fish Ingestion... Other:... Discharger Specific Variance(s):.. Ammonia(ac/ch) = See Section 35.6(4) for details on the variance for the Town of Nucla.. Expiration Date of 12/31/2031.. *Phosphorus(chronic) = applies only above the facilities listed at 35.5(4).. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 12d. All tributaries and wetlands to Tabeguache Creek from the point it exits the Uncompahgre National Forest to the confluence with the San Miguel River.. COGUSM12D.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards... Other:... *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13. All lakes and reservoirs tributary to the San Miguel River that are within the boundaries of the Lizard Head or Mount Sneffels Wilderness Areas.. COGUSM13.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14. All lakes and reservoirs tributary to the San Miguel River from its source to a point immediately below the confluence of Leopard Creek, except for the specific listings in Segments 13, 15, 16, 17 and 20. This segment includes Lake Hope, Cushman Lake, Alta Lakes, Blue Lake, Mud Lake, and Woods Lake.. COGUSM14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15. All lakes and reservoirs tributary to Ingram Creek from the source to the confluence with the San Miguel River. This segment includes Ingram Lake.. COGUSM15.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16. All lakes and reservoirs tributary to Marshall Creek from the source to the confluence with the San Miguel River. This segment includes Thorne Lake.. COGUSM16.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; ---.; 190.... |
| --- | --- |

|  | 17. All lakes and reservoirs tributary to the Howard Fork from a point immediately below the confluence of Swamp Gulch to the confluence with the South Fork of the San Miguel River.. COGUSM17.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 18. All lakes and reservoirs tributary to the San Miguel River from a point immediately below the confluence of Leopard Creek to the confluence with the Dolores River, and that are within Uncompahgre National Forest boundaries. This segment includes Hoffman Reservoir, Paxton Reservoir, and Hotchkiss Reservoir.. COGUSM18.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 19. All lakes and reservoirs tributary to the San Miguel River from a point immediately below the confluence of Leopard Creek to the Dolores River, and not within Uncompahgre National Forest boundaries, excluding the listings in Segment 20. This segment includes Point Reservoir, Palmers Lake, Williams Reservoir, Town Reservoir, and Lilylands Reservoir.. COGUSM19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Town Reservoir.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; TVS.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 20. Trout Lake, Gurley Reservoir, Cone Reservoir, and Miramonte Reservoir.. COGUSM20.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Gurley Reservoir.. *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CLL.; CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1a. Mainstem of the Dolores River from the bridge at Bradfield Ranch (Forest Route 505, near Montezuma/Dolores County Line) to a point immediately above the confluence with Big Canyon Creek near Dove Creek.. COGULD01A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = DM and MWAT=CS-II from 11/1-3/22 DM=26.6 and MWAT=23.8 from 3/23-10/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 1b. Mainstem of the Dolores River from a point immediately above the confluence with Big Canyon Creek near Dove Creek to a point immediately above the Highway 141 road crossing near Slick Rock.. COGULD01B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(chronic) = See 35.5(3) for details.. *Temperature = DM=CS-II and MWAT=9.1 from 11/1-3/22 DM= 27.6 and MWAT=24.7 from 3/23-10/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 2. Mainstem of the Dolores River from the Highway 141 road crossing near Slick Rock to the Colorado/Utah border.. COGULD02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 3a. All tributaries to the Dolores River, including all wetlands, from the bridge at Bradfield Ranch (Forest Route 505, near Montezuma/Dolores County Line) to the Colorado/Utah border, except for specific listings in Segments 3b, 3c, 4, 5, and 6.. COGULD03A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3b. All tributaries to the Dolores River, including wetlands, that are within national forest boundaries, from the bridge at Bradfield Ranch (Forest Route 505, near the Montezuma/Dolores County Line) to the Colorado/Utah border, excluding the small area of Uncompahgre National Forest within the Disappointment Valley and the listings in Segments 3c and 5. Disappointment Creek, including all tributaries and wetlands, from the source to a point immediately below the confluence with Morrison Creek.. COGULD03B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; TVS.; TVS.. Zinc.; TVS.; TVS/TVS(sc).... 3c. Mainstem of Salt Creek, including all tributaries and wetlands, from the source within the Sinbad Valley to the confluence with the Dolores River.. COGULD03C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; 6.6.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4. Mainstem of West Paradox Creek from the Manti-La Sal National Forest boundary to the confluence with the Dolores River. Mainstem of Blue Creek, including all tributaries and wetlands, from the Uncompahgre National Forest boundary to the confluence with the Dolores River.. COGULD04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... 5. Mainstem of West Creek from the source to the confluence with the Dolores River. Roc Creek including all tributaries and wetlands from the Manti-La Sal National Forest boundary to the confluence with the Dolores River. La Sal Creek, including all tributaries and wetlands, from the Utah/Colorado border to the confluence with the Dolores River. Mesa Creek, including all tributaries and wetlands, from the Uncompahgre National Forest boundary to the confluence with the Dolores River.. COGULD05.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; TVS.; varies*.. Uranium(T).; ---.; 16.8-30.; A. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6. North Fork of West Creek, including all tributaries and wetlands, from the source to the confluence with West Creek. Granite Creek, including all tributaries and wetlands, from the source the Colorado/Utah border.. COGULD06.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Beryllium(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7. All lakes and reservoirs tributary to the Dolores River, from the bridge at Bradfield Ranch (Forest Route 505, near Montezuma/Dolores County Line) to the Colorado/Utah border, and within national forest boundaries. This segment includes Long Park Reservoir, Cabin Reservoir, Beef Trail Reservoir, Dry Lake, Glade Lake, Glade Point Reservoir, Arrowhead Lake, Buckeye Reservoir, Black Pine Reservoir, Casto Reservoir, and Big Creek Reservoir.. COGULD07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. All lakes and reservoirs tributary to the Dolores River, from the bridge at Bradfield Ranch (Forest Route 505, near Montezuma/Dolores County Line) to the Colorado/Utah border, and not within national forest boundaries.. COGULD08.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 35.5(3) for details.. *Uranium(chronic) = See 35.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

STREAM CLASSIFICATIONS and WATER QUALITY STANDARDS – FOOTNOTES

(A)	Whenever a range of standards is listed and referenced to this footnote, the first number in the range is a strictly health-based value, based on the Commission’s established methodology for human health-based standards. The second number in the range is a maximum contaminant level, established under the federal Safe Drinking Water Act that has been determined to be an acceptable level of this chemical in public water supplies, taking treatability and laboratory detection limits into account. Control requirements, such as discharge permit effluent limitations, shall be established using the first number in the range as the ambient water quality target, provided that no effluent limitation shall require an “end-of-pipe” discharge level more restrictive than the second number in the range. Water bodies will be considered in attainment of this standard, and not included on the Section 303(d) List, so long as the existing ambient quality does not exceed the second number in the range.

(B)	Reserved.

(C)	For certain site-specific temperature standards, the temperature excursions listed in Table I - Footnote 5(c) of 31.16 do not apply.  Assessment of ambient-based temperature standards should be conducted in a way that represents similar conditions to those under which the criteria were developed (i.e., air, low flow, and warming event excursions should not apply).  Similarly, where site-specific adjustments to the winter shoulder season have been adopted, the winter shoulder season excursion does not apply.