# DEPARTMENT OF REGULATORY AGENCIES

State Plumbing Board

## PLUMBING RULES AND REGULATIONS

3 CCR 720-1

[Editor’s Notes follow the text of the rules at the end of this CCR Document.]

_________________________________________________________________________

…

## 1.2	STANDARDS

A.	Colorado Plumbing Code

The Board hereby adopts and incorporates by reference, with certain additions, revisions, and deletions, the following chapters, sections, and appendices of the International Plumbing Code (“IPC”) and International Residential Code (“IRC”).

These Rules, together with the following incorporated provisions, shall be known as the Colorado Plumbing Code. The Colorado Plumbing Code sets forth the minimum standards governing the inspection, installation, alteration, and repair of the plumbing fixtures, appliances, and systems throughout Colorado. The Colorado Plumbing Code does not include any later amendments to or editions of the IPC or IRC provisions listed below.

1.	Chapter 1, Sections 101.2, 102, 105, 106.1, 106.3, 106.4, 106.5, 106.5.5, 106.6.1, 107, 108.1, 110.3 Chapters 2-13, 15; and Appendices B, C, D, and E of the IPC, 2024 edition promulgated by the International Code Council 1st printing (June 2023).

2.	Chapter 1, Section R101, R102; Chapter 2; Chapter 3, Section R313.1 exceptions 1 and 2, Section R326; Chapters 25-32, of the IRC, 2024 edition, promulgated by the International Code Council, 1st printing (May 2024).

B.	Colorado Fuel Gas Code

The Board hereby adopts and incorporates by reference, with certain additions, revisions, and deletions, the following chapters, sections, and appendices of the International Fuel Gas Code (“IFGC”) and the IRC. These Rules, together with the following incorporated provisions, shall be known as the Colorado Fuel Gas Code. The Colorado Fuel Gas Code sets forth the minimum standards governing the inspection, installation, alteration, and repair of fuel gas piping and systems throughout Colorado. The Colorado Fuel Gas Code does not include any later amendments to or editions of the IFGC and IRC provisions listed below.

1.	Chapter 1 Section 101, 102, 105, 107, Chapters 2-8 save and except Sections 412 and 413, Appendices A-C of the IFGC, 2024 edition, promulgated by the International Code Council, 1st printing (October 2023).

2.	Chapter 1, Section R101, R102; Chapter 2; Chapter 24, Sections G2411-G2422, and G2401.1-Table G2428.3(4) and G2448 of the IRC, 2024 edition, promulgated by the International Code Council, 1st printing (May 2024).

C.	Public Copies

Copies of the provisions of the IPC, IFGC, and IRC, adopted and incorporated into the Colorado Plumbing Code and the Colorado Fuel Gas Code, are available for public inspection during regular business hours at the Board office at the Department of Regulatory Agencies Welcome Center, 1560 Broadway, Civic Center Plaza, Denver, Colorado, 80202, and at any state publications depository library. For further information regarding how this material can be obtained or examined, contact the Program Director for the Board (“Program Director”) at 1560 Broadway, Suite 1350, Denver, Colorado, 80202, (303) 894-2309. Copies of the IPC, IFGC, and IRC may be obtained from the International Code Council, 500 New Jersey Avenue, NW, 6th Floor, Washington, DC 20001. A list of ICC regional offices is available at http://www.iccsafe.org/AboutICC/Pages/ContactICC.aspx.

D.	General Interpretations

The following shall apply to the Colorado Plumbing Code and Colorado Fuel Gas Code.

1.	Code Official. Whenever the Colorado Plumbing Code and the Colorado Fuel Gas Code refer to “the code official,” it shall mean the Board or its designee, if any.

2.	Reasonable Time. As used in the Rules, the term “reasonable time” shall mean thirty calendar days.

3.	Amendments to the Colorado Codes. Any city, town, county or city and county which adopts more stringent standards than the Colorado Codes shall furnish a copy thereof to the Board.

4.	Abbreviations

…

5.	Alternate Materials and Methods Review

a.	Criteria. The Board shall consider requests for approval of alternate materials or methods under the procedures and limitations of Section 105 of the Colorado Plumbing Code and Colorado Fuel Gas Code.

b.	Procedures

(1)	Petitions for Approval. Any interested person may petition the Board to amend the Colorado Plumbing or Fuel Gas Code so as to approve the use of an alternate material or method, pursuant to Section 105 of the IPC. Such petition shall conform to the requirements in subsection (2)(b). Incomplete petitions will not be processed.

(2)	Petition Contents. Each petition filed under this Rule shall comply with the following requirements:

(a)	Where to Submit. Petitions shall be submitted in duplicate to the Program Director.

(b)	Petition Document. Petitions shall begin with a short concise document labeled as the “petition,” and which includes the following information:

…

(c)	Submission of National Standards. Each petition shall be accompanied by a set of the applicable standards from the Referenced Standards identified in the Colorado Plumbing Code, Colorado Fuel Gas Code or other standards listed by the petitioner in Rule 1.2(D)(5)(b)(2)(iii). If the listed standard incorporates other standards into its criteria, copies of the incorporated standards shall be submitted as well.

(d)	Results of Independent Third Party Compliance Testing. Each petition shall be supported with complete copies of test reports with protocols issued within the previous eighteen months prior to the date of submission by a laboratory or other testing facility that is recognized as a nationally recognized testing laboratory, pursuant to 29 C.F.R. section 1910.7, or by an ANSI accredited third-party certifier.

(e)	Compliance with Section 104. Each petition shall be supported by detailed written discussion as to why the proposed alternate material or method meets the following requirements from Section 104 of the IPC and IFGC:

…

E.	Revisions and Exceptions to the Incorporated Codes

1.	Revisions and Exceptions to the Colorado Plumbing Code

a.	IPC Section 102.8 Referenced codes and standards

…

b.	IPC Section 105.5 Permit issuance

…

c.	IPC Section 202 Definitions

...

d.	IPC Section 202 Definitions

…

e.	IPC Section 202 Definitions

…

f.	IPC Section 202 Definitions

…

g.	IPC Section 202 Definitions

…

h.	IPC Section 305.1 Protection against contact.

Delete section in it's entirety and replace with:

IPC Section 305.1 Protection against contact. Piping except for, cast iron, ductile iron, and galvanized steel shall not be placed in direct contact with steel framing members. Piping shall not be placed in direct contact with concrete or cinder walls and floors, other masonry, and corrosive soil. Where sheathing is used to prevent direct contact, the sheathing shall have a thickness of not less than .025 inch (.64 mm). Where sheathing protects piping that penetrates concrete or masonry walls or floors, the sheathing shall be installed in a manner that allows movement of the piping within the sheathing.

i.	IPC Section 306.2.4 Tracer wire

Delete in its entirety.

j.	IPC Section 308 Interval of support

…

k.	Add new Table 308.5.1 Hanger Rod Size

…

l.	IPC Section 308.7.1 Location

…

m.	IPC Section 312.1 Required tests

…

n.	IPC Section 312.3 Drainage and vent air test

Delete the words “Plastic piping shall not be tested using air.”

o.	IPC Section 312.5 Drainage and vent final test

…

p.	IPC Section 312.6 Water supply system test

…

q.	IPC Section 312.10 Shower liner test

…

r.	IPC Section 312.11.2 Testing

…

s.	IPC Section 403.1 Minimum number of fixtures

…

t.	IPC Section 403.3 Employee and public toilet facilities, effective January 1, 2024

…

u.	IPC Section 405.3.2 Public Lavatories

Add a new exception reading:

Exception: Lavatories located outside toilet facilities located within a classroom serving students from that classroom only. These toilet facilities and lavatories shall not count toward the total fixture count required by Table 403.1.

v.	IPC Section 405.3.6 Service sink location

Add new section:

Service sinks, including mop sinks, shall not be permitted to be installed in toilet facilities, bathing rooms, or areas intended for the preparation of food.

w.	IPC Section 410.4 Substitution.

…

x.	IPC Section 419.5

Add new sentence to the end of the section to read:

Lavatories in patient rooms of health care facilities shall be considered public for the purposes of this section.

y.	IPC Section 421.7 Shower head location

…

z.	IPC Section 421.8 Shower valve location

…

aa.	IPC Section 425.3 Water closet seats

…

bb.	IPC Section 504.6.1 Collection of Relief Valve Discharge

…

cc.	IPC Section 504.6.1.1 Pumped discharge of relief valve collection

…

dd.	IPC Section 504.7 Required pan.

Add new exception to read:

Replacements for water heaters that did not have a pan previously installed due to code in effect at the time of the original installation.

ee.	IPC Section 604.3 Water distribution design criteria

Add new exception to read:

Exception: Interior domestic water distribution pipe sizing using IAPMO’s Water Demand Calculator shall be approved as an alternative method for calculations in residential and multifamily installations. This does not include water distribution calculations for fire prevention systems.

ff.	IPC Section 605.15.2 Solvent cementing

…

gg.	IPC Section 608.3.1 Special equipment, water supply protection

…

hh.	IPC Section 608.9.1 Signage required

…

ii.	IPC Section 608.9.2 Distribution pipe labeling and marking

…

jj.	IPC Section 608-17 Connections to the Potable Water Systems

…

kk.	IPC Section 608 Protection of potable water system

…

ll.	IPC Section 701.2 Connection to sewer required

…

mm.	IPC Section 705.10.2 Solvent cementing

…

nn.	IPC Section 706.3. Installation of fittings

…

oo.	IPC Table 706.3 Fittings for change of direction

…

pp.	IPC Section 708.1.3 Building drain and building sewer junction.

…

qq.	IPC Section 802.1.8 Domestic dishwashing machines

Add a new section to read:

Domestic dishwashing machines may be connected to a separately trapped stand pipe provided with an air break (with drain hose secured to the underside of the counter top) or air gap as shown in the illustration below. Refer to 2024 IPC 409.4 for additional waste connections.

2” sink drain

rr.	IPC Section 802.3 Installation

Delete the last sentence

ss.	IPC Section 802.4 Waste receptors.

…

tt.	IPC Section 903.1.1 Roof extension unprotected

…

uu.	IPC Section 903.1.3 Protected vent terminal

…

vv.	IPC Section 903.2 Frost Closure

…

ww.	IPC Section 912.1 Horizontal wet vent permitted

…

xx.	IPC Section 915.1 Type of fixtures

Add a new sentence to read:

Floor sinks shall be considered “sinks” for the purposes of this section.

yy.	IPC Section 1002.1 Fixture traps

…

zz.	IPC Section 1003.1 Where required

…

aaa.	IPC.1003.3.2 Food waste disposers restriction.

…

bbb.	IPC Section 1101.3 Prohibited drainage

…

ccc.	IPC Section 1301.2.2 Filtration Required Exception

…

ddd.	IPC Section 1301 General

…

eee.	IPC Section 1301.3.2 Signage required graywater treatment works.

…

fff.	IPC Section 1301 General

…

ggg.	IPC Section 1301 General

…

hhh.	IPC Section 1301 General

…

iii.	IPC Section 1301 General

…

jjj.	IPC Section 1301.9.2 Materials Exception Add section

…

kkk.	IPC Section 1301.9.5 Overflow

…

lll.	IPC Section 1301.9.8 Draining of tanks

…

mmm.	IPC Section 1301.11 Trenching Requirements

…

nnn.	IPC Section 1301.12 Outdoor Outlet Access

…

ooo.	IPC Section 1302.1 General

…

ppp.	IPC Section 1302.5 Filtration

…

qqq.	IPC Section 1302.6.1 Graywater used for fixture flushing

…

rrr.	IPC Section 1302.7.3 Overflow

…

sss.	IPC Section 1302.7.4 Venting

…

ttt.	IPC Section 1302.7.5 Tank Drains

…

uuu.	IPC Section 1302.8.1 Bypass Valve

…

vvv.	IPC Section 1303 Nonpotable rainwater collection and distribution systems

…

www.	IPC Chapter 14 Subsurface graywater soil absorption systems

Delete in its entirety.

2.	Revisions and Exceptions to the International Residential Code Part VII incorporated as the Colorado Plumbing Code

a.	IRC Section 105.1 Required

…

b.	IRC Section R202 Definitions

…

c.	IRC Section R202 Definitions

…

d.	IRC Section R202 Definitions

…

e.	IRC Section R202 Definitions

…

f.	IRC Section R202 Definitions

Add new definition to read as follows:

Manufactured Housing and Tiny Home Hookup-Sewer. The point of connection from the completed building drain to the building sewer, the building underground system, or on-site plumbing systems not part of the manufactured home or tiny home.

g.	IRC Section R202 Definitions

Add new definition to read as follows:

Manufactured Housing and Tiny Home Hookup-Water. The point of connection from the completed manufactured home or tiny home water supply system to the site-installed potable water riser.

h.	IRC Section 202 Definitions

…

i.	IRC Section P2503.5.1 Rough Plumbing

…

j.	IRC Section P2503.6 Shower liner test

…

k.	IRC Section P2503.7 Water supply system testing

…

l.	IRC Section P2603.3 Protection against corrosion.

Delete section in its entirety and replace with:

IRC P2603.3 Protection against corrosion. Piping except for, for cast iron, ductile iron, and galvanized steel shall not be placed in direct contact with steel framing members. Piping shall not be placed in direct contact with concrete or cinder walls and floors, other masonry, and corrosive soil. Where sheathing is used to prevent direct contact, the sheathing shall have a thickness of not less than .025 inch (.64 mm). Where sheathing protects piping that penetrates concrete or masonry walls or floors, the sheathing shall be installed in a manner that allows movement of the piping within the sheathing.

m.	IRC Section P2605.2 Thermal expansion tanks.

Add new section P2605.2 to read:

P2605.2 Thermal expansion tanks. A thermal expansion tank shall be supported in accordance with the manufacturer’s instructions. Thermal expansion tanks shall not be supported by the piping that connects to such tanks.

n.	IRC Section P2705.1 General

…

o.	IRC P2706.1 General (waste receptors)

…

p.	IRC Section P2708.6 Shower head location

…

q.	IRC Section P2717.3 Dishwasher drain

Add new section to read:

IRC Section P2717.3 Dishwasher drain. Dishwashers may drain into a standpipe complying with Section P2706.2 as shown in the following illustration. The standpipe shall be provided with an air break (with drain hose secured to the underside of the counter top) or air gap as shown in the illustration below.

2” sink drain

r.	IRC Section P2801.5 Required pan.

Add new exception to read:

Exception: Replacements for water heaters that did not have a pan previously installed due to code in effect at the time of the original installation.

s.	IRC Section P2804.6.2 Collection of Relief Valve Discharge

…

t.	IRC Section P2803.6.2.1 Pumped discharge of relief valve collection

Add new Section P2804.6.2.1 to read:

P2803.6.2.1 Pumped discharge of relief valve collection. Pumps used to discharge the clear water collection of relief valves shall have an operating temperature equal to or exceeding that of the relief valve discharge temperature and shall have a gpm rating equal to or greater than the discharge of the relief valve.

u.	IRC Section P2901.1 Potable water required

…

v.	IRC Section P2901.1.2.1 Signage required

…

w.	IRC New Section P2901.1.2.3

Add new Section P2901.1.2.3 to read:

Graywater used for toilet and urinal flushing shall be dyed with blue or green food grade vegetable dye and be visibly distinct from potable water.

x.	IRC Section P2901.2.2 Distribution pipe labeling and marking

…

y.	IRC Section P2902.5 Protection of potable water connections

…

z.	IRC New Section P2902.5.6 Protection of potable water system

…

aa.	IRC Section P2903.10.1 Service valve.

…

bb.	IRC Section P2904 General

…

cc.	IRC Section P2910 Nonpotable Water Systems

…

dd.	IPC Section 2910.1 Scope

…

ee.	IRC Section P2910.2.2 Filtration Required Exception

…

ff.	IRC Section P2910 Graywater recycling systems

…

gg.	IRC Section P2910 Graywater recycling systems

…

hh.	IRC Section P2910 General

…

ii.	IRC Section 2910.9.3 Materials

…

jj.	IRC Section 2911.5 Filtration

Delete the section in its entirety and replace with:

P2911.5 Filtration. Graywater used for dispersed subsurface irrigation system requires a cartridge filter. The cartridge filter must be a minimum of sixty mesh located between the storage tank and the irrigation system. If a pump is being used to pressurize the graywater distribution system the filter must be located after the pump. Filters shall be accessible for inspection and maintenance. Filters shall utilize a pressure gauge or other approved method to provide indication when a filter requires servicing or replacement. Filters shall be installed with shutoff valves immediately upstream and downstream to allow for isolation during maintenance.

Exemption. Filtration is not required for mulch basin subsurface irrigation systems.

kk.	IRC Section P2911.7.6 Overflow

…

ll.	IRC Section P2911.7.4 Venting

…

mm.	IRC Section P2911.7.5 Draining of tanks

…

nn.	IRC Section P2911.8.1 Bypass valve

…

oo.	IRC Section 3001.1 Scope

Add exception 1:

Exception 1: Floor drains or trench drains may be allowed to be daylighted or discharged to an approved alternative disposal system when written approval has been provided from the jurisdiction having authority.

pp.	IRC Section P3003.9.2 Solvent cementing

…

qq.	IRC Table P3005.1 Fittings for changes in direction

Delete footnotes “a” and “b” from table P3005.1 and adding wording to section P3005.1.1 stating: Fixture crosses will not be required to meet the exception of this section.

rr.	IRC Section P3005.2.3 Building drain and building sewer junction.

Add new sentence at the end of the section to read:

When the cleanout is installed at the junction of the building drain and building sewer, It shall be an approved two way fitting with a single riser not to exceed 4 feet in depth or a two riser cleanout using back to back combination fittings of schedule 40 material. Cleanouts shall be installed and tested at underground or rough-in in accordance with Section P2503.5.

ss.	IRC Section P3008.1 Where required (backwater valves)

Delete section in its entirety and replace with:

Backwater valves shall be installed where waste receptors are located in a crawl space for the purpose of receiving condensate discharge from equipment located in that crawl space. Refer to new exception for 1.2(E)(1)(pp).

tt.	IRC Section P3009 Graywater soil absorption systems

…

uu.	IRC Section P3103.1.3 Roof extension covered

…

vv.	IRC Section P3103.2 Frost closure

…

ww.	IRC Section P3108.1 Horizontal wet vent permitted

…

3.	Revisions and exceptions to the International Fuel Gas Code incorporated as the Colorado Fuel Gas Code

a.	IFGC Section 101.1 Title

Delete in its entirety.

b.	IFGC Section 101.2.3 Piping Systems

Delete the words “and maintenance” from the end of the last sentences.

c.	IFGC Section 101.2.5 Systems, appliances and equipment outside the scope.

Delete 13 from the list.

d.	IFGC Section 303.3. Prohibited locations

Replace the words “toilet rooms” with “toilet facilities” in the section.

e.	IFGC Section 403.9.6 Welded joints

Add new section 403.9.6 to read:

403.9.6 Welded Joints. Welded joints shall be performed by a person holding a valid certificate of competency based on the requirements of the ANSI/ASME Boiler and Pressure Vessel Code, Section IX, Brazing and Welding Qualifications. Welded joints shall comply with ASTM 139.

f.	IFGC Section 404.14.1 Conduit with one end terminating outdoors

Delete in its entirety.

g.	IFGC Section 406.1 Inspection, Testing, and Purging

Add a new sentence to the end of the section reading:

Inspection and pressure testing shall apply to all temporary installations connected to a primary or temporary fuel gas source, natural or LP gas, for the purpose of supplying temporary heat.

h.	IFGC Section 409.5.3 Located at manifold

Delete in its entirety.

i.	IFGC 409.6 Shutoff valve for laboratories

Add new subsection 409.6.1 to read

409.6.1 Electric Solenoid Valve. A remotely located electric solenoid emergency shutoff valve may be used for compliance to Section 409.6, when all the following requirements are met.

(1)	The emergency control shutoff “panic button” shall be readily accessible, located within the laboratory space served, adjacent to all egress doors from the space and shall be identified by approved signage stating “Gas Shutoff”.

(2)	The gas solenoid valve shall be a “normally closed” type valve with a manual reset.

j.	IFGC Section 411.1 Connecting appliances

…

k.	IFGC Section 503.4.1 Plastic Piping,

…

4.	Revisions and exceptions to the International Residential Code Chapter 24 Fuel Gas incorporated as the Colorado Fuel Gas Code

a.	IRC Section G2414.9.6 Welded joints

Add new section G2414.10.5 Welded joints to read:

Welded joints. Welded joints shall be performed by a person holding a valid certificate of competency based on the requirements of the ANSI/ASME Boiler and Pressure Vessel Code, Section IX, Brazing and Welding Qualifications. Welded joints shall comply with ASTM 139.

b.	IRC 2415.14.1 Conduit with one end terminating outdoors

Delete in its entirety.

c.	IRC Section G2420.5.3 (Shutoffs) Located at manifolds

Delete in its entirety.

d.	IRC G2427.4.1 Plastic Piping,

Add new sentence to the end of the section saying:

Where installed as an exhaust vent for a gas fired water heater, the new plastic pipe shall be tested with 5 psi maximum air pressure or 10 ft. head water column at the time of inspection prior to being connected to the water heater.

Engineered UL-1738 vent systems do not require the pressure test as long as they are properly installed per the manufacturer's instructions.

…

_________________________________________________________________________

## Editor’s Notes

History

Entire rule eff. 01/01/2008.

Entire rule eff. 04/01/2010.

Rules 2.3.A, 2.4.1-2.4.2, 6.4 eff. 09/01/2011.

Entire rule eff. 03/15/2014.

Rules 2.3, 3.1 eff. 12/15/2014.

Entire rule eff. 02/14/2016.

Rules 2.5.1.27, 4.1, 4.2, 4.5.4, 4.5.5, 4.6-4.13, 6.1, 7.4 eff. 04/01/2016.

Rules 1.2 A-C, 1.2 D.4, 1.2 D.7-10, 1.2 E, 1.3, 1.4 A, 1.4 E, 1.6 B.8 eff. 06/14/2020. Rule 1.4.D repealed eff. 06/14/2020.

Rule 1.3 C eff. 08/30/2021.

Rule 1.4 L.2 eff. 12/15/2021.

Rule 1.10 emer. rule eff. 10/26/2022.

Rule 1.10 eff. 12/15/2022.

Rules 1.2, 1.3 A,B, 1.4 A,B,E,L, 1.5 E, 1.7 D eff. 04/14/2023.

Rule 1.2 E.1 eff. 12/15/2023. Rule 1.10 repealed eff. 12/15/2023.

Rules 1.1 C, 1.2 E.1.c, 1.2 E.2.c, 1.3 D, 1.4 A.3.e-f, 1.4 B.1, 1.4 E.2, 1.4 E.3.e(4)(b)(i), 1.4 E.4.a(1), 1.4 E.4.b(2)(c)(i), 1.4 F, 1.4 N, 1.7 B.1, 1.7 D, 1.9 eff. 04/30/2025.

Rules 1.2.D.10,.1.2.E.3.l, 1.3.B-E, 1.4.A.3, 1.4.E.2-4 eff. 02/14/2026.

Annotations

Rules 1.10 B. and 1.10 C. (adopted 10/26/2022) were not extended by Senate Bill 23-102 and therefore expired 05/15/2023.