colorado department of public health and environment

WATER QUALITY CONTROL COMMISSION

5 ccr 1002-33

REGULATION NO. 33

classifications and numeric standards
for
upper colorado river basin and
north platte river (planning region 12)

APPENDIX 33-1

Stream Classifications and Water Quality Standards Tables

Effective 12/31/2026

### Abbreviations and Acronyms

Aq	=	Aquatic
°C	=	degrees Celsius
CL	=	cold lake temperature tier
CLL	=	cold large lake temperature tier
CS-I	=	cold stream temperature tier one
CS-II	=	cold stream temperature tier two
D.O. 	=	dissolved oxygen
DM	=	daily maximum temperature
DUWS	=	direct use water supply
E. coli 	=	Escherichia coli
EQ	=	existing quality
mg/L 	=	milligrams per liter
mg/m2	=	milligrams per square meter
mL	=	milliliter
MWAT	=	maximum weekly average temperature
OW 	=	outstanding waters
sc	=	sculpin
SSE	=	site-specific equation
T	=	total recoverable
t	=	total
tr	=	trout
TVS	=	table value standard
µg/L	=	micrograms per liter
UP	=	use-protected
WS	=	water supply
WS-I	=	warm stream temperature tier one
WS-II	=	warm stream temperature tier two
WS-III	=	warm stream temperature tier three
WL	=	warm lake temperature tier

|  | 1. Mainstem of the Colorado River, including all tributaries and wetlands, within or flowing into Rocky Mountain National Park.. COUCUC01.. Designation.. OW..; Classifications.. Water Supply.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 2. Mainstem of the Colorado River, including all tributaries and wetlands, within or flowing into Arapahoe National Recreation Area, except for the specific listing in Segment 5.. COUCUC02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 3. Mainstem of the Colorado River from the outlet of Lake Granby to below the confluence with the Roaring Fork River.. COUCUC03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = See 33.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 4. All tributaries to the Colorado River, including all wetlands, from the outlet of Lake Granby to above the confluence with the Roaring Fork River, which are on National Forest lands, except for the specific listings in Segments 2, 8, 9 and 10a.. COUCUC04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 5. Mainstem of Willow Creek from the outlet of Willow Creek Reservoir to the confluence with the Colorado River.. COUCUC05.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 6a. All tributaries to the Colorado River, including all wetlands, from the border of Rocky Mountain National Park and Arapahoe National Recreation Area to a point immediately above the confluence with the Blue River and Muddy Creek, which are not on National Forest lands, except for the specific listings in Segments 5, 6b, 8 and 10a-c.. COUCUC06A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 6b. Mainstem of un-named tributary to Willow Creek from the headwaters to the confluence with Willow Creek (40.131422, -105.920895).. COUCUC06B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation N... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Manganese(T).; ---.; 200.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7a. All tributaries to the Colorado River, including all wetlands, from a point immediately above the confluence with the Blue River and Muddy Creek to a point immediately below the confluence with the Roaring Fork River, which are not on National Forest lands, except for waterbodies in Upper Colorado Segments 7b, 7c, 7d, and 7e and waterbodies in the Blue River, Eagle River, and Roaring Fork River basins.. COUCUC07A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = See 33.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 7b. All tributaries to Muddy Creek, including all wetlands, from the inlet of Wolford Mountain Reservoir to the confluence with the Colorado River. Mainstems of Rock Creek, Deep Creek, Sheephorn Creek, Sweetwater Creek, Piney River and Blacktail Creek, including all tributaries and wetlands, from their sources to their confluences with the Colorado River, which are not on National Forest lands.. COUCUC07B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 7c. Mainstem of Muddy Creek from the source to a point immediately below the confluence with Eastern Gulch, except those waters on National Forest lands. All tributaries to Muddy Creek, including all wetlands, from the source to the inlet of Wolford Mountain Reservoir, except those waters on National Forest lands. The mainstems of Derby Creek, Cabin Creek, and Red Dirt Creeks (all tributary to the Colorado River), including all tributaries and wetlands, from their sources to their confluences with the Colorado River, except those waters on National Forest lands.. COUCUC07C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation N.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 7d. Mainstem of Muddy Creek from the outlet of Wolford Mountain Reservoir to above the Highway 40 Bridge in Kremmling (40.060574, -106.398739).. COUCUC07D.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 7e. Mainstem of Muddy Creek from above the Highway 40 Bridge in Kremmling (40.060574, -106.398739) to the confluence with the Colorado River.. COUCUC07E.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. Mainstem of the Williams Fork River, including all tributaries and wetlands, from the source to the confluence with the Colorado River, except for those tributaries in Segment 9.. COUCUC08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Iron(chronic) = Point of compliance at Aspen Canyon Ranch well.. *Manganese(chronic) = Point of compliance at Aspen Canyon Ranch well.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS*.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS*.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 190.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 9. All tributaries to the Colorado River and Fraser River, including all wetlands, within the Never Summer, Indian Peaks, Byers Peak, Vasquez Peak, Eagles Nest and Flat Tops Wilderness Areas.. COUCUC09.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10a. Mainstem of the Fraser River from the source to a point immediately below the Rendezvous Bridge (39.933728, -105.789785). All tributaries to the Fraser River, including wetlands, from the source to the confluence with the Colorado River, except for those tributaries included in Segments 2 and 9.. COUCUC10A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 10b. Mainstem of the Fraser River from a point immediately below the Rendezvous Bridge (39.933728, -105.789785) to a point immediately below the Hammond No 1 Ditch (39.952113, -105.814481).. COUCUC10B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 10c. Mainstem of the Fraser River from a point immediately below the Hammond No 1 Ditch (39.952113, -105.814481) to the confluence with the Colorado River.. COUCUC10C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 11. All lakes and reservoirs tributary to the Colorado River within Rocky Mountain National Park, Never Summer, Indian Peaks, Byers Peak, Vasquez Peak, Eagles Nest and Flat Tops Wilderness Areas.. COUCUC11.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = DM and MWAT=CL,CLL from 1/1-3/31 Rim Lake DM=CL and MWAT=16.6 from 4/1-12/31 All others DM and MWAT=CL,CLL from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 12. Lakes and reservoirs within Arapahoe National Recreation Area, including Grand Lake, Shadow Mountain Lake and Lake Granby.. COUCUC12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:.. Goal Qualifier Grand Lake Clarity... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Goal Qualifier Grand Lake: 7/1-9/11, Clarity = 3.8 meter average and 2.5 meter minimum Secchi disk depth.. *Classification: DUWS applies to Grand Lake.. *Nitrogen(chronic) = applies only above the facilities listed at 33.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *clarity(chronic) = For Grand Lake, the highest level of clarity attainable, consistent with the exercise of established water rights, the protection of aquatic life, and protection of water quality throughout the Three Lakes system.. *Temperature = See 33.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*.; B.. acute.; chronic.. clarity.; ---.; narrative*.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13. All lakes and reservoirs tributary to the Colorado River from the boundary of Rocky Mountain National Park and Arapahoe National Recreation Area to a point immediately above the confluence with the Roaring Fork River, except for specific listings in Upper Colorado Segments 11 and 12 and the Blue River and Eagle River subbasins.. COUCUC13.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Classification: DUWS applies to Ute Creek Reservoir.. *Nitrogen(chronic) = applies only above the facilities listed at 33.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = See 33.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*.; B.. acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. Mainstem of the Blue River from the source to above the confluence with French Gulch.. COUCBL01.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 2a. Mainstem of the Blue River from above the confluence with French Gulch to a point one half mile below Coyne Valley Road (39.523189, -106.050805).. COUCBL02A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Zinc(acute) = e^(1.25 (ln(hard)+0.799)). *Zinc(chronic) = e^(1.25 (ln(hard)+0.799))..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; 4.; 4.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; SSE*.; SSE*.... |
| --- | --- |

|  | 2b. Mainstem of the Blue River from a point one half mile below Coyne Valley Road (39.523189, -106.050805) to above the confluence with the Swan River.. COUCBL02B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Cadmium(acute) = 1/2e^(1.0166(ln(hard)-3.132)). *Cadmium(chronic) = 1/2e^(1.0166(ln(hard)-3.132)). *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Zinc(acute) = e^(0.9805(ln(hard)+1.402)). *Zinc(chronic) = e^(0.9805(ln(hard)+1.402))..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; SSE*.; SSE*.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; SSE*.; SSE*.... 2c. Mainstem of the Blue River from above the confluence with the Swan River to Dillon Reservoir.. COUCBL02C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 3. Deleted.. COUCBL03.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... 4a. All direct tributaries, including wetlands, to Dillon Reservoir and all tributaries, including wetlands, to the Blue River above Dillon Reservoir, except for specific listings in Segments 1, 2a, 2b, 2c, 4b, 6a, 10-14 and 16.. COUCBL04A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 4b. North Fork of the Swan River, including all tributaries and wetlands, from the source to the confluence with the Swan River.. COUCBL04B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 5. Deleted.. COUCBL05.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... |
| --- | --- |

|  | 6a. Mainstem of the Snake River, including all tributaries and wetlands, from the source to Dillon Reservoir, except for specific listings in Segments 6b, 7, 8 and 9.. COUCBL06A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6b. Mainstem of Camp Creek, including all tributaries and wetlands, from the source to the confluence with the Snake River.. COUCBL06B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Zinc(acute) = 0.978*e^0.8537(ln Hardness)+1.5227. *Zinc(chronic) = 0.986*e^0.8537(ln Hardness)+1.3519..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... |
| --- | --- |

|  | 7. Mainstem of Peru Creek, including all tributaries and wetlands, from the source to the confluence with the Snake River, except for specific listings in Segment 8.. COUCBL07.. Designation.. UP..; Classifications.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; ---.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; ---.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; ---.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 8. Mainstem of Keystone Gulch, including all tributaries and wetlands, from the source to the confluence with the Snake River. Mainstem of Chihuahua Creek, including all tributaries and wetlands, from the source to the confluence with Peru Creek. Mainstem of the North Fork Snake River, including all tributaries and wetlands, from the source to the confluence with the Snake River. Mainstem of Jones Gulch, including all tributaries and wetlands, from the source to the confluence with the Snake River.. COUCBL08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 9. Mainstem of Deer Creek, including all tributaries and wetlands, from the source to the confluence with the Snake River.. COUCBL09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 10. Mainstem of French Gulch, including all tributaries and wetlands, from the source to a point 1.5 miles below Lincoln (39.484661, -105.995074).. COUCBL10.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11. Mainstem of French Gulch from a point 1.5 miles below Lincoln (39.484661, -105.995074) to the confluence with the Blue River.. COUCBL11.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Cadmium(acute) = existing quality. *Cadmium(chronic) = existing quality. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Zinc(acute) = existing quality. *Zinc(chronic) = existing quality..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; EQ*.; EQ*.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; EQ*.; EQ*.... 12. Mainstem of Illinois Gulch and Fredonia Gulch from their sources to their confluences with the Blue River.. COUCBL12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13. Mainstem of Tenmile Creek from the Climax Parshall Flume (39.447556, -106.157003) to a point immediately above the confluence of West Tenmile Creek and all tributaries and wetlands from the source of Tenmile Creek to a point immediately above the confluence with West Tenmile Creek, except for the specific listing in Segment 15.. COUCBL13.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P... Qualifiers:... Other:... *Any water quality based effluent limit shall not cause or contribute to exceedances of water quality standards adopted to protect downstream uses.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; ---.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 14. Mainstem of Tenmile Creek, including all tributaries and wetlands, from a point immediately above the confluence with West Tenmile Creek to Dillon Reservoir, except for the specific listings in Segment 16.. COUCBL14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 530.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 15. Mainstem of Clinton Creek from the source to the confluence with Tenmile Creek.. COUCBL15.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 210.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16. All tributaries to the Blue River, including all wetlands, within the Eagles Nest and Ptarmigan Peak Wilderness Areas.. COUCBL16.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 17. Mainstem of the Blue River from the outlet of Dillon Reservoir to the confluence with the Colorado River.. COUCBL17.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 18. All tributaries to the Blue River, including all wetlands, from the outlet of Dillon Reservoir to the outlet of Green Mountain Reservoir, except for the specific listings in Segment 16.. COUCBL18.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 19. All tributaries to the Blue River, including all wetlands, from the outlet of Green Mountain Reservoir to the confluence with the Colorado River, except for specific listings in Segment 20.. COUCBL19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation N.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 20. Mainstems of Elliot Creek and Spruce Creek, including all tributaries and wetlands, from their sources to the confluence with the Blue River.. COUCBL20.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation N.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 21. All lakes and reservoirs tributary to the Blue River within the Eagles Nest and Ptarmigan Peak Wilderness Areas.. COUCBL21.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 22. Dillon Reservoir and all lakes and reservoirs tributary to the Blue River above Dillon Reservoir, except for specific listings in Segment 21.. COUCBL22.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to Goose Pasture Tarn.. *Nitrogen(chronic) = applies only above the facilities listed at 33.5(4).. *Phosphorus(chronic) = 0.0074 mg/l for Dillon Reservoir in the top 15 meters of the water column for the months of July, August, September & October. Additional total phosphorus or Chla standards adopted for this segment do not apply to Dillon Reservoir.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; 0.0074*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 23. All lakes and reservoirs tributary to the Blue River below Dillon Reservoir, except for specific listings in Segment 21.. COUCBL23.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Nitrogen(chronic) = applies only above the facilities listed at 33.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = DM and MWAT=CL/CLL from 1/1-3/31 Green Mountain Reservoir DM=22.4 and MWAT=16.6 from 4/1-12/31 All others DM and MWAT=CL/CLL from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. All tributaries to the Eagle River, including all wetlands, within the Gore Range - Eagles Nest and Holy Cross Wilderness Areas. Mainstem of East Brush Creek from the source to 39.498914, -106.671722. All tributaries to East Brush Creek, including wetlands, excluding the following: tributaries and wetlands within Sylvan Lake State Park, the portion of Hat Creek below 39.511013, -106.688716, the portion of Nolan Creek below 39.525486, -106.686495, and the portion of the unnamed tributary below 39.512799, -106.680681; these excluded portions remain in Segment 10a. Mainstem of West Brush Creek from the source to the Sylvan Lake State Park boundary (39.469516, -106.729231). All tributaries to West Brush Creek, including wetlands, which are not within Sylvan Lake State Park.. COUCEA01.. Designation.. OW*..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Designation: Consistent with the provisions of section 25-8-104 C.R.S. the OW designation shall not apply with respect to the Homestake Water Project of the Cities of Aurora and Colorado Springs.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 2. Mainstem of the Eagle River from the source to above the compressor house bridge at Belden (39.526879, -106.394950).. COUCEA02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 3. All tributaries to the Eagle River, including wetlands, from the source to above the compressor house bridge at Belden (39.526879, -106.394950), except for the specific listings in Segments 1 and 4.. COUCEA03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 4. Mainstem of Homestake Creek from the confluence of the East Fork to the confluence with the Eagle River.. COUCEA04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 5a. Mainstem of the Eagle River from above the compressor house bridge at Belden (39.526879, -106.394950) to a point immediately above the Highway 24 Bridge near Tigiwon Road (39.554936, -106.401691).. COUCEA05A.. Designation.. Reviewable*..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Designation: 9/30/00 Baseline does not apply. *Cadmium(chronic) = (1.101672-[ln(hardness)*(0.041838)])* e^(0.7998 [ln (hardness)]-3.1725). *Copper(acute) = 0.96*e^0.9801[ln(hardness)] – 1.1073. *Copper(chronic) = 0.96*e^0.5897[ln(hardness)] – 0.0053. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Zinc(acute) = 0.978*e^0.8537[ln(hardness)]+2.1302. *Zinc(chronic) = 0.986*e^0.8537[ln(hardness)]+1.9593..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; SSE*.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; ---.; SSE*.. Copper.; SSE*.; ---.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... 5b. Mainstem of the Eagle River from a point immediately above the Highway 24 Bridge near Tigiwon Road (39.554936, -106.401691) to a point immediately above the confluence with Martin Creek.. COUCEA05B.. Designation.. Reviewable*..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Designation: 9/30/00 Baseline does not apply. *Cadmium(chronic) = (1.101672-[ln(hardness)*(0.041838)])* e^(0.7998 [ln (hardness)]-3.1725). *Copper(acute) = 0.96*e^0.9801[ln(hardness)]-1.5865. *Copper(chronic) = 0.96*e^0.5897[ln(hardness)]-0.4845. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Zinc(acute) = 0.978*e^0.8537[ln(hardness)]+2.1302 from 1/1 - 4/30 0.978*e^0.8537[ln(hardness)]+1.4189 from 5/1 - 12/31. *Zinc(chronic) = 0.986*e^0.8537[ln(hardness)]+1.9593 from 1/1 - 4/30 0.986*e^0.8537[ln(hardness)]+1.2481 from 5/1 - 12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; SSE*.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; ---.; SSE*.. Copper.; SSE*.; ---.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... |
| --- | --- |

|  | 5c. Mainstem of the Eagle River from a point immediately above Martin Creek to a point immediately above the confluence with Gore Creek.. COUCEA05C.. Designation.. Reviewable*..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Designation: 9/30/00 Baseline does not apply. *Cadmium(chronic) = (1.101672-[ln(hardness)*(0.041838)])* e^(0.7998 [ln (hardness)]-3.1725). *Copper(acute) = 0.96*e^0.9801[ln(hardness)]-1.5865. *Copper(chronic) = 0.96*e^0.5897[ln(hardness)]-0.4845. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Zinc(acute) = 0.978*e^0.8537[ln(hardness)]+1.4189. *Zinc(chronic) = 0.986*e^0.8537[ln(hardness)]+1.2481..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; SSE*.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; ---.; SSE*.. Copper.; SSE*.; ---.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... 6. All tributaries to the Eagle River, including all wetlands, from above the compressor house bridge at Belden (39.526879, -106.394950) to a point immediately below the confluence with Lake Creek, except for the specific listings in Segments 1, 7a, 7b, and 8.. COUCEA06.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 7a. Mainstem of Cross Creek from the source to below the Minturn Water Facility (39.565419, -106.417032), except for the specific listings in Segment 1.. COUCEA07A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 7b. Mainstem of Cross Creek from below the Minturn Water Facility (39.565419, -106.417032) to the confluence with the Eagle River.. COUCEA07B.. Designation.. Reviewable*..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Designation: 9/30/00 Baseline does not apply. *Cadmium(chronic) = (1.101672-[ln(hardness)*(0.041838)])* e^(0.7998 [ln (hardness)]-3.1725). *Copper(acute) = 0.96*e^0.9801[ln(hardness)]-1.5865. *Copper(chronic) = 0.96*e^0.5897[ln(hardness)]-0.4845. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Zinc(acute) = 0.978*e^0.8537[ln(hardness)]+2.1302 from 1/1 - 4/30 0.978*e^0.8537[ln(hardness)]+1.4189 from 5/1 - 12/31. *Zinc(chronic) = 0.986*e^0.8537[ln(hardness)]+1.9593 from 1/1 - 4/30 0.986*e^0.8537[ln(hardness)]+1.2481 from 5/1 - 12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; SSE*.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; ---.; SSE*.. Copper.; SSE*.; ---.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... |
| --- | --- |

|  | 8. Mainstem of Gore Creek from the confluence with Black Gore Creek to the confluence with the Eagle River.. COUCEA08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = DM=CS-I and MWAT=14 from 6/1-6/30 DM=CS-I and MWAT=CS-I from 7/1-9/30 DM=CS-I and MWAT=12 from 10/1-10/15 DM=CS-I and MWAT=CS-I from 10/16-5/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 9a. Mainstem of the Eagle River from above Gore Creek to a point immediately below the confluence with Squaw Creek.. COUCEA09A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = DM=CS-I and MWAT=16 from 6/1-6/30 DM=CS-I and MWAT=CS-I from 7/1-9/30 DM=CS-I and MWAT=12 from 10/1-10/15 DM=CS-I and MWAT=11 from 10/16-10/31 DM=CS-I and MWAT=CS-I from 11/1-5/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 9b. Mainstem of the Eagle River from a point immediately below the confluence with Squaw Creek to a point immediately below the confluence with Rube Creek.. COUCEA09B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = DM=15 and MWAT=12 from 4/1-5/31 DM and MWAT=CS-II from 6/1-9/30 DM=15 and MWAT=12 from 10/1-10/15 DM=15 and MWAT=11 from 10/16-10/31 DM and MWAT=CS-II from 11/1-3/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9c. Mainstem of the Eagle River from a point immediately below the confluence with Rube Creek to the confluence with the Colorado River.. COUCEA09C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10a. All tributaries to the Eagle River, including all wetlands, from a point immediately below the confluence with Lake Creek to the confluence with the Colorado River, except for specific listings in Segments 10b, 11 and 12, and those waters included in Segment 1.. COUCEA10A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 10b. Abrams Creek, including all tributaries and wetlands, from the source to the eastern boundary of the United States Bureau of Land Management lands.. COUCEA10B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11. Mainstem of Alkali Creek (near Wolcott) from the source to the confluence with the Eagle River. Mainstem of Milk Creek from the source to the confluence with the Eagle River.. COUCEA11.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation P... Qualifiers:.. Fish Ingestion Standards Apply... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Beryllium(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Manganese(T).; ---.; 200.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12. Mainstem of Brush Creek, from the confluence of West Brush Creek and East Brush Creek to the confluence with the Eagle River. Mainstem of East Brush Creek from 39.498914, -106.671722 to the confluence with West Brush Creek. Mainstem of West Brush Creek from the Sylvan Lake State Park boundary (39.469516, -106.729231) to the confluence with East Brush Creek.. COUCEA12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13. All lakes and reservoirs tributary to the Eagle River within the Gore Range - Eagles Nest and Holy Cross Wilderness Areas.. COUCEA13.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14. All lakes and reservoirs tributary to the Eagle River except for specific listings in Segment 13.. COUCEA14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. All tributaries to the Roaring Fork River, including all wetlands, within the Maroon Bells/Snowmass, Holy Cross, Raggeds, Collegiate Peaks and Hunter/Fryingpan Wilderness Areas. Avalanche Creek, including all tributaries and wetlands, from the Maroon Bells/Snowmass Wilderness boundary to the National Forest boundary (39.248331, -107.232393). Woody Creek, including all tributaries and wetlands, from the Hunter/Fryingpan Wilderness boundary to USFS Rd 103/Woody Creek Rd (39.244983, -106.751780). Hunter Creek, including all tributaries and wetlands, from the Hunter/Fryingpan Wilderness boundary to the National Forest boundary (39.205635, -106.798061).. COUCRF01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2. Mainstem of the Roaring Fork River, including all tributaries and wetlands, from the source to a point immediately below the confluence with Hunter Creek, except for those tributaries included in Segment 1.. COUCRF02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 3a. Mainstem of the Roaring Fork River, from a point immediately below the confluence with Hunter Creek, to a point immediately below the confluence with the Fryingpan River. All tributaries to the Roaring Fork River, including wetlands, from a point immediately below the confluence with Hunter Creek to the confluence with the Colorado River, except for those tributaries included in Segment 1, 3b, 3d, 4-10b.. COUCRF03A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3b. Mainstem of Red Canyon, including all tributaries and wetlands, from the source to the confluence with the Roaring Fork River, except for Landis Creek from the source to the Hopkins Ditch (39.522138, -107.223479).. COUCRF03B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3c. Mainstem of the Roaring Fork River from a point immediately below the confluence with the Fryingpan River to the confluence with the Colorado River.. COUCRF03C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = See 33.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3d. Mainstem of Cattle Creek, including all tributaries and wetlands, from the source to the most downstream White River National Forest boundary.. COUCRF03D.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4. Mainstem of Brush Creek from the source to the confluence with the Roaring Fork River.. COUCRF04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5. Mainstem of the Fryingpan River from the source to the confluence with the North Fork Fryingpan River, except for the portion included in Segment 1.. COUCRF05.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 6. Mainstem of the Fryingpan River from the confluence with the North Fork Fryingpan River to the confluence with the Roaring Fork River.. COUCRF06.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 7. All tributaries to the Fryingpan River, including all wetlands, from the source to the confluence with the Roaring Fork River, except for those tributaries included in Segment 1.. COUCRF07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 8. Mainstem of the Crystal River, including all tributaries and wetlands, from the source to the confluence with the Roaring Fork River, except for the specific listings in Segments 1, 9, 10a and 10b.. COUCRF08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9. Mainstem of Coal Creek, including all tributaries and wetlands, from the source to the confluence with the Crystal River.. COUCRF09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10a. Mainstem of Thompson Creek, including all tributaries and wetlands, from the source to the confluence with the Crystal River, except for specific listings in Segment 10b.. COUCRF10A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 10b. Mainstem of North Thompson Creek, including all tributaries and wetlands, from the source to the White River National Forest boundary (39.316522, -107.305749). Mainstem of Middle Thompson Creek, including all tributaries and wetlands, from the source to the confluence with Thompson Creek.. COUCRF10B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 11. All lakes and reservoirs tributary to the Roaring Fork River within the Maroon Bells/Snowmass, Holy Cross, Raggeds, Collegiate Peaks and Hunter/Fryingpan Wilderness Areas.. COUCRF11.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = DM and MWAT=CL,CLL from 1/1-3/31 Savage Lake, Ivanhoe Lake DM=CL and MWAT=16.6 from 4/1-12/31 All others DM and MWAT=CL,CLL from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12. All lakes and reservoirs tributary to the Roaring Fork River, except for the specific listings in Segment 11.. COUCRF12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to Leonard Thomas Reservoir and Wildcat Reservoir.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = DM and MWAT=CL,CLL from 1/1-3/31 Ruedi Reservoir DM=22.4 and MWAT=20.3 from 4/1-12/31 All others DM and MWAT=CL,CLL from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*.; B.. acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. All tributaries to the North Platte and Encampment Rivers, including all wetlands, within the Mount Zirkel, Never Summer, and Platte River Wilderness Areas.. COUCNP01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2. Mainstem of the Encampment River, including all tributaries and wetlands, from the source to the Colorado/Wyoming border, except for those tributaries included in Segment 1.. COUCNP02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3. Mainstem of the North Platte River from the confluence of Grizzly Creek and Little Grizzly Creek to the Colorado/Wyoming border.. COUCNP03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4a. All tributaries to the North Platte River, including all wetlands, from the source to the Colorado/Wyoming border, except for those tributaries included in Segments 1, 4b, 5a, 5b, 6, 7a and 7b.. COUCNP04A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4b. Mainstem of the Illinois River, including all tributaries and wetlands, from a point immediately below the confluence with Indian Creek to the confluence with the Michigan River, except for specific listings in Segments 7a and 7b. Mainstem of the Canadian River from below 12E Road (40.720033, -106.088912) to the confluence with the North Platte River. All tributaries to the Canadian River, including wetlands, which enter the mainstem from the southwest from below 12E Road to the confluence with the North Platte River.. COUCNP04B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5a. Mainstem of the Michigan River from the source to a point immediately below the confluence with the North Fork Michigan River.. COUCNP05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5b. Mainstem of the Michigan River from a point immediately below the confluence with the North Fork Michigan River to the confluence with the North Platte River.. COUCNP05B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation N.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6. Mainstem of Pinkham Creek from the Routt National Forest boundary to the confluence with the North Platte River.. COUCNP06.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation N.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 7a. Mainstem of Government Creek from the boundary of the Colorado State Forest to the confluence with the Canadian River. Mainstem of Spring Creek from the source to Spring Creek (Number 31) Reservoir.. COUCNP07A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation N... Qualifiers:.. Fish Ingestion Standards Apply... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7b. Mainstem of Spring Creek from the outlet of Spring Creek (Number 31) Reservoir to the confluence with the Illinois River.. COUCNP07B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:.. Fish Ingestion Standards Apply... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. All lakes and reservoirs tributary to the North Platte and Encampment Rivers within the Mount Zirkel, Never Summer, and Platte River Wilderness Areas.. COUCNP08.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = DM and MWAT=CL,CLL from 1/1-3/31 Blue Lake, Lower Big Twin Lake, Katherine Lake DM=CL and MWAT=16.6 from 4/1-12/31 All others DM and MWAT=CL,CLL from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9. All lakes and reservoirs tributary to the North Platte and Encampment Rivers except for specific listings in Segment 8.. COUCNP09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = See 33.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*.; B.. acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. All tributaries to the Yampa River, including all wetlands, which are within the Mount Zirkel, Flat Tops and Sarvis Creek Wilderness Areas. Walton Creek, including all tributaries and wetlands, from the source to below the confluence with Beaver Creek. Soda Creek, including all tributaries and wetlands, from the source to the National Forest boundary (40.541953, -106.790266). North Fork Elk River, including all tributaries and wetlands, from the Mount Zirkel Wilderness boundary to above the confluence with Lost Dog Creek. Middle Fork Elk River, including all tributaries and wetlands, from the Mount Zirkel Wilderness boundary to the confluence with the North Fork Elk River. South Fork Elk River, including all tributaries and wetlands, from the Mount Zirkel Wilderness boundary to the confluence with the Elk River.. COUCYA01.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 2a. Mainstem of the Yampa River from the confluence of the Bear River and Phillips Creek to a point immediately above the confluence with Oak Creek.. COUCYA02A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 2b. Mainstem of the Yampa River from a point immediately above the confluence with Oak Creek to a point immediately below the confluence with Elkhead Creek.. COUCYA02B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. temperature(MWAT) = current conditions*. Expiration Date of 12/31/2027.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = See 33.6(4) for temperature standards.. *TempMod: temperature = applies from 7/1-9/30. Adopted 6/10/2019..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 3. All tributaries to the Yampa River, including all wetlands, from the source to above the confluence with the Elk River, except for specific listings in Segments 1 and 4-7. Mainstem of the Bear River, including all tributaries and wetlands, from the boundary of the Flat Tops Wilderness Area to the confluence with the Yampa River.. COUCYA03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 4. Mainstem of Little White Snake Creek from the source to the confluence with the Yampa River.. COUCYA04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation N.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5. Mainstem of Chimney Creek and Phillips Creek, including all tributaries and wetlands, which are not on National Forest lands, from their sources to the confluence with the Yampa River.. COUCYA05.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Water Supply.. Recreation P... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6. Mainstem of Oak Creek, including all tributaries and wetlands, from the source to a point 0.25 mile below County Road 27 (40.279241, -106.965405).. COUCYA06.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7. Mainstem of Oak Creek, including all tributaries and wetlands, from a point 0.25 mile below County Road 27 (40.279241, -106.965405) to the confluence with the Yampa River.. COUCYA07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation P.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. Discharger Specific Variance(s):.. Nitrate(acute) = See Section 33.6(6) for details on the variance for the Town of Oak Creek.. Expiration Date of 6/30/2026.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. Mainstem of the Elk River, including all tributaries and wetlands, from the source to the confluence with the Yampa River, except for those tributaries included in Segments 1 and 20a. Mainstem of the West Fork Elk River from the source to the confluence with the Yampa River.. COUCYA08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 9. Deleted.. COUCYA09.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... |
| --- | --- |

|  | 10. Deleted.. COUCYA10.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... 11. Fish Creek, including all tributaries and wetlands, from the source to County Road 27 (40.355559, -107.105131), except for specific listings in Segment 20a.. COUCYA11.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Water Supply.. Recreation N... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Manganese(T).; ---.; 200.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 12. All tributaries to the Yampa River, including all wetlands, from above the confluence with the Elk River to above the confluence with Elkhead Creek, except for specific listings in Segments 8, 11, 13a-13j and 20a.. COUCYA12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Manganese(T).; ---.; 200.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13a. Mainstem of Trout Creek, including all tributaries and wetlands, from the source to the headgate of Spruce Hill Ditch (40.317190, -107.005110), except for specific listings in Segments 1 and 20a. Mainstem of Middle Creek, including all tributaries and wetlands, from the source to County Road 27 (40.339183, -107.025533), except for specific listings in Segment 20a.. COUCYA13A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 13b. Mainstem of Foidel Creek, including all tributaries and wetlands, from the source to the confluence with Middle Creek. Mainstem of Fish Creek, including all tributaries and wetlands, from County Road 27 (40.355559, -107.105131) to the confluence with Trout Creek, except for specific listings in Segment 13g. Mainstem of Middle Creek, including all tributaries and wetlands, from County Road 27 (40.339183, -107.025533) to the confluence with Trout Creek.. COUCYA13B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Iron(T)(chronic) = See section 33.6(4) for standards and assessment locations for Foidel Creek and Middle Creek.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = See 33.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; varies*.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13c. Mainstem of Trout Creek, including all tributaries and wetlands, from the headgate of Spruce Hill Ditch (40.317190, -107.005110) to the confluence with Fish Creek, except for specific listings in Segment 13b.. COUCYA13C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13d. Mainstem of Dry Creek, including all tributaries and wetlands, from the source to above the confluence with Temple Gulch.. COUCYA13D.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Iron(T)(chronic) = See section 33.6(4) for standards and assessment locations.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; varies*.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13e. Mainstem of Sage Creek, including all tributaries and wetlands, from the source to the confluence with the Yampa River.. COUCYA13E.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Water Supply.. Recreation N... Qualifiers:... Other:... *Iron(T)(chronic) = See section 33.6(4) for standards and assessment locations for Sage Creek.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; varies*.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13f. Mainstem of Trout Creek, including all tributaries and wetlands, from a point immediately below the confluence with Fish Creek to the confluence with the Yampa River.. COUCYA13F.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = See 33.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13g. All tributaries to Fish Creek from the confluence with Cow Camp Creek (40.398773, -107.016467) to the confluence with Trout Creek.. COUCYA13G.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Selenium(chronic) = See section 33.6(4) for standards and assessment locations for Bond Creek and Cow Camp Creek... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium; ---; varies*. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13h. Mainstem of Dry Creek (near Hayden), including all tributaries and wetlands, from above the confluence with Temple Gulch to the confluence with the Yampa River.. COUCYA13H.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13i. Mainstem of Grassy Creek, including all tributaries and wetlands, from the source to immediately above the confluence with Scotchmans Gulch.. COUCYA13I.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13j. Mainstem of Grassy Creek (near Hayden), including all tributaries and wetlands, from above the confluence with Scotchmans Gulch to the confluence with the Yampa River.. COUCYA13J.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14. Mainstem of Elkhead Creek, including all tributaries and wetlands, from the boundary of the National Forest lands, to a point immediately below the confluence with Calf Creek. Dry Fork Elkhead Creek, including all tributaries and wetlands, from the source to a point immediately below 80A Road (40.612676, -107.228533), which are not on National Forest lands.. COUCYA14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15. Mainstem of Elkhead Creek, including all tributaries and wetlands, from a point immediately below the confluence with Calf Creek to the confluence with the Yampa River. Dry Fork Elkhead Creek, including all tributaries and wetlands, from a point immediately below 80A Road (40.612676, -107.228533) to the confluence with Elkhead Creek.. COUCYA15.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16. Deleted.. COUCYA16.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... |
| --- | --- |

|  | 17. Deleted.. COUCYA17.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... 18. South Fork Little Snake River and Middle Fork Little Snake River, including all tributaries and wetlands, from their sources to the confluence with the Little Snake River, which are not on National Forest lands. North Fork Little Snake River, including all tributaries and wetlands, from the Colorado/Wyoming border to the confluence with the Little Snake River.. COUCYA18.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... |
| --- | --- |

|  | 19a. All tributaries to the South Fork Little Snake River, including all wetlands, which are on National Forest lands in Routt County.. COUCYA19A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 19b. Middle Fork Little Snake River, including all tributaries and wetlands, which are on National Forest lands in Routt County.. COUCYA19b.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc)... 20a. All tributaries to the Yampa River, including all wetlands, from above the confluence with the Elk River to below the confluence with Elkhead Creek, which are on National Forest lands, except for specific listings in Segment 20b and 20c.. COUCYA20A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 20b. Mainstem of First Creek from the eastern boundary of state lands in California Park (40.731309, -107.141684) to the confluence with Elkhead Creek. Mainstem of Elkhead Creek from the eastern boundary of state lands in California Park (40.743796, -107.141684) to the National Forest boundary.. COUCYA20B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation N.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 20c. First Creek, including all tributaries and wetlands, from the source to the eastern boundary of state lands in California Park (40.731309, -107.141684). Elkhead Creek, including all tributaries and wetlands, from the source to the eastern boundary of state lands in California Park (40.743796, -107.141684).. COUCYA20C.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS/TVS(sc).... 21. All lakes and reservoirs tributary to the Yampa River within the Mount Zirkel, Flat Tops and Sarvis Creek Wilderness Areas, except for those lakes and reservoirs included in Lower Yampa River Segment 28.. COUCYA21.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 22. All lakes and reservoirs tributary to the Yampa River from the source to the confluence with Elkhead Creek, except for those listed in Segment 21. All lakes and reservoirs tributary to Elkhead Creek from the source to the confluence with the Yampa River, except for specific listings in Segment 23. All lakes and reservoirs tributary to the Little Snake River, including those on National Forest lands.. COUCYA22.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Classification: DUWS applies to Stagecoach Reservoir, Steamboat Lake, and Yampa River Holding Pond.. *Nitrogen(chronic) = applies only above the facilities listed at 33.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 33.5(4).. *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details.. *Temperature = See 33.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*.; B.. acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 23. Elkhead Reservoir. COUCYA23.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 33.5(3) for details.. *Uranium(chronic) = See 33.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

STREAM CLASSIFICATIONS and WATER QUALITY STANDARDS – FOOTNOTES

(A)	Whenever a range of standards is listed and referenced to this footnote, the first number in the range is a strictly health-based value, based on the Commission’s established methodology for human health-based standards. The second number in the range is a maximum contaminant level, established under the federal Safe Drinking Water Act that has been determined to be an acceptable level of this chemical in public water supplies, taking treatability and laboratory detection limits into account. Control requirements, such as discharge permit effluent limitations, shall be established using the first number in the range as the ambient water quality target, provided that no effluent limitation shall require an “end-of-pipe” discharge level more restrictive than the second number in the range. Water bodies will be considered in attainment of this standard, and not included on the Section 303(d) List, so long as the existing ambient quality does not exceed the second number in the range.

(B) 	Assessment of adequate refuge shall rely on the Cold Large Lake table value temperature criterion and applicable dissolved oxygen standard rather than the site-specific temperature standard.