# COLORADO DEPARTMENT OF PUBLIC HEALTH AND ENVIRONMENT

WATER QUALITY CONTROL COMMISSION

5 CCR 1002-32

REGULATION NO. 32 
CLASSIFICATIONS AND NUMERIC STANDARDS 
FOR 
ARKANSAS RIVER BASIN

APPENDIX 32-1

Stream Classifications and Water Quality Standards Tables

Effective 12/31/2026

### Abbreviations and Acronyms

Aq	=	Aquatic
°C	=	degrees Celsius
CL	=	cold lake temperature tier
CLL	=	cold large lake temperature tier
CS-I	=	cold stream temperature tier one
CS-II	=	cold stream temperature tier two
D.O. 	=	dissolved oxygen
DM	=	daily maximum temperature
DUWS	=	direct use water supply
E. coli 	=	Escherichia coli
EQ	=	existing quality
mg/L 	=	milligrams per liter
mg/m2	=	milligrams per square meter
mL	=	milliliter
MWAT	=	maximum weekly average temperature
OW 	=	outstanding waters
SSE	=	site-specific equation
T	=	total recoverable
t	=	total
tr	=	trout
TVS	=	table value standard
µg/L	=	micrograms per liter
UP	=	use-protected
WS	=	water supply
WS-I	=	warm stream temperature tier one
WS-II	=	warm stream temperature tier two
WS-III	=	warm stream temperature tier three
WL	=	warm lake temperature tier

|  | 1a. All streams and wetlands within Mount Massive and Collegiate Peaks Wilderness areas.. COARUA01A.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 1b. Mainstem of the East Fork Arkansas River from its source to a point immediately above the confluence with Birdseye Gulch.. COARUA01B.. Designation.. Reviewable..; Classifications.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; ---.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 210.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 2a. Mainstem of the East Fork Arkansas River and the Arkansas River from a point immediately above the confluence with Birdseye Gulch to a point immediately above the confluence with California Gulch.. COARUA02A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2b. Mainstem of the Arkansas River from a point immediately above California Gulch to a point immediately above the confluence with Lake Fork.. COARUA02B.. Designation.. Reviewable*..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Designation: 9/30/00 Base-line does not apply. *Cadmium(chronic) = (1.101672-[ln(hardness)*0.041838])*e^(0.7998[In hardness]-3.1725). *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Zinc(acute) = 0.978*e^(0.8537[ln(hardness)]+2.2178). *Zinc(chronic) = 0.986*e^(0.8537[ln(hardness)]+2.0469)..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; SSE*.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... |
| --- | --- |

|  | 2c. Mainstem of the Arkansas River from a point immediately above the confluence with the Lake Fork to a point immediately above the confluence with Lake Creek.. COARUA02C.. Designation.. Reviewable*..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Designation: 9/30/00 Base-line does not apply. *Cadmium(chronic) = (1.101672-[ln(hardness)*0.041838])*e^(0.7998[In hardness]-3.1725). *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Zinc(acute) = 0.978*e^(0.8537[ln(hardness)]+2.2178). *Zinc(chronic) = 0.986*e^(0.8537[ln(hardness)]+2.0469)..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; SSE*.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... 3. Mainstem of the Arkansas River from a point immediately above the confluence with Lake Creek to the Chaffee/Fremont County line.. COARUA03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4a. Mainstem of the Arkansas River from the Chaffee/Fremont County Line to a point immediately above Highway 115 bridge (38.390243, -105.068648), due east of Florence.. COARUA04A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = DM and MWAT=CS-II from 11/1-3/31 DM= 24.8 and MWAT=22.1 from 4/1-10/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4b. Mainstem of the Arkansas River from a point immediately above Highway 115 bridge (38.390243, -105.068648), due east of Florence, to the inlet of Pueblo Reservoir.. COARUA04B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5a. All tributaries to the Arkansas River, including wetlands, from the source to immediately below the confluence with Browns Creek, except for specific listings in segments 5b through 12b.. COARUA05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5b. Mainstem of Trout Creek from its source to Trout Creek Reservoir, including all tributaries and wetlands.. COARUA05B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6. Mainstem of California Gulch, including all tributaries and wetlands, from the source to the confluence with the Arkansas River. Mainstem of St. Kevin's Gulch from the source to the confluence with Tennessee Creek.. COARUA06.. Designation.. Reviewable..; Classifications.. Agriculture.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT... acute.; chronic.. D.O. (mg/L).; ---.; ---.. pH.; ---.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; ---.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; ---.; ---.. Nitrate.; ---.; ---.. Nitrite.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic.; ---.; ---.. Cadmium.; ---.; ---.. Chromium III.; ---.; ---.. Chromium VI.; ---.; ---.. Copper.; ---.; ---.. Iron.; ---.; ---.. Lead.; ---.; ---.. Manganese.; ---.; ---.. Mercury(T).; ---.; ---.. Molybdenum(T).; ---.; ---.. Nickel.; ---.; ---.. Selenium.; ---.; ---.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc.; ---.; ---.... 7. Mainstem of Evans Gulch from the source to the confluence with the Arkansas River.. COARUA07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8a. Mainstem of Iowa Gulch from the source to the historic upper ASARCO water supply intake at 39.224327, -106.223432.. COARUA08A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Cadmium(acute) = (1.136672-[ln(hardness)*0.041838]*e^(0.9789*ln(hardness)-3.5146). *Cadmium(chronic) = (1.101672-[ln(hardness)*0.041838])*e^(0.7977*ln(hardness)-3.5338). *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Zinc(acute) = 0.978*e^(0.8571[ln(hardness)]+1.3673). *Zinc(chronic) = 0.986*e^(0.8571[ln(hardness)]+1.1711)..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; ---.; SSE*.. Cadmium.; SSE*.; ---.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... 8b. Mainstem of Iowa Gulch from a point immediately below the historic upper ASARCO water supply intake at 39.224327, -106.223432 to a point immediately below the headgate of the Paddock #1 Ditch (Iowa Ditch) at 39.215532, -106.286037.. COARUA08B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Cadmium(acute) = (1.136672-[ln(hardness)*0.041838]*e^(0.9789*ln(hardness)-3.5146). *Cadmium(chronic) = (1.101672-[ln(hardness)*0.041838])*e^(0.7977*ln(hardness)-3.5338). *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Zinc(acute) = 0.978*e^(0.8571[ln(hardness)]+1.3673). *Zinc(chronic) = 0.986*e^(0.8571[ln(hardness)]+1.1711)..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; ---.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; ---.; SSE*.. Cadmium.; SSE*.; ---.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... |
| --- | --- |

|  | 9. Mainstem of Iowa Gulch from a point immediately below the headgate of the Paddock #1 Ditch (Iowa Ditch) at 39.215532, -106.286037 to the confluence with the Arkansas River.. COARUA09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Cadmium(acute) = (1.136672-[ln(hardness)*0.041838]*e^(0.9789*ln(hardness)-3.5146). *Cadmium(chronic) = (1.101672-[ln(hardness)*0.041838])*e^(0.7977*ln(hardness)-3.5338). *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Zinc(acute) = 0.978*e^(0.8571[ln(hardness)]+1.3673). *Zinc(chronic) = 0.986*e^(0.8571[ln(hardness)]+1.1711)..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; ---.; SSE*.. Cadmium.; SSE*.; ---.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; SSE*.. Zinc.; SSE*.; ---.... 10. Mainstem of Lake Creek, including all tributaries and wetlands, from the source to the confluence with the Arkansas River, except for the specific listing in segment 11.. COARUA10.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; 14.6.; 10.6.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11. Mainstem of South Fork of Lake Creek, including all tributaries and wetlands, from the source to the confluence with Lake Creek.. COARUA11.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 5.0-9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Aluminum(T).; 750.; ---.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12a. Mainstem of Chalk Creek from the source to the confluence with the Arkansas River.. COARUA12A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 12b. Mainstem of Cottonwood Creek (Chaffee County), from the source to the confluence with the Arkansas River; South Fork Arkansas River, including all tributaries and wetlands, from the National Forest boundary to the confluence with the Arkansas River.. COARUA12B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13. All tributaries to the Arkansas River, including wetlands, which are on National Forest lands, from the confluence with Browns Creek to the inlet to Pueblo Reservoir, except for waterbodies in segments 14a, 14c and 15-27.. COARUA13.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 14a. Mainstem of Big Red Creek, Little Red Creek, and Hardscrabble Creek from their sources to their confluence with the Arkansas River.. COARUA14A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:.. Fish Ingestion Standards Apply... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14b. All tributaries to the Arkansas River, including wetlands, which are not on National Forest lands, from the confluence with Browns Creek to the Chaffee/Fremont County line.. COARUA14B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 14c. Mainstems of North and South Hardscrabble Creeks, including all tributaries and wetlands, from their sources to their confluences.. COARUA14C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = DM and MWAT=CS-I from 11/1-5/31 DM=22.1 and MWAT=17 from 6/1-10/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14d. All tributaries to the Arkansas River, including wetlands, which are not on National Forest lands, from immediately above the confluence of 6-mile Creek (38.405677, -105.122321) to the inlet to Pueblo Reservoir, except for specific listings in segments 14a, 14c, 14e, 14f, and 15-27.. COARUA14D.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic; 340; ---.. Arsenic(T).; ---.; 7.6.. Beryllium(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III; TVS; TVS. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 14e. All tributaries to the Arkansas River, including wetlands, which are not on National Forest lands from the Chaffee/Fremont County line to immediately below the confluence with Chandler Creek (38.407024, -105.137940). Newlin Creek (except for listings in segment 15b), Mineral Creek, Adobe Creek, and Oak Creek, including all tributaries and wetlands which are not on National Forest Service Land.. COARUA14E.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14f. Turkey Creek including all tributaries and wetlands from its source to immediately below the confluence with Little Turkey Creek at 38.594727, -104.851458.. COARUA14F.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic; 340; ---.. Arsenic(T).; ---.; 7.6.. Beryllium(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III; TVS; TVS. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15a. Mainstem of Badger Creek from the source to the confluence with the Arkansas River, including all tributaries and wetlands. Mainstem of Texas Creek from the Forest Service boundary to the confluence with the Arkansas River, including all tributaries and wetlands which are not on Forest Service land.. COARUA15A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 15b. Mainstem of Grape Creek, including all tributaries and wetlands, from the source to the outlet of De Weese Reservoir, except for waterbodies in Upper Arkansas segment 25 and Middle Arkansas segment 1. Mainstems of Hayden, Hamilton, Stout, and Big Cottonwood Creeks, including all tributaries and wetlands, from the sources to the confluences with the Arkansas River, except for waterbodies in Middle Arkansas segment 1. Tributaries and wetlands to Texas Creek which are on Forest Service land, except for waterbodies in Middle Arkansas segment 1. Mainstem of Newlin Creek from the National Forest boundary to County Road 92 (38.300765, -105.140927).. COARUA15B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 16a. Mainstem of Middle Tallahassee Creek, including all tributaries and wetlands, from the source to the intersection with Road 23.. COARUA16A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16b. Mainstem of North Tallahassee Creek, South Tallahassee Creek, Middle Tallahassee Creek, and Tallahassee Creek from their sources to a point immediately below their confluence with South Tallahassee Creek, except for the specific listing in segment 16a.. COARUA16B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 16c. Mainstem of Tallahassee Creek from a point immediately below the confluence with South Tallahassee Creek to the confluence with the Arkansas River.. COARUA16C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 17a. Mainstem of Cottonwood Creek (Fremont County), including all tributaries and wetlands, from the source to a point immediately below the confluence with North Waugh Creek.. COARUA17A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 17b. Mainstem of Cottonwood Creek (Fremont county), including all tributaries and wetlands, from a point immediately below the confluence with North Waugh Creek to the intersection with F6 Road.. COARUA17B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 17c. Mainstem of Cottonwood Creek from F6 Road to the confluence with Currant Creek.. COARUA17C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies.; varies.. Zinc.; ---.; TVS.... |
| --- | --- |

|  | 18. Mainstem of Currant Creek (Park County), including all tributaries and wetlands, from the source to the confluence with Tallahassee Creek, except for the specific listings in 17a, 17b, and 17c.. COARUA18.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 19. Mainstem of Fourmile Creek, including all tributaries and wetlands, from the source to immediately below the confluence with High Creek.. COARUA19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 20a. Mainstem of Fourmile Creek, including all tributaries and wetlands, from immediately below the confluence with High Creek to a point immediately above the confluence with Long Gulch, except for the specific listing to segment 23.. COARUA20A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... Discharger Specific Variance(s): Fluoride(acute) = See Section 32.6(6) for details on the variance for Cripple Creek & Victor Gold Mining Company, LLC. Expiration Date of 12/31/2031. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = DM=14.2 and MWAT=9.7 from 11/1-2/29 DM= 27.1 and MWAT=21 from 3/1-10/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 20b. Mainstem of Fourmile Creek, including all tributaries and wetlands, from the confluence with Long Gulch to the confluence with the Arkansas River.. COARUA20B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. Discharger Specific Variance(s): Fluoride(acute) = See Section 32.6(6) for details on the variance for Cripple Creek & Victor Gold Mining Company, LLC. Expiration Date of 12/31/2031. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Fluoride(acute) = 2.12 mg/L applies to the mainstem of Fourmile Creek; TVS applies to all tributaries and wetlands.. *Sulfate(chronic) = See 32.73(B) for existing quality as of January 1, 2000 for Water Supply.. *Manganese(chronic) = See 32.73(B) for existing quality as of January 1, 2000 for Water Supply.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = DM=13 and MWAT=9.4 from 11/1-2/29 DM= 28.1 and MWAT=22 from 3/1-10/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Fluoride; varies*; ---. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS*.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS*.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 21a. Mainstem of Cripple Creek from the source to a point 1.5 miles upstream of the confluence with Fourmile Creek.. COARUA21A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS(sa).; TVS(ela).. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 21b. Mainstem of Cripple Creek from a point 1.5 miles upstream of the confluence with Fourmile Creek to the confluence with Fourmile Creek.. COARUA21B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS(sp).; TVS(elp).. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 22a. Mainstem of Arequa Gulch from the source to the confluence with Cripple Creek.. COARUA22A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.0 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Aluminum(T).; 11000.; 11000.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; 5903.; 3674.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; 3500.; 600.... 22b. Squaw Gulch from the source to the confluence with Cripple Creek.. COARUA22B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 5.0.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 200.. Cadmium(T).; ---.; 50.. Chromium III(T).; ---.; 1000.. Chromium VI(T).; ---.; 1000.. Copper(T).; ---.; 500.. Iron.; ---.; ---.. Lead(T).; ---.; 100.. Manganese.; ---.; ---.. Mercury(T).; ---.; 10.. Molybdenum(T).; ---.; 150.. Nickel.; ---.; ---.. Selenium(T).; ---.; 50.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 25000.... |
| --- | --- |

|  | 23. Mainstem of Wilson Creek (Teller County), including all tributaries and wetlands, from the source to the confluence with Fourmile Creek.. COARUA23.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 24. Mainstem of East and West Beaver Creeks, including all tributaries and wetlands, from the source to the confluence with Beaver Creek; mainstem of Beaver Creek from the source to the point of diversion to Brush Hollow Reservoir.. COARUA24.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 25. Mainstem of Cottonwood Creek (Custer County) from the headwaters to 37.940597, -105.411656, except for waterbodies in Middle Arkansas Segment 1.. COARUA25.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 26. Mainstem of Beaver Creek from the point of diversion for Brush Hollow Reservoir to the confluence with the Arkansas River.. COARUA26.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 27. Mainstem of Eightmile Creek, including all tributaries and wetlands, from the source to the mouth of Phantom Canyon (38.495270, -105.110024).. COARUA27.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 28. All lakes and reservoirs within the Mount Massive and Collegiate Peaks Wilderness areas.. COARUA28.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 29. All lakes and reservoirs tributary to the Arkansas River from the source to immediately below the confluence with Browns Creek, except for specific listings in segments 28 and 30.. COARUA29.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 30. Turquoise Reservoir, Clear Creek Reservoir, Twin Lakes and Mt. Elbert Forebay.. COARUA30.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Twin Lakes and Elbert Forebay.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = DM and MWAT=CLL from 1/1-3/31 Turquoise Reservoir, Twin Lakes (Upper and Lower), Mt. Elbert Forebay DM=22.4 and MWAT=16.6 from 4/1-12/31 All others DM and MWAT=CLL from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 31. All lakes and reservoirs tributary to the Arkansas River which are on National Forest lands, from the confluence with Browns Creek to the inlet to Pueblo Reservoir, except for specific listings in segments 32 and 34-40.. COARUA31.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 32. All lakes and reservoirs tributary to the South Fork of the Arkansas from the source to the confluence with the Arkansas River.. COARUA32.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 33. All lakes and reservoirs tributary to the Arkansas River which are not on National Forest lands, from the confluence with Browns Creek to the inlet to Pueblo Reservoir, except for specific listings in segments 32 and 34-40.. COARUA33.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 34. All lakes and reservoirs tributary to the mainstems of Texas, Badger, Hayden, Hamilton, Stout, and Big Cottonwood Creeks from their sources to their confluences with the Arkansas River. All lakes and reservoirs tributary to the mainstem of Grape Creek from the source to the outlet of DeWeese Reservoir, except for the specific listing in segment 35.. COARUA34.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 35. DeWeese Reservoir.. COARUA35.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = DM=CLL and MWAT=CLL from 1/1-3/31 DM= CLL and MWAT=21.3 from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 36. All lakes and reservoirs tributary to the mainstem of Currant Creek (Park County) from the source to the confluence with Tallahassee Creek, except lakes and reservoirs tributary to Cottonwood Creek (Fremont County) from a point immediately below the confluence with North Waugh Creek to the intersection with F6 Road. All lakes and reservoirs tributary to the mainstem of Middle Tallahassee Creek from the source to the intersection with Road 23.. COARUA36.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 37. All lakes and reservoirs tributary to the mainstem of Fourmile Creek from the source to the confluence with the Arkansas River. This segment includes Wrights Reservoir.. COARUA37.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to Ott Reservoir.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 38. All lakes and reservoirs tributary to the mainstem of East and West Beaver Creeks from the source to the confluence with Beaver Creek. This segment includes Skagway and Bison Reservoirs.. COARUA38.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Classification: DUWS applies to Bison Reservoir.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 39. All lakes and reservoirs tributary to the mainstem of Eightmile Creek from the source to the mouth of Phantom Canyon (38.495270, -105.110024).. COARUA39.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 40. Brush Hollow Reservoir.. COARUA40.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 41. Teller Reservoir.. COARUA41.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CLL.; CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. All tributaries, including wetlands, to the Arkansas River within the Sangre de Cristo, Greenhorn, and Spanish Peaks Wilderness Areas.. COARMA01.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2. Mainstem of the Arkansas River from the outlet of Pueblo Reservoir to a point immediately above the confluence with Wildhorse/Dry Creek Arroyo.. COARMA02.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = See 32.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3. Mainstem of the Arkansas River from a point immediately above the confluence with Wildhorse/Dry Creek Arroyo to a point immediately above the confluence with Fountain Creek.. COARMA03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; 26.3.; 17.1.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4a. Mainstem of Wildhorse Creek from the source to the confluence with the Arkansas River.. COARMA04A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Selenium(acute) = See selenium assessment location at 32.6(4).. *Selenium(chronic) = See selenium assessment location at 32.6(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; 2376*.; 2110*.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4b. Mainstem of Rock Creek, Salt Creek and Peck Creek from their sources to the confluence with the Arkansas River.. COARMA04B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4c. Mainstem of Chico Creek, including all tributaries and wetlands, from the source to the confluence with the Arkansas River, except for specific listings in segment 4f.. COARMA04C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Water Supply.. Recreation E... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4d. All tributaries, including wetlands, to the Arkansas River and Pueblo Reservoir from the inlet to Pueblo Reservoir to the Colorado Canal headgate, except for specific listings in the Fountain Creek Subbasin and in segments 4a, 4b, 4c and 4e through 18b.. COARMA04D.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Water Supply.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. =*Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 10.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 0.02-10.; A. Beryllium(T).; ---.; 100.. Cadmium(T).; 5.0.; 10.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI(T).; ---.; 100.. Copper(T).; ---.; 200.. Iron.; ---.; WS.. Lead(T).; 50.; 100.. Manganese.; ---.; WS.. Mercury(T).; 2.0.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 100.. Selenium(T).; ---.; 20.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... 4e. Golf Course Wash.. COARMA04E.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Beryllium(T).; ---.; 100.. Cadmium(T).; ---.; 10.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI(T).; ---.; 100.. Copper(T).; ---.; 200.. Iron.; ---.; ---.. Lead(T).; ---.; 100.. Manganese.; ---.; ---.. Mercury(T).; ---.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 200.. Selenium.; TVS.; TVS.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... |
| --- | --- |

|  | 4f. Mainstem of Black Squirrel Creek, including all tributaries and wetlands, from just below Highway 94 to Squirrel Creek Road.. COARMA04F.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation P... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 205... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 100.. Beryllium(T).; ---.; 100.. Cadmium(T).; ---.; 10.. Chromium III(T).; ---.; 100.. Chromium VI(T).; ---.; 100.. Copper(T).; ---.; 200.. Iron.; ---.; ---.. Lead(T).; ---.; 100.. Manganese(T).; ---.; 200.. Mercury(T).; ---.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 200.. Selenium(T).; ---.; 20.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... 4g. Mainstem of Pesthouse Gulch, from the source to the confluence with Wildhorse Creek.. COARMA04G.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Selenium(acute) = See selenium assessment location at 32.6(4).. *Selenium(chronic) = See selenium assessment location at 32.6(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 100.. Beryllium(T).; ---.; 100.. Cadmium(T).; ---.; 10.. Chromium III(T).; ---.; 100.. Chromium VI(T).; ---.; 100.. Copper(T).; ---.; 200.. Iron.; ---.; ---.. Lead(T).; ---.; 100.. Manganese(T).; ---.; 200.. Mercury(T).; ---.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 200.. Selenium.; 389*.; 369*.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... |
| --- | --- |

|  | 5a. Mainstem of the Saint Charles River, including all tributaries and wetlands, from the source to the San Isabel National Forest boundary.. COARMA05A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5b. Mainstem of the Saint Charles River, including all tributaries and wetlands, from the San Isabel National Forest boundary to a point immediately above the CF&I diversion canal (38.045800, -104.802787) near Burnt Mill.. COARMA05B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6a. Mainstem of the Saint Charles River from a point immediately above the CF&I diversion canal (38.045800, -104.802787) near Burnt Mill to a point immediately upstream of the confluence with Edson Arroyo.. COARMA06A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6b. Mainstem of the Saint Charles River from the confluence with Edson Arroyo to the confluence with the Arkansas River.. COARMA06B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Selenium(acute) = See selenium assessment location at 32.6(4).. *Selenium(chronic) = See selenium assessment location at 32.6(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = DM=32.6 and MWAT=WS-II from 3/1-11/30 DM=WS-II and MWAT=WS-II from 12/1-2/29..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; 173*.; 50*.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 7a. Mainstem of Greenhorn Creek, including all tributaries and wetlands, from the source to the San Isabel National Forest boundary, except for specific listings in segment 1. Mainstem of Graneros Creek, from the source to the San Isabel National Forest boundary, except for specific listings in segment 1. All tributaries to Muddy Creek, including wetlands, from the source to the San Isabel National Forest boundary.. COARMA07A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7b. Mainstem of Greenhorn Creek, including all tributaries and wetlands, from the San Isabel National Forest boundary to a point immediately below the Greenhorn Highline (Hayden Supply Ditch) diversion dam. Mainstem of Graneros Creek below the San Isabel National Forest boundary. Muddy Creek, including all tributaries and wetlands, from the San Isabel National Forest boundary to 232/Bondurant Road.. COARMA07B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. Deleted.. COARMA08.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... 9. Mainstem of Greenhorn Creek, from a point immediately below the Greenhorn Highline (Hayden Supply Ditch) diversion dam, to the confluence with the Saint Charles River.. COARMA09.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards Apply... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; 700.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10. Mainstem of Sixmile Creek from the source to the confluence with the Arkansas River.. COARMA10.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 11a. Mainstem of the Huerfano River including all tributaries and wetlands, from the source to 570 Road near Malachite, except for the specific listings in segment 1. Pass Creek, including all tributaries and wetlands, from the source to 565 Road. Muddy Creek, including all tributaries and wetlands, from the source to a point immediately below the confluence with Bruff Creek, except for the specific listings in segment 1. Mainstem of Turkey Creek (in Huerfano County) from the source to 620 Road, except for the specific listings in segment 1.. COARMA11A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11b. Mainstem of the Huerfano River, including all tributaries and wetlands, from 570 Road near Malachite to Highway 69 at Badito, except for the specific listings in segment 1, 11a and 17.. COARMA11B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12. Mainstem of Huerfano River from Highway 69 at Badito to the confluence with the Arkansas River.. COARMA12.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Water Supply.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13a. All tributaries, including wetlands, to the Cucharas River within the San Isabel National Forest boundaries, except for the specific listings in segment 1. Mainstem of the Cucharas River, from the source to a point immediately above the confluence with Middle Creek, except for the specific listings in segment 1. Wahatoya Creek, including all tributaries and wetlands, from the source to the confluence with the Cucharas River, except for the specific listings in segment 1. All tributaries to Middle Creek, including wetlands, from the source to a point immediately below the confluence of North and South Middle Creeks.. COARMA13A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 13b. Mainstem of the Cucharas River from a point immediately above the confluence with Middle Creek to the confluence with North Abeyta Creek (37.567852, -104.907046). All tributaries, including wetlands, to the Cucharas River from the San Isabel National Forest boundary to a point immediately below North Abeyta Creek (37.567852, -104.907046), except for specific listings in Segment 13a. Mainstem of Middle Creek, including all tributaries and wetlands, from a point immediately below the confluence of North and South Middle Creeks to the confluence with the Cucharas River, except for specific listings in 13a.. COARMA13B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13c. All tributaries and wetlands to the Cucharas and Huerfano Rivers not on Forest Service lands, except for waterbodies in segments 13a and 13b.. COARMA13C.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 10.; ---.. Nitrite.; 1.0.; ---.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.05...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 0.02-10.; A. Beryllium(T).; ---.; 4.0.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI(T).; 50.; 100.. Copper(T).; ---.; 200.. Iron.; ---.; WS.. Lead(T).; 50.; 100.. Manganese.; ---.; WS.. Mercury(T).; 2.0.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 100.. Nickel(T).; ---.; 100.. Selenium(T).; ---.; 20.. Silver(T).; ---.; 100.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... 14. Mainstem of the Cucharas River from the point of diversion for the Walsenburg public water supply to the outlet of Cucharas Reservoir.. COARMA14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Water Supply.. Recreation E... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15. Mainstem of the Cucharas River from the outlet of Cucharas Reservoir to the confluence with the Huerfano River.. COARMA15.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic; 340; ---. Arsenic(T).; ---.; 100.. Beryllium(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16. Deleted.. COARMA16.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... |
| --- | --- |

|  | 17. All tributaries to Apache Creek, including wetlands, from the source to a point immediately below the confluence of North and South Apache Creeks, except for the specific listings in segment 1. All tributaries, including wetlands, to the Huerfano River above the confluence with the Cucharas River that are within the San Isabel National Forest boundaries, except for the specific listings in segment 1 and 11a.. COARMA17.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 18a. Mainstem of Boggs Creek from the source to Pueblo Reservoir.. COARMA18A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 18b. Turkey Creek (Pueblo County) from U.S. Highway 50 to Pueblo Reservoir. Unnamed tributary to the Arkansas River, that flows from the south and whose confluence with the Arkansas River is located at 38.267623, -104.668298. Mainstem of Rush Creek (Pueblo County) from the source to the confluence with the Arkansas River.. COARMA18B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 19. All lakes and reservoirs tributary to the Arkansas River within the Sangre de Cristo, Greenhorn, and Spanish Peaks Wilderness areas.. COARMA19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 20. Pueblo Reservoir.. COARMA20.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *chlorophyll a (ug/L)(chronic) = See assessment location at 32.6(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = DM=CLL and MWAT=CLL from 1/1-3/31 DM= CLL and MWAT=23.6 from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; 5*.. chlorophyll a (ug/L); TVS. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 21. All lakes and reservoirs tributary to Chico Creek from the source to the confluence with the Arkansas River.. COARMA21.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 22. All lakes and reservoirs tributary to the Saint Charles River from the source to a point immediately above the CF&I diversion canal near Burnt Mill.. COARMA22.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 23. All lakes and reservoirs tributary to Greenhorn Creek from the source to a point immediately below the Greenhorn Highline (Hayden Supply Ditch) diversion dam, except for specific listings in segment 19. All lakes and reservoirs tributary to Graneros Creek from the source to the San Isabel National Forest boundary, except for specific listings in segment 19. All lakes and reservoirs tributary to Muddy Creek from the source to 232/Bondurant Road. Beckwith Reservoir.. COARMA23.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Beckwith Reservoir.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 24. All lakes and reservoirs tributary to the Huerfano River from the source to Highway 69 at Badito, except for the specific listings in segment 19. All lakes and reservoirs tributary to the Huerfano River above the confluence with the Cucharas River that are within the San Isabel National Forest boundaries, except for the specific listings in segment 19.. COARMA24.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 25. All lakes and reservoirs tributary to the Cucharas River from the source to the point of diversion for the Walsenburg public water supply, except for the specific listings in segment 19. Huajatolla Reservoirs and Diagre Reservoir.. COARMA25.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 26. Horseshoe Lake, Martin Lake (Ohem Lake) and Walsenburg Lower Town Lake.. COARMA26.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = Horseshoe DM=CLL and MWAT=CLL from 1/1-3/31, DM= CLL and MWAT=18.8 from 4/1-12/31. Martin DM=CLL and MWAT=CLL from 1/1-3/31, DM= CLL and MWAT=21.7 from 4/1-12/31. Walsenburg DM=CL and MWAT=CL..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 27. Deleted.. COARMA27.. Designation..; Classifications... Qualifiers:... Other:...; Physical and Biological.. DM.; MWAT... acute.; chronic... Inorganic (mg/L).. acute.; chronic...; Metals (ug/L).. acute.; chronic.... |
| --- | --- |

|  | 28. Valco Ponds and Runyon/Fountain Lake.. COARMA28.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1a. Mainstem of Fountain Creek, including all tributaries and wetlands, from the source to a point immediately above the confluence with Monument Creek, except for specific listings in segment 1b.. COARFO01A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 1b. Severy Creek, including all tributaries and wetlands, from the source to a point just upstream of where US Forest Service Road 330 crosses the stream.. COARFO01B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 2a. Mainstem of Fountain Creek from a point immediately above the confluence with Monument Creek to a point immediately above the State Highway 47 Bridge.. COARFO02A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2b. Mainstem of Fountain Creek from a point immediately above the State Highway 47 Bridge to the confluence with the Arkansas River.. COARFO02B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; 485.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 3300.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; 28.1.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3a. All tributaries to Fountain Creek which are within the boundaries of National Forest or Air Force Academy lands, including all wetlands, from a point immediately above the confluence with Monument Creek to the confluence with the Arkansas River, except for the mainstem of Monument Creek in the Air Force Academy lands and specific listings in segment 3b. Cheyenne Creek, including tributaries and wetlands from the source to the confluence with Fountain Creek. Bear Creek below Gold Camp Road to the confluence with Fountain Creek. Little Fountain Creek from the source to Highway 115. Rock Creek from the source to Highway 115. North Monument Creek from the source to the confluence with Monument Creek. Beaver Creek from the source to the confluence with Monument Creek.. COARFO03A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3b. Bear Creek, including all tributaries and wetlands, from the source to a point immediately upstream of Gold Camp Road.. COARFO03B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4a. Mainstems of Jackson Creek, Monument Branch, Elkhorn Springs, Pine Creek, South Pine Creek, South Rockrimmon Creek, Templeton Gap North, Templeton Gap Floodway, Douglas Creek and South Douglas Creek, from the sources to confluences with Monument Creek, including all tributaries and wetlands, which are not within the boundaries of the National Forest or Air Force Academy lands.. COARFO04A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4b. All tributaries to Monument Creek from the sources to the confluences with Monument Creek which are not within the boundaries of National Forest or Air Force Academy lands, including all wetlands, from a point immediately below the confluence with North Monument Creek to the confluence with Fountain Creek, except for specific listings in segments 3a, 4a and 4c. This includes Dirty Woman Creek, Smith Creek, Black Squirrel Creek, Cottonwood Creek, Dry Creek and an unnamed tributary with the confluence at Monument Creek located near (38.948613, -104.829623).. COARFO04B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4c. Mainstems of Kettle Creek, North Rockrimmon Creek and Mesa Creek, including tributaries and wetlands, from the sources to confluences with Monument Creek.. COARFO04C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4d. All tributaries to Fountain Creek from South Academy Blvd (CO83) to below the unnamed tributary immediately south of Old Pueblo Road (38.585843, -104.669591), including all wetlands, except for Little Fountain Creek and its tributaries and wetlands, and specific listings in segments 3a, 5a and 5b. All tributaries to Fountain Creek from a point immediately above University Blvd (CO47) (38.312846, -104.590524), to the confluence with the Arkansas River.. COARFO04D.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4e. All tributaries to Fountain Creek, including all wetlands, from a point immediately below the confluence with Monument Creek to University Blvd (CO47) near Pueblo except for specific listings in 3a, 4d, 5a and 5b.. COARFO04E.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5a. Jimmy Camp Creek, including all tributaries and wetlands, from the source to Old Pueblo Road (38.673200, -104.696739). Williams Creek, including all tributaries and wetlands, from the source to the confluence with Fountain Creek.. COARFO05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Water Supply.. Recreation E... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5b. Jimmy Camp Creek from Old Pueblo Road (38.673200, -104.696739) to the confluence with Fountain Creek, including the marshland located on the 60-acre parcel at 13030 Old Pueblo Road. Unnamed tributary from the boundary of Fort Carson (38.694465, -104.738735) to the confluence with Fountain Creek.. COARFO05B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6. Mainstem of Monument Creek, from the boundary of National Forest lands to the confluence with Fountain Creek.. COARFO06.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Copper(acute) = Copper BLM –based Fixed Monitoring Benchmark (FMB) Copper FMBa = 28.4µg/L for a subsegment of Monument Creek from immediately above the Tri-Lakes Wastewater Treatment Facility to the North Gate Boulevard Bridge.. *Copper(chronic) = Copper BLM –based Fixed Monitoring Benchmark (FMB) Copper FMBc = 17.8µg/L for a subsegment of Monument Creek from immediately above the Tri-Lakes Wastewater Treatment Facility to the North Gate Boulevard Bridge.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; ---.; TVS*.. Copper.; TVS*.; ---.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 7a. Pikeview Reservoir, Willow Springs Pond #1, and Willow Springs Pond #2.. COARFO07A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards Apply... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7b. Prospect Lake, Quail Lake, and Monument Lake.. COARFO07B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:.. Fish Ingestion Standards Apply... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. All lakes and reservoirs tributary to the mainstem of Fountain Creek from the source to a point immediately above the confluence with Monument Creek, except for specific listings in segment 9.. COARFO08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to Big Tooth Reservoir, Lake Moraine, and Woodmoor Lake.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9. North Catamount Reservoir, South Catamount Reservoir, and Crystal Creek Reservoir.. COARFO09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CLL.; CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10. All lakes and reservoirs tributary to Fountain Creek which are within the boundaries of National Forest or Air Force Academy lands from a point immediately above the confluence with Monument Creek to the confluence with the Arkansas River, except for specific listings in Segment 11. This segment includes Rampart Reservoir.. COARFO10.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Rampart Reservoir.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL,CLL.; CL,CLL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 11. AFA Non Potable Reservoir #1 (38.70939, -104.82928). All lakes and reservoirs tributary to Fountain Creek from a point immediately above the confluence with Monument Creek to the confluence with the Arkansas River, except for lakes and reservoirs within the boundaries of the National Forest, other lakes on Air Force Academy lands, and the waterbodies in segments 7a and 7b.. COARFO11.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Lower Reservoir, Keeton Reservoir, Unknown Reservoir at 38.70939, -104.82928, Gold Camp Reservoir, and South Suburban Reservoir.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1a. Mainstem of the Arkansas River from a point immediately above the confluence with Fountain Creek to immediately above the Colorado Canal headgate near Avondale.. COARLA01A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... Discharger Specific Variance(s):.. Selenium(ac/ch) = See Section 32.6(6) for details on the variance for the City of Pueblo.. Expiration Date of 12/31/2028. Sulfate(chronic) = See Section 32.6(6) for details on the variance for the City of Pueblo.. Expiration Date of 12/31/2028.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = DM=WS-II and MWAT=WS-II from 1/1-11/30 DM= 21.5 and MWAT=20.7 from 12/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; 329.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 2800.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; 19.1.; 14.1.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 1b. Mainstem of the Arkansas River from the Colorado Canal headgate to the inlet to John Martin Reservoir.. COARLA01B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards Apply... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; 902.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1950.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1c. Mainstem of the Arkansas River from the outlet of John Martin Reservoir to the Colorado/Kansas border.. COARLA01C.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards Apply... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; 1900.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/190.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2a. All tributaries to the Arkansas River, including wetlands, from the Colorado Canal headgate to the Colorado/Kansas border except for specific listings in segments 2b, 2c, 2d, 3a, through 9b, and Middle Arkansas Basin listings.. COARLA02A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Beryllium(T).; ---.; 4.0.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 2b. King Arroyo.. COARLA02B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:.. Livestock Watering Only... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 5.0.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 200.. Cadmium(T).; ---.; 50.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 1000.. Chromium VI(T).; ---.; 1000.. Copper(T).; ---.; 500.. Iron.; ---.; ---.. Lead(T).; ---.; 100.. Manganese.; ---.; ---.. Mercury(T).; ---.; 10.. Molybdenum(T).; ---.; 150.. Nickel.; ---.; ---.. Selenium(T).; ---.; 50.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 25000.... 2c. Mainstem of Wildhorse Creek, including all tributaries and wetlands, from a point immediately below US Highway 287 in Kit Carson to the confluence with Big Sandy Creek.. COARLA02C.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 100.. Beryllium(T).; ---.; 100.. Cadmium(T).; ---.; 50.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI(T).; ---.; 100.. Copper(T).; ---.; 200.. Iron.; ---.; ---.. Lead(T).; ---.; 100.. Manganese.; ---.; ---.. Mercury(T).; ---.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 200.. Selenium(T).; ---.; 50.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... |
| --- | --- |

|  | 2d. Unnamed tributary from the source north of county road 350 (37.304487, -104.29068) to the confluence with the Purgatoire.. COARLA02D.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 3a. Mainstem of the Apishapa River, including all tributaries and wetlands, from the source to I-25, except for specific listings in Middle Arkansas segment 1 and Lower Arkansas segments 3b and 3c.. COARLA03A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3b. Mainstem of West Torrino Canyon Creek, North Fork Trujillo Creek, Middle Fork Trujillo Creek and mainstem Trujillo Creek, Mitotes Canyon Creek, Luis Canyon Creek, Wheeler Canyon Creek, Mauricio Canyon Creek, Daisy Canyon Creek, Adobe Canyon Creek, Gonzales Canyon Creek, Frio Canyon Creek, Borrego Canyon Creek, Munoz Canyon Creek, William Canyon Creek and Castro Canyon Creek, including all tributaries and wetlands, from the sources to the confluences with the Apishapa River, except for the waterbodies in Middle Arkansas Segment 1.. COARLA03B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; 0.5.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 10.; ---.. Nitrite.; 1.0.; ---.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.05...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI(T).; 50.; ---.. Copper(T).; ---..; 200.. Iron.; ---.; WS.. Lead(T).; 50.; ---.. Manganese.; ---.; WS.. Mercury(T).; 2.0.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 100.. Selenium(T).; ---.; 20.. Silver(T).; 100.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... 3c. The mainstem of Jarosa Canyon Creek, including all tributaries and wetlands, from the source to the confluence with the Apishapa River.. COARLA03C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4a. Mainstem of the Apishapa River from I-25 to the confluence with the Arkansas River. Mainstem of Timpas Creek from the source to the Arkansas River.. COARLA04A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1805.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 4b. Mainstem of Lorencito Canyon, from the source to the confluence with the Purgatoire River.. COARLA04B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 4.0.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5a. Mainstem of the North Fork of the Purgatoire River, including all tributaries and wetlands, from the source to a point immediately below the confluence with Guajatoyah Creek. Mainstem of the Middle Fork of the Purgatoire River, including all tributaries and wetlands, from the source to the Bar Ni Ranch Road at Stonewall Gap. Mainstem of the South Fork of the Purgatoire River, including all tributaries and wetlands, from the source to Tercio.. COARLA05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 4.0.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5b. Mainstem of the North Fork of the Purgatoire River, including all tributaries and wetlands, from a point immediately below the confluence with Guajatoyah Creek to the confluence with the Purgatoire River. Mainstem of the Middle Fork of the Purgatoire River from the Bar Ni Ranch Road at Stonewall Gap to the confluence with the North Fork of the Purgatoire River. Mainstem of the South Fork of the Purgatoire River from Tercio to the confluence with the Purgatoire River. Mainstem of the Purgatoire River to Trinidad Lake. Mainstem of Long Canyon Creek from the source to Trinidad Reservoir.. COARLA05B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 4.0.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5c. Mainstem of the Purgatoire River from Trinidad Lake outlet to I-25. Mainstem of Raton Creek from the source to the confluence with the Purgatoire River.. COARLA05C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 2.0.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6a. All tributaries to the Purgatoire River, including all wetlands, from the source to I-25, except for specific listings in segments 4b, 5a, 5b, 5c and 6b.. COARLA06A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 32.5(4).. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 4.0.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6b.Wet Canyon and all tributaries, including wetlands, from the source to the confluence with the Purgatoire River.. COARLA06B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 2.0.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 0.02-10.; A. Beryllium(T).; ---.; 4.0.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7. Mainstem of the Purgatoire River from Interstate 25 to the confluence with the Arkansas River.. COARLA07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Water Supply.. Recreation E... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. Mainstem of Ricardo Creek, including all tributaries and wetlands, which are within Colorado (Costilla and Las Animas Counties). Mainstem of the Canadian River, including all tributaries and wetlands.. COARLA08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9a. Mainstems of Adobe, Buffalo, Cheyenne, Clay, Gageby, Horse, Two Butte, Wildhorse and Wolf Creeks from their sources to their confluences with the Arkansas River. Mainstems of Chacuacho Creek, San Francisco Creek, Trinchera Creek and Van Bremer Arroyo from their sources to their confluences with the Purgatoire River. Mainstem of Willow Creek from Highway 287 to the confluence with the Arkansas River. Mainstem of Big Sandy Creek from the source to the El Paso/Elbert county line. Mainstem of South Rush Creek from the source to the confluence with Rush Creek. Mainstem of Middle Rush Creek from the source to the confluence with North Rush Creek. North Rush Creek from the source to the confluence with South Rush Creek. Mainstem of Rush Creek to the Lincoln County Line. Mainstem of Antelope Creek from the source to the confluence with Rush Creek. The West May Valley drain from the Fort Lyon Canal to the confluence with the Arkansas River.. COARLA09A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 9b. Mainstem of Apache Creek from the source to the confluence with North Rush Creek. Mainstem of Breckenridge Creek from the source to the confluence with Horse Creek. Mainstem of Little Horse Creek from the source to the confluence with Horse Creek. Mainstem of Bob Creek from the source to Meredith Reservoir. Mainstem of Big Sandy Creek within Prowers County. Mainstem of Rule Creek from the Bent/Las Animas county line to John Martin Reservoir. Mainstem of Muddy Creek from the south boundary of the Setchfield State Wildlife Area to the confluence with Rule Creek. Mainstem of Caddoa Creek from CC Road to the confluence with the Arkansas River. Mainstem of Cat Creek from the source to the confluence with Clay Creek. Mainstem of Mustang Creek from the source to the confluence with Apishapa River. Mainstem of Chicosa Creek from the source to the Arkansas River. Mainstem of Smith Canyon from the Otero/Las Animas county line to the confluence with the Purgatoire River. Mainstem of Mud Creek from V Road to the confluence with the Arkansas River. Mainstems of Frijole Creek and Luning Arroyo from their sources to their confluences with the Purgatoire River. Mainstem of Blackwell Arroyo from its source to the confluence with Luning Arroyo. Mainstem of San Isidro Creek from the source to the confluence with San Francisco Creek.. COARLA09B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:.. Water + Fish Standards Apply... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 10. Two Buttes Reservoir, Two Buttes Pond, Lake Hasty, Holbrook Reservoir, Burchfield Lake, Nee-Skah (Queens) Reservoir, Adobe Creek Reservoir, Neeso Pah Reservoir, Nee Noshe Reservoir, Nee Gronda Reservoir.. COARLA10.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11. John Martin Reservoir.. COARLA11.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12. Lake Henry, Lake Meredith.. COARLA12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13. American Crystal Reservoir, Chancellor Ponds, Horse Creek Reservoir, Hugo Ponds, Jim Davis Pond, John Robertson Ponds, Karval Lake, Kinney Lake, Kissel Pond, La Junta Kids Pond, Las Animas Kids Pond, Mayhem Pond, Merit Lake, Olney Springs Pond, Otero Pond, Pursley Ponds, Ranch Reservoir, Reynolds Gravel Pit, Pyan Ponds, Thurston Reservoir, Turks Pond, Ramah Reservoir.. COARLA13.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14. All lakes and reservoirs tributary to the Apishapa River from the source to I-25, except for specific listings in Middle Arkansas segment 19.. COARLA14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15. All lakes and reservoirs tributary to the mainstem of the North Fork of the Purgatoire River from the source to a point immediately below the confluence with Guajatoyah Creek. All lakes and reservoirs tributary to the Middle Fork of the Purgatoire River from the source to the USGS gage at Stonewall. Mainstem of the South Fork of the Purgatoire River, from the source to Tercio. Monument Lake, North Lake, Trinidad Lake, Long Canyon Reservoir and Lake Dorothey.. COARLA15.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... *Classification: DUWS applies to Monument Lake and North Lake.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details.. *Temperature = Trinidad Reservoir (CLL)..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL.. Temperature °C.; CLL*.; CLL *... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16. All lakes and reservoirs tributary to the Purgatoire River from the source to I-25, except for the specific listings in segment 15 and 17.. COARLA16.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 100.. Beryllium(T).; ---.; 100.. Cadmium(T).; ---.; 10.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI(T).; ---.; 100.. Copper(T).; ---.; 200.. Iron.; ---.; ---.. Lead(T).; ---.; 100.. Manganese.; ---.; ---.. Mercury(T).; ---.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 200.. Selenium(T).; ---.; 20.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... |
| --- | --- |

|  | 17. All lakes and reservoirs tributary to Wet Canyon, from the source to the confluence with the Purgatoire River.. COARLA17.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.05...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 0.02-10.; A. Beryllium(T).; ---.; 4.0.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI(T).; 50.; 100.. Copper(T).; ---.; 200.. Iron.; ---.; WS.. Lead(T).; 50.; 100.. Manganese.; ---.; WS.. Mercury(T).; 2.0.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 100.. Nickel(T).; ---.; 100.. Selenium(T).; ---.; 20.. Silver(T).; 100.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... 18. All lakes and reservoirs tributary to Ricardo Creek in Colorado (Costilla and Las Animas Counties). All lakes and reservoirs tributary to the Canadian River.. COARLA18.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 19. All lakes and reservoirs tributary to the Arkansas River, except for specific listings in segments 10-18 and Middle Arkansas Basin segments 19-28.. COARLA19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 1. Mainstem of the Cimarron River, including all tributaries and wetlands, in Las Animas, Baca, and Prowers Counties, except for the waterbodies in segment 2.. COARCI01.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation N... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; ---.. E. Coli (per 100 mL).; ---.; 630... Inorganic (mg/L).. acute.; chronic.. Ammonia.; ---.; ---.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; ---.; ---.. Cyanide.; 0.2.; ---.. Nitrate.; 100.; ---.. Nitrite.; 10.; ---.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; ---...; Metals (ug/L).. acute.; chronic.. Arsenic(T).; ---.; 100.. Beryllium(T).; ---.; 100.. Cadmium(T).; ---.; 10.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI(T).; ---.; 100.. Copper(T).; ---.; 200.. Iron.; ---.; ---.. Lead(T).; ---.; 100.. Manganese.; ---.; ---.. Mercury(T).; ---.; ---.. Molybdenum(T).; ---.; 150.. Nickel(T).; ---.; 200.. Selenium(T).; ---.; 20.. Silver.; ---.; ---.. Uranium.; varies*.; varies*.. Zinc(T).; ---.; 2000.... 2. Mainstem of North Carrizo Creek from the source to the Colorado/Oklahoma state line. Mainstem of East Carrizo Creek and West Carrizo Creek to the confluence with North Carrizo Creek. Mainstem of Cottonwood Creek and Tecolote Creek to the confluence with West Carrizo Creek.. COARCI02.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 3. All lakes and reservoirs tributary to the Cimarron River.. COARCI03.. Designation.. UP..; Classifications.. Recreation E.. Agriculture.. Aq Life Warm 2... Qualifiers:.. Fish Ingestion Standards Apply... Other:... *Uranium(acute) = See 32.5(3) for details.. *Uranium(chronic) = See 32.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. Coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

STREAM CLASSIFICATIONS and WATER QUALITY STANDARDS – FOOTNOTES

(A)	Whenever a range of standards is listed and referenced to this footnote, the first number in the range is a strictly health-based value, based on the Commission’s established methodology for human health-based standards. The second number in the range is a maximum contaminant level, established under the federal Safe Drinking Water Act that has been determined to be an acceptable level of this chemical in public water supplies, taking treatability and laboratory detection limits into account. Control requirements, such as discharge permit effluent limitations, shall be established using the first number in the range as the ambient water quality target, provided that no effluent limitation shall require an “end-of-pipe” discharge level more restrictive than the second number in the range. Water bodies will be considered in attainment of this standard, and not included on the Section 303(d) List, so long as the existing ambient quality does not exceed the second number in the range.

(B)	Reserved.

(C)	Reserved.