colorado department of public health and environment

WATER QUALITY CONTROL COMMISSION

5 CCR 1002-38

regulation no. 38
classifications and numeric standards
for
south platte river basin, laramie river basin
republican river basin, smoky hill river basin

APPENDIX 38-1

Stream Classifications and Water Quality Standards Tables

Effective 12/31/2026

### Abbreviations and Acronyms

Aq	=	Aquatic
°C	=	degrees Celsius
CL	=	cold lake temperature tier
CLL	=	cold large lake temperature tier
CS-I	=	cold stream temperature tier one
CS-II	=	cold stream temperature tier two
D.O. 	=	dissolved oxygen
DM	=	daily maximum temperature
DUWS	=	direct use water supply
E. coli 	=	Escherichia coli
EQ	=	existing quality
mg/L 	=	milligrams per liter
mg/m2	=	milligrams per square meter
mL	=	milliliter
MWAT	=	maximum weekly average temperature
OW 	=	outstanding waters
SSE	=	site-specific equation
T	=	total recoverable
t	=	total
tr	=	trout
TVS	=	table value standard
µg/L	=	micrograms per liter
UP	=	use-protected
WS	=	water supply
WS-I	=	warm stream temperature tier one
WS-II	=	warm stream temperature tier two
WS-III	=	warm stream temperature tier three
WL	=	warm lake temperature tier

|  | 1a. Mainstem of the South Platte River from the source of the South and Middle Forks to the inlet of Cheesman Reservoir.. COSPUS01A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *Temperature = summer criteria apply from 4/1-10/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I*.; CS-I*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 1b. All tributaries to the South Platte River, including wetlands, within the Lost Creek and Mt. Evans Wilderness Areas.. COSPUS01B.. Designation.. OW..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 2a. All tributaries to the South Platte River system, including wetlands, from the headwaters of the South and Middle Forks to a point immediately below the confluence with Tarryall Creek except for listings in Segment 1b, 2b, and 2c.. COSPUS02A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 2b. Mainstem of Mosquito Creek from Road #698 (39.270971, -106.098846) to its confluence with the Middle Fork of the South Platte River.. COSPUS02B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; 220.... |
| --- | --- |

|  | 2c. South Mosquito Creek from the source to confluence with Mosquito Creek, Mosquito Creek from the confluence with South Mosquito Creek to Road #698 (39.270971, -106.098846), and No Name Creek from the source to the confluence with South Mosquito Creek.. COSPUS02C.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; ---.; 280.... 3. All tributaries to the South Platte River, including wetlands, from a point immediately below the confluence with Tarryall Creek to a point immediately above the confluence with the North Fork of the South Platte River, except for listings in Segment 1b.. COSPUS03.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 4. Mainstem of the North Fork of the South Platte River, including all tributaries and wetlands, from the source to the confluence with the South Platte River, except for listings in Segments 1b, 5a, 5b, 5c, and 5d.. COSPUS04.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5a. Mainstem of Geneva Creek from the source to the confluence with Scott Gomer Creek.. COSPUS05A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 3.5-9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; ---.; ---.. Cadmium(T).; ---.; 2.. Chromium III.; ---.; ---.. Chromium III(T).; ---.; 100.. Chromium VI.; ---.; ---.. Chromium VI(T).; ---.; 25.. Copper.; ---.; 18.. Iron(T).; ---.; 1200.. Lead.; ---.; ---.. Lead(T).; ---.; 4.. Manganese.; ---.; 530.. Mercury(T).; ---.; 0.05.. Molybdenum(T).; ---.; 150.. Nickel.; ---.; ---.. Nickel(T).; ---.; 50.. Selenium.; ---.; ---.. Selenium(T).; ---.; 4.6.. Silver.; ---.; ---.. Silver(T).; ---.; 1.. Uranium.; varies*.; varies*.. Zinc.; ---.; 190.... |
| --- | --- |

|  | 5b. Mainstem of Geneva Creek from the confluence with Scott Gomer Creek to the confluence with the North Fork of the South Platte River. All tributaries to Geneva Creek, including wetlands, from source to confluence with the North Fork of the South Platte River.. COSPUS05B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 5c. Mainstem of Gooseberry Gulch, including all tributaries and wetlands, from the source to Sunset Trail.. COSPUS05C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation U.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 5d. Mainstem of Gooseberry Gulch, including all tributaries and wetlands, from Sunset Trail to the confluence with Elk Creek.. COSPUS05D.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation U.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 6a. Mainstem of the South Platte River from the outlet of Cheesman Reservoir to the inlet of Chatfield Reservoir.. COSPUS06A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 6b. Chatfield Reservoir. COSPUS06B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029. *chlorophyll a (ug/L)(chronic) = measured through samples that are representative of the mixed layer during July-Sept, with an allowable exceedance frequency of 1in 5 yrs. See section 38.6(4) for assessment thresholds.. *Phosphorus(chronic) = See section 38.6(4) for assessment thresholds.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *Temperature = DM=CLL and MWAT=CLL from 1/1-3/31 DM=CLL and MWAT=23.5 from 4/1-12/31..; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; 7/1 - 9/30.; ---.; 10*.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; 0.03*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 7. All tributaries to the South Platte River, including wetlands, from a point immediately below the confluence with the North Fork of the South Platte River to the outlet of Chatfield Reservoir, except for listings in Segments 8, 9, 10, 11, 12, and 13.. COSPUS07.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 8. Mainstems of East and West Plum Creek from the source to the boundary of National Forest lands, including all tributaries and wetlands within the Plum Creek drainage which are on National Forest Lands, except for the listing in Segment 9.. COSPUS08.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 9. Mainstem of Bear Creek, including all tributaries and wetlands, from the source to the inlet of Perry Park Reservoir, a.k.a. Waucondah Reservoir (Douglas County).. COSPUS09.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-I.; CS-I... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 10. Mainstems of East Plum Creek, West Plum Creek, and Plum Creek from the boundary of National Forest lands to Chatfield Reservoir, mainstems of Stark Creek and Gove Creek from the boundary of National Forest lands to their confluence.. COSPUS10.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 11a. All tributaries to the East Plum Creek system, including wetlands, which are not on national forest lands.. COSPUS11A.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 11b. All tributaries to the West Plum Creek system, including all wetlands, which are not on national forest lands, except for listings in Segments 9 and 12.. COSPUS11B.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Water Supply.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 12. Mainstem of Garber Creek and Jackson Creek from the boundary of National Forest lands to the confluence with West Plum Creek; mainstem of Bear Creek from the outlet of Perry Park Reservoir, a.k.a. Waucondah Reservoir, to the confluence with West Plum Creek.. COSPUS12.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 13. Mainstem of Deer Creek, including the North and South Forks, from the source to Chatfield Reservoir.. COSPUS13.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CS-II.; CS-II... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 14. Mainstem of the South Platte River from the outlet of Chatfield Reservoir to the Burlington Ditch diversion in Denver, Colorado.. COSPUS14.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Copper(acute) = Copper BLM-based FMB Cu FMB(ac)=31.5 ug/l applies downstream of Marcy Gulch. TVS applies to the rest of the segment.. *Copper(chronic) = Copper BLM-based FMB Cu FMB(ch)=20.8 ug/l applies downstream of Marcy Gulch. TVS applies to the rest of the segment.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *Temperature = summer criteria apply from 2/14 - 11/30..; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I*.; WS-I*... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; ---.; TVS*.. Copper.; TVS*.; ---.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/190.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 15. Mainstem of the South Platte River from the Burlington Ditch diversion in Denver, Colorado, to a point immediately below the confluence with Big Dry Creek.. COSPUS15.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Ammonia(acute) = See section 38.6(4) for site-specific standards.. *Ammonia(chronic) = See section 38.6(4) for site-specific standards.. *Copper(acute) = Copper BLM-based FMB Cu FMB(ac)=26.4 ug/l applies downstream of the Metro Hite WWTF outfall. TVS applies to the rest of the segment.. *Copper(chronic) = Copper BLM-based FMB Cu FMB(ch)=18.0 ug/l applies downstream of the Metro Hite WWTF outfall. TVS applies to the rest of the segment.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *D.O. (mg/L)(acute) = See section 38.6(4) for site-specific standards.. *D.O. (mg/L)(chronic) = See section 38.6(4) for site-specific standards.. *pH(acute) = 6.0 - 9.0 from 64th Ave. downstream 2 miles..; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-I.; WS-I... acute.; chronic.. D.O. (mg/L).; varies*.; varies*.. pH.; 6.0-9.0*.; ---.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS*.; TVS*.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; 1.0.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; ---.; TVS*.. Copper.; TVS*.; ---.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/400.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16a. Mainstem of Sand Creek from the confluence of Murphy Creek and Coal Creek in Arapahoe County to the confluence with Toll Gate Creek.. COSPUS16A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Water Supply.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 16b. Aurora Reservoir.. COSPUS16B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E.. Water Supply.. DUWS... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16c. All tributaries to the South Platte River, including all wetlands, from the outlet of Chatfield Reservoir, to a point immediately below the confluence with Big Dry Creek, except for listings in the subbasins of the South Platte River, and in Segments 16a, 16d, 16e, 16f, 16g, 16h, 16i, 16j, and 16k.. COSPUS16C.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 16d. Second Creek, including all tributaries, from the source to the O’Brian Canal at 39.898789, 104.817661.. COSPUS16D.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *D.O. (mg/L)(chronic) = 15th percentile of D.O. measurements collected between 6:30 a.m. and 6:30 p.m...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 3.3*.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16e. Third Creek, including all tributaries, from the source to the O’Brian Canal at 39.917346, -104.784028.. COSPUS16E.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Water Supply.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *D.O. (mg/L)(chronic) = 15th percentile of D.O. measurements collected between 6:30 a.m. and 6:30 p.m...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; 4.0*.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 16f. Barr Lake Tributary, including all tributaries, from the source to the Denver Hudson Canal at 39.941142, -104.748387.. COSPUS16F.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *D.O. (mg/L)(chronic) = When water is present, D.O. concentrations shall be maintained at levels that protect classified uses...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-III.; WS-III... acute.; chronic.. D.O. (mg/L).; ---.; narrative*.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16g. Marcy Gulch, including all wetlands from the source to the confluence with the South Platte.. COSPUS16G.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Copper(acute) = Copper BLM-based FMB Cu FMB(ac)=67.1 ug/l applies below the Marcy Gulch WWTF. TVS applies to the rest of the segment.. *Copper(chronic) = Copper BLM-based FMB Cu FMB(ch)=43.3 ug/l applies below the Marcy Gulch WWTF. TVS applies to the rest of the segment.. *Selenium(acute) = See section 38.6(4)(b) for assessment locations.. *Selenium(chronic) = See section 38.6(4)(b) for assessment locations.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; ---.; TVS*.. Copper.; TVS*.; ---.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; 21*.; 13*.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 16h. Mainstem of West Toll Gate Creek, including all tributaries and wetlands, upstream of the confluence with East Toll Gate Creek. Mainstem of East Toll Gate Creek, including all tributaries and wetlands, upstream of the confluence with West Toll Gate Creek. Mainstem of Toll Gate Creek, downstream of the confluence of East and West Toll Gate Creeks, to the confluence with Sand Creek.. COSPUS16H.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:.. Fish Ingestion Standards... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Selenium(acute) = See section 38.6(4)(b) for selenium standards and assessment locations.. *Selenium(chronic) = See section 38.6(4)(b) for selenium standards and assessment locations.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; varies*.; varies*.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16i. Mainstem of Sand Creek from the confluence with Toll Gate Creek to the confluence with the South Platte River.. COSPUS16I.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Mercury(T)(chronic) = 0.026 below Brighton Blvd, see section 38.6(4)(f) for mercury assessment locations. *Selenium(acute) = See section 38.6(4)(f) for selenium standards and assessment locations.. *Selenium(chronic) = See section 38.6(4)(f) for selenium standards and assessment locations.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.026*.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; ---.; varies*.. Selenium.; varies*.; ---.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 16j. Lee Gulch, Little’s Creek, Big Dry Creek (Douglas and Arapahoe Counties), and Little Dry Creek, including all wetlands from the source to the confluence with the South Platte.. COSPUS16J.. Designation.. UP..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Selenium(acute) = See section 38.6(4)(h) for selenium standards and assessment locations.. *Selenium(chronic) = See section 38.6(4)(h) for selenium standards and assessment locations.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; varies*.; varies*.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 16k. Mainstem of Lakewood Gulch from the source to the confluence with the South Platte.. COSPUS16K.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Water Supply.. Recreation E... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WS-II.; WS-II... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (mg/m2).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 17a. Washington Park Lakes, City Park Lakes, Rocky Mountain Lake, and Berkeley Lake.. COSPUS17A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 17b. Sloan’s Lake.. COSPUS17B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 17c. Bowles Lake, a.k.a. Patrick Reservoir or Bow Mar Lake.. COSPUS17C.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Aluminum(T).; TVS.; TVS.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 18. Lakes and reservoirs within the boundaries of the Lost Creek and Mt. Evans Wilderness areas.. COSPUS18.. Designation.. OW..; Classifications.. Water Supply.. Agriculture.. Aq Life Cold 1.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS.. Phosphorus.; ---.; TVS.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 19. Lakes and reservoirs in the South Platte River system from headwaters to Chatfield Reservoir, except for listings in Segment 18. Includes Antero, Spinney Mountain, Elevenmile, Cheesman, and Strontia Springs.. COSPUS19.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply.. DUWS*... Qualifiers:... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to Strontia Springs Reservoir and Woodland Park Reservoir.. *Nitrogen(chronic) = applies only above the facilities listed at 38.5(4).. *Phosphorus(chronic) = applies only above the facilities listed at 38.5(4).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details.. *Temperature = See 38.6(4) for temperature standards...; Physical and Biological.. DM.; MWAT.. Temperature °C.; varies*.; varies*... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Nitrogen.; ---.; TVS*.. Phosphorus.; ---.; TVS*.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 20. Lakes and reservoirs in the Plum Creek system within National Forest boundaries; and lakes and reservoirs in the Bear Creek drainage between the National Forest boundary and to the inlet of Perry Park Reservoir, a.k.a. Waucondah Reservoir (Douglas County).. COSPUS20.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Cold 1.. Recreation E.. Water Supply... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; CL.; CL... acute.; chronic.. D.O. (mg/L).; ---.; 6.0.. D.O. (spawning).; ---.; 7.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.05.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS(tr).. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 21. Lakes and reservoirs in the Plum Creek system except for listings in Segment 20.. COSPUS21.. Designation.. Reviewable..; Classifications.. DUWS*.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply... Qualifiers:... Other:... *Classification: DUWS applies to Aurora Rampart Reservoir.. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02-10.; A. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 22a. Lakes and reservoirs in watersheds tributary to the South Platte River from the outlet of Chatfield Reservoir to a point immediately below the confluence with Big Dry Creek, except for listings in the subbasins of the South Platte River, and in Segments 16b, 17a, 17b, 17c, 22b, and 23.. COSPUS22A.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E.. Water Supply.. DUWS*... Qualifiers:.. Water + Fish Standards... Other:... Temporary Modification(s):.. Arsenic(chronic) = hybrid. Expiration Date of 12/31/2029.. *Classification: DUWS applies to McLellan Reservoir, Quincy Reservoir, and Marshall Reservoir.. *Molybdenum(T)(chronic) = 210 ug/L for McLellan Reservoir. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; DUWS.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; 250.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 10.; ---.. Nitrite.; ---.; 0.5.. Nitrogen.; ---.; ---.. Phosphorus.; ---.; ---.. Sulfate.; ---.; WS.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 0.02.. Cadmium.; TVS.; TVS.. Cadmium(T).; 5.0.; ---.. Chromium III.; ---.; TVS.. Chromium III(T).; 50.; ---.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron.; ---.; WS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Lead(T).; 50.; ---.. Manganese.; TVS.; TVS/WS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Molybdenum(T).; ---.; 210*.. Nickel.; TVS.; TVS.. Nickel(T).; ---.; 100.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |

|  | 22b. Lakes and reservoirs located in the Rocky Mountain Arsenal National Wildlife Refuge. COSPUS22B.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:... Other:... *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 100.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... 23. Lakes and reservoirs in watersheds tributary to the Upper South Platte River and within the City and County of Denver, except for listings in the other subbasins of the South Platte River and in Segments 17a and 17b.. COSPUS23.. Designation.. Reviewable..; Classifications.. Agriculture.. Aq Life Warm 2.. Recreation E... Qualifiers:.. Fish Ingestion Standards... Other:... *See section 38.7 (Marston Forebay).. *Uranium(acute) = See 38.5(3) for details.. *Uranium(chronic) = See 38.5(3) for details...; Physical and Biological.. DM.; MWAT.. Temperature °C.; WL.; WL... acute.; chronic.. D.O. (mg/L).; ---.; 5.0.. pH.; 6.5 - 9.0.; ---.. chlorophyll a (ug/L).; ---.; TVS.. E. coli (per 100 mL).; ---.; 126... Inorganic (mg/L).. acute.; chronic.. Ammonia.; TVS.; TVS.. Boron.; ---.; 0.75.. Chloride.; ---.; ---.. Chlorine.; 0.019.; 0.011.. Cyanide.; 0.005.; ---.. Nitrate.; 100.; ---.. Nitrite.; ---.; 0.5.. Phosphorus.; ---.; ---.. Sulfate.; ---.; ---.. Sulfide.; ---.; 0.002...; Metals (ug/L).. acute.; chronic.. Arsenic.; 340.; ---.. Arsenic(T).; ---.; 7.6.. Cadmium.; TVS.; TVS.. Chromium III.; TVS.; TVS.. Chromium III(T).; ---.; 100.. Chromium VI.; TVS.; TVS.. Copper.; TVS.; TVS.. Iron(T).; ---.; 1000.. Lead.; TVS.; TVS.. Manganese.; TVS.; TVS.. Mercury(T).; ---.; 0.01.. Molybdenum(T).; ---.; 150.. Nickel.; TVS.; TVS.. Selenium.; TVS.; TVS.. Silver.; TVS.; TVS.. Uranium.; varies*.; varies*.. Zinc.; TVS.; TVS.... |
| --- | --- |