COLORADO DEPARTMENT OF REGULATORY AGENCIES

Public Utilities Commission

4 Code of Colorado Regulations (CCR) 723-3

PART 3

# RULES REGULATING ELECTRIC UTILITIES

## BASIS, PURPOSE, AND STATUTORY AUTHORITY.

The basis and purpose of these rules is to describe the electric service to be provided by jurisdictional utilities and master meter operators to their customers; to designate the manner of regulation over such utilities and master meter operators; and to describe the services these utilities and master meter operators shall provide.  In addition, these rules identify the specific provisions applicable to public utilities or other persons over which the Commission has limited jurisdiction.  These rules address a wide variety of subject areas including, but not limited to, service interruption, meter testing and accuracy, safety, customer information, customer deposits, rate schedules and tariffs, discontinuance of service, master meter operations, flexible regulation, procedures for administering the Low-Income Energy Assistance Act, electric service low-income program, cost allocation between regulated and unregulated operations, recovery of costs, the acquisition of renewable energy, small power producers and cogeneration facilities, and appeals regarding local government land use decisions.  The statutory authority for these rules can be found at §§ 29-20-108, 40-1-103.5, 40-2-108, 40-2-124(2), 40-2-126, 40-2-202, 40-2-203, 40-3-102, 40-3-102.5, 40-3-103, 40-3-104.3, 40-3-106, 40-3-111, 40-3-114, 40-4-101, 40-4-106, 40-4-108, 40-4-109, 40-5-103, 40-7-113.5, 40-7-116.5, 40-8.7-105(5), and 40-9.5-107(5), C.R.S.

## GENERAL PROVISIONS

### 3000.	Scope and Applicability.

(a)	Absent a specific statute, rule, or Commission Order which provides otherwise, all rules in this Part 3 (the 3000 series) shall apply to all jurisdictional electric utilities and electric master meter operators and to Commission proceedings concerning electric utilities or electric master meter operators providing electric service.

(b)	The following rules in this Part 3 shall apply to cooperative electric associations which have elected to exempt themselves from the Public Utilities Law pursuant to § 40-9.5-103, C.R.S.

(I)	Paragraphs 3002 (a)(I), (a)(II), (a)(IV), (a)(V), (a)(XVI), (b), and (c) concerning the filing of applications for certificate of public convenience and necessity for franchise or service territory, for certificate amendments, to merge or transfer, or for appeals of local government actions.

(II)	Paragraphs 3005 (a)(III), (IV), (d), (e), (g), and (h) concerning records under RUS accounting system and preservation of records.

(III)	Paragraphs 3006 (a), (b), (c), (d), and (e) concerning the filing of annual reports, designation for service of process, and election of applicability of Title 40, Article 8.5.

(IV)	Paragraphs 3008 (b) and (d) concerning incorporation by reference.

(V)	Rules 3100 and 3103 concerning application for and amendment of a certificate of public convenience and necessity relating to a franchise.

(VI)	Rules 3101 and 3103 concerning application for and amendment of a certificate of public convenience and necessity relating to service territory.

(VII)	Rule 3104 concerning application to transfer assets, to obtain a controlling interest, or to merge with another entity.

(VIII)	Rule 3204 concerning incidents occurring in connection with the operation of facilities.

(IX)	Paragraphs 3207 (a) and (b) concerning construction and expansion of distribution facilities.

(X)	Rules 3250 through 3253 concerning major event reporting.

(XI)	Rule 3411 concerning the Low-Income Energy Assistance Act unless the cooperative electric association has exempted themselves pursuant to paragraph 3411(a).

(XII)	Rules 3650(b), 3651, 3652, 3654(b), (d) through (i), and (l); 3655(h)-(m); 3659(a)(I) through (a)(V), (b) and (d) through (i), 3660(l), 3661(b), (c), (g), and (i), 3662(a)(I), (a)(II), (a)(IV) through (a)(X), (a)(XIII), (a)(XV), (b), (d) and (e), and 3667.

(XIII)	Rules 3700 through 3707 concerning appeals of local governmental land use decisions actions.

(XIV)	Rules 3850 through 3859 concerning interconnection procedures and standards.

*           *           *           *

[indicates omission of unaffected rules]

## INTERCONNECTION PROCEDURES AND STANDARDS.

### 3850.	Applicability.

The following interconnection procedures shall apply to the interconnection of all retail renewable distributed generation and other distributed energy resources including energy storage systems that operate in parallel with and are connected to the utility, when such interconnections are not subject to the jurisdiction of FERC.  This rule largely tracks the 2013 FERC amended version of the FERC 2006 Small Generator Interconnection Procedures.

### 3851.	Overview and Purpose.

Infrastructure security of electric system equipment and operations and control hardware and software is essential to ensure day-to-day reliability and operational security.  The Commission expects all utilities, market participants, and Interconnection Customers interconnected with electric systems to comply with the recommendations offered by the President's Critical Infrastructure Protection Board and best practice recommendations from the electric reliability authority.  All utilities are expected to meet basic standards for electric system infrastructure and operational security, including physical, operational, and cyber-security practices.

The purpose of these rules is to establish reasonable interconnection procedures and insurance requirements for all utilities to adhere to when interconnecting retail renewable distributed generation, and other distributed energy resources that connect to a utility’s system that operate in parallel with and are connected to the utility.

### 3852.	Definitions.

The following definitions apply only to rules 3850 to 3859.

(a)	“Business day” means Monday through Friday, excluding federal holidays.

(b)	“Distributed energy resource” or “DER” means the interconnection customer's source of electric power connected to the utility’s distribution grid, including retail renewable distributed generation, other small generation facilities for the production of electricity, energy storage systems, or combination of any of these elements, as identified in the interconnection request, but shall not include the interconnection facilities not owned by the interconnection customer.  DER includes an interconnection system or a supplemental DER device that is necessary for compliance with IEEE 1547-2018, until January 1, 2022, or until such time new DERs applying for interconnection will comply with IEEE 1547 2018.  This rule does not include any later amendments or editions of this standard.  This standard is available for public inspection at the Commission’s office, 1560 Broadway, Suite 250, Denver, CO 80202.

(c)	“Distribution system” means the utility's facilities and equipment used to transmit electricity to ultimate usage points such as homes and industries directly from interconnection resources or from interchanges with higher voltage transmission networks which transport bulk power over longer distances.  The voltage levels at which distribution systems operate differ among areas.

(d)	“Energy storage system" means any commercially available, customer-sited system or utility-sited system, including batteries and batteries paired with on-site generation, that does not generate energy, that is capable of retaining, storing, and delivering electrical energy by chemical, thermal, mechanical, or other means.

(e)	“Export capacity” means the amount of alternating current (AC) electrical energy that an interconnection resource is intended to transfer to the utility’s system across the point of interconnection.

(f)	“Highly seasonal circuit” means a circuit with a ratio of annual peak load to off-season peak load greater than six.

(g)	“Inadvertent export” means the potential condition in which a normally non-exporting or limited-exporting DER experiences a momentary export that does not exceed limitations specified in paragraph 3853(c).

(h)	“Interconnection agreement” means a contract between the interconnection customer and the utility that formally documents terms and conditions related to the operation and maintenance of any DER in accordance with the utility’s tariffs on file with the Commission.

(i)	“Interconnection customer” or “IC” means any entity, including the utility, any affiliates or subsidiaries of either, that proposes to interconnect its DER with the utility's system.

(j)	“Interconnection facilities” means the utility's interconnection facilities and the interconnection customer's interconnection facilities.  Collectively, interconnection facilities include all facilities and equipment between the DER and the point of interconnection, including any modification, additions or upgrades that are necessary to physically and electrically interconnect the DER to the utility's system.  Interconnection facilities are sole use facilities and shall not include distribution upgrades.

(k)	“Interconnection request” means the interconnection customer's request, in accordance with any applicable utility tariff, to interconnect a new small generating facility, or to increase the capacity of, or make a material modification to the operating characteristics of, an existing DER that is interconnected with the utility’s system.

(l)	“Interconnection resource” means the interconnection customer's source of electric power connected to the utility’s distribution grid, including retail renewable distributed generation, other small generation facilities for the production of electricity, energy storage systems, bidirectional storage, electric vehicle chargers with vehicle to grid, vehicle to home, vehicle to building or combination of any of these elements, as identified in the interconnection request, but shall not include the interconnection facilities not owned by the interconnection customer.  “Interconnection resource” includes an interconnection system or a supplemental DER device that is necessary for compliance with IEEE Standard 1547-2018, until January 1, 2022, or until such time new DERs applying for interconnection will comply with IEEE 1547-2018.  This rule does not include any later amendments or editions of this standard.  This standard is available for public inspection at the Commission’s office, 1560 Broadway, Suite 250, Denver, CO 80202.

(m)	“Interconnection tariffs” are required filings from the utilities that set forth fees associated with interconnection.  Tariff filings would accommodate utility-specific costs, while allowing for appropriate statewide standardization in the provisions set forth.

(n)	“Line section” means that portion of the utility’s electric delivery system that is connected to a Customer and bounded by automatic sectionalizing devices or the end of the distribution line.

(o)	“Material modification” means a modification that has a material impact on the cost or timing of processing an application with a later queue priority date or a change in the point of interconnection.  A material modification does not include, for example:  a change of ownership of an interconnection resource; changes to the address of the generating facility, so long as the generating facility remains on the same parcel; a change or replacement of interconnection resource that is a like-kind substitution in size, ratings, impedances, efficiencies, or capabilities of the equipment specified in the original application; or a reduction in the capacity of the interconnection resource of ten percent or less.

(p)	“Meter collar adapter” or “meter socket adapter” means a customer-owned device that is installed between the electric meter and the meter socket box on a utility customer’s premises and that has electrical connection points both electrically upstream and electrically downstream of the meter.

(q)	“Minor modifications” means modifications to the utility’s distribution system or to the interconnection facilities that do not have a material impact on the cost or on the timing of an interconnection request.

(r)	“Non-exporting system” means an interconnection resource that is designed so that it does not intentionally transfer electrical energy to the utility’s distribution or transmission system across the point of common coupling.  Such systems may be used to supply part or all of a customer’s load continuously or during an outage.  A system can be non-exporting by virtue of inverter programing or by some other on-site limiting element.  Non-exporting systems may or may not produce inadvertent exports as defined in paragraph (g) of this rule.

(s)	“Operating mode” means the mode of DER operational characteristics that determines the performance during normal and abnormal conditions.  For example, an operating mode such as “export only,” “import only,” and “no exchange.”

(t)	“Parallel operation” means a DER facility that is connected to the utility’s system and can supply AC electricity to the interconnection customer simultaneously with the utility’s supply of AC electricity.

(u)	“Party” or “Parties” means the utility, interconnection customer, or any combination thereof.

(v)	“Point of interconnection” means the point where the interconnection facilities connect with the utility's system.

(w)	“Study process” means the procedure for evaluating an interconnection request that includes the Level 3 scoping meeting, feasibility study, system impact study, and facilities study.

(x)	“System upgrades” means the additions, modifications, and upgrades to the utility's distribution or Commission-jurisdictional transmission system at or beyond the point of interconnection to facilitate interconnection of interconnection resources and render the service necessary to effect the interconnection customer's operation of interconnection resources.  System upgrades do not include interconnection facilities.

(y)	“Transmission system” means an interconnected group of transmission lines and associated equipment for the movement or transfer of electric energy between points of supply and points at which it is transformed for delivery to customers or is delivered to other electric systems.

(z)	“Utility system” means the facilities owned, controlled, or operated by the utility that are used to provide electric service under the tariff.

(aa)	“Upgrades” means the additions and modifications to the utility's system at or beyond the point of interconnection that are necessary to interconnect an interconnection resource.  Upgrades do not include interconnection facilities.

### 3853.	General Interconnection Procedures.

*           *           *           *

[indicates omission of unaffected rules]

(p)	Implementation by tariff.

(I)	Each utility shall have on file with the Commission an interconnection tariff that sets forth fees, deadlines and interconnection procedures.  A utility’s interconnection tariff shall comply with these Interconnection Rules, but when appropriate may include shorter deadlines for certain procedures.

(II)	The interconnection tariff shall be filed along with an advice letter.  Tariffs filed by cooperative electric associations shall be informational only.  Tariffs filed by investor-owned electric utilities may be set for hearing and suspended in accordance with the Commission Rules of Practice and Procedure and applicable statutes.

(III)	The tariff shall include the following provisions:

(A)	timelines:  paragraphs 3853(a),(d),(f), 3854(a), 3855(b),(c),(d), 3856(a),(b),(c),(d);

(B)	any fees:  including but not limited to those referenced at paragraphs 3853(a),(d),(f),(j), 3854(a) and (b), and 3856(a);

(i) the utility shall demonstrate that any fee established in tariff is cost-based;

(C)	material modification withdrawals:  paragraph 3853(d);

(D)	maximum rated capacity:  paragraph 3853(b), and (c); and

(E)	a process to request and install a meter collar adapter, including the provision of any estimate of costs to the interconnection customer.

(q)	Reporting.

(I)	Each utility shall submit an interconnection report to the Commission two times per year and shall make it available to the public on its website.  A cooperative electric association that has voted to exempt itself from regulation pursuant to C.R.S. § 40-9.5-103 shall submit an interconnection report to the Commission once per year.  The first interconnection report shall be due 180 days after the effective date of these interconnection rules.  Upon a filing by a party with proper standing showing good cause, and when necessary and appropriate, the Commission may by order increase the frequency of such reporting on a temporary basis.  The report shall contain relevant totals for both the year and the most recent reporting period, including the following information listed in subparagraphs (q)(II) and (III) of this rule.  The report shall also contain the total number of missed deadlines contained in these rules in the reporting period as well as copies of any notices of delay or missed deadlines issued by the utility to an interconnection customer pursuant to paragraph 3853(g).

(II)	Pre-application reports:

(A)	total number of reports requested;

(B)	total number of reports in process;

(C)	total number of reports issued;

(D)	total number of requests withdrawn;

(E)	maximum, mean, and median processing times from receipt of request to issuance of report; and

(F)	number of reports processed in more than the 20 business days allowed in subparagraph 3853(a)(IV)(A).

(III)	Interconnection applications:

(A)	total number received, broken down by:

(i)	primary fuel type (e.g., solar, wind, bio-gas, etc.); and

(ii)	system size (e.g., <25 kW, <1 MW, <5MW, >5MW).

(B)	Level 1 review process.

(i)	total number of applications processed; and

(ii)	maximum, mean, and median processing times from receipt of complete application to provision of a counter-signed interconnection agreement.

(C)	Level 2 review process.

(i)	total number of applications that passed the screens in paragraph 3855(b);

(ii)	total number of applications that failed the screens in paragraph 3855(b); and

(iii)	maximum, mean, and median processing times from receipt of complete application to issuance of an interconnection agreement.

(D)	Supplemental review.

(i)	total number of applications that passed the screens in paragraph 3855(d);

(ii)	total number of applications that failed the screens in paragraph 3855(d); and

(iii)	maximum, mean, and median processing times from receipt of complete application to issuance of interconnection agreement.

(E)	Level 3 review process:

(i)	system impact studies

(ii)	total number of system impact studies completed under paragraph 3856(c); and

(iii)	maximum, mean, and median processing times from receipt of a signed interconnection system impact study agreement to provision of study results.

(r)	Filing of interconnection manual.  Utilities subject to these rules shall maintain an interconnection manual and provide to the Commission any updated version within 30 days after any material changes have been made to its manual.  Utilities shall establish an internal process of acquiring timely feedback from stakeholders regarding the material changes filed with the Commission.  Each time the utility updates the interconnection manual, the utility shall make available a redline highlighting the changes.

(s)	Interconnection information on website.  Each utility shall provide on its website: a copy of its current interconnection manual filed with the Commission in accordance with paragraph 3853(r); interconnection standards including a list of approved meter collar adapters in accordance with rule 3859; detailed information on the review process by which a manufacturer of a meter collar adapter can seek the utility’s approval for its installation in the utility’s service territory; and other technical guidance not included in either the utility’s interconnection tariff on file with the Commission in accordance with paragraph 3853(p) or its interconnection manual.

*           *           *           *

[indicates omission of unaffected rules]

### 3857.	Certification Codes and Standards.

Unless one or more of the following standards has been incorporated by reference into these interconnection rules, the Commission encourages the utilities and their interconnection customers, to whom these rules apply, to use the following standards and reference materials for guidance.

ANSI C84.1-2016 Electric Power Systems and Equipment – Voltage Ratings (60 Hertz)

ANSI/NEMA MG 1--2016, Motors and Generators

IEEE Std C37.90.1-2012, IEEE Standard Surge Withstand Capability (SWC) Tests for Protective Relays and Relay Systems

IEEE Std C37.90.2-2004, IEEE Standard Withstand Capability of Relay Systems to Radiated Electromagnetic Interference from Transceivers

IEEE Std C37.108-2002, IEEE Guide for the Protection of Network Transformers

IEEE Std C57.12.44-2014, IEEE Standard Requirements for Secondary Network Protectors

IEEE Std C62.41.2-2002/Cor 1-2012, IEEE Recommended Practice on Characterization of Surges in Low Voltage (1000V and Less) AC Power Circuits Corrigendum 1: Deletion of Table A.2 and Associated Text

IEEE Std C62.45-2002, IEEE Recommended Practice on Surge Testing for Equipment Connected to Low-Voltage (1000V and Less) AC Power Circuits

IEEE Std 100-2000, The Authoritative Dictionary of IEEE Standards Terms, Seventh Edition

IEEE Std 519-2014, IEEE Recommended Practices and Requirements for Harmonic Control in Electrical Power Systems

IEEE Std 1453-2015 IEEE Recommended Practice for the Analysis of Fluctuating Installation on Power Systems

IEEE Std 1547-2018, IEEE Standard for Interconnection and Interoperability of Distributed Energy Resources with Associated Electric Power Systems Interfaces

IEEE Std 1547.1-2005, IEEE Standard Conformance Test Procedures for Equipment Interconnecting Distributed Resources with Electric Power Systems

NFPA 70 (2017), National Electrical Code

UL 414 Meter Sockets, and its relevant Annexes.

UL 1741 Inverters, Converters, and Controllers for Use in Independent Power Systems

UL 1741 SA, until January 1, 2022, or until such time new DERs applying for interconnection will comply with IEEE 1547-2018, IEEE Standards for Inverters, Converters, Controllers and Interconnection System Equipment for Use with Distributed Energy Resources

*           *           *           *

[indicates omission of unaffected rules]

### 3859.	Meter Collar Adapters.

(a)	Each utility shall allow for the use of a meter collar adapter to permit the interconnection of distributed energy resources and for electrical isolation of the customer’s site for energy backup purposes.  The utility may require, at no additional cost to the interconnecting customer, the installation of the meter collar adapter to be performed by the utility or a licensed electrical contractor or other third party approved by the utility.  The interconnection customer is responsible for any necessary costs to relocate the meter enclosure or replace the meter housing.

(b)	The utility shall maintain a list of at least one approved meter collar adapter on its website in accordance with paragraph 3853(s).  The utility shall approve a meter collar adapter submitted for review by its manufacturer if it:

(I)	is certified for compliance with the standards referenced in UL 414, including its appliable annexes;

(II)	is physically and electrically compatible with the UL listings of existing equipment at the installation location and meets all National Electric Code clearance and safety standards;

(III)	is rated adequately for the connected equipment that is used for purposes of solar, battery storage technology, electric vehicle charging, or energy efficiency measures; and

(IV)	does not compromise the structural integrity of the meter enclosure or interfere with safe meter operation, considering that the installation of the meter collar adapter may require the relocation of the meter enclosure or replacement of the meter housing.

(c)	If the utility determines a meter collar adapter fails to meet the requirements for approval in paragraph 3859(b), the utility shall provide written notice to its manufacturer explaining why the adapter fails to meet the requirements in paragraph 3959(b) and contrasting the rejected meter collar adapter to the meter collar adapter(s) approved by the utility. an interconnection customer to request and install a meter collar adapter

(d)	The utility review process for approving or rejecting a meter collar adapter must be complete, including the provision of notice pursuant to paragraph 3859(c), within 90 days after the manufacturer submits a request to the utility for its review.  A manufacturer seeking the Commission to resolve any disputes concerning the substance or procedures involved in the approval process for a meter collar adapter may file a formal complaint in accordance with rule 1302.

(e)	The utility must update its website to provide information on the review process by which a manufacturer of a meter collar adapter can seek the utility’s approval for its installation in the utility’s service territory review as required by paragraphs 3853(s) and 3859(d) no later than 30 days following the effective date of these rules.  Notice to the Commission regarding the utility’s compliance with this requirement shall be included in the tariff filing required by paragraph 3859(h).

(f)	The list of approved meter collar adapters posted on the utility’s website in accordance with paragraph 3853(s) must be updated at least annually.  A meter collar adapter approved pursuant to this rule shall be included in the list of approved meter collar adapters on the utility’s website within 30 days upon the completion of the utility’s review.

(g)	The process for an interconnection customer to request and install a meter collar adapter as set forth in the utility’s tariff in accordance with subparagraph 3853(p)(III)(E) shall take no longer than 30 days.  The process shall facilitate the installation of the meter collar adapter by an electrical contractor registered with the Colorado State Electrical Board.  The installation shall be performed by a master electrician, journeyman electrician, residential wireman, or properly supervised apprentice.

(h)	The utility must file with the Commission the tariff sheets governing the process for an interconnection customer to request and install a meter collar adapter as required by paragraph 3859(f) and subparagraph 3853(p)(III)(E) no later than 30 days following the effective date of these rules.